| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0086-v2 |
| --- | --- |

| Title: | AHG9: On the encoder optimization information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Francesco Cricri, Honglei Zhang Nokia Technologies, Tampere, Finland | Email: | {miska.hannuksela, francesco.cricri, honglei.1.zhang}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The encoder optimization information (EOI) SEI message is one of the SEI messages included in the Technologies under Consideration (JVET-AF2032). This contribution proposes the following changes to EOI SEI message:

- 2-bit indicators the eoi_for_human_viewing_idc and eoi_for_machine_analysis_idc are proposed to replace the respective flags present in JVET-AF2032. The values for the eoi_for_human_viewing_idc are specified as follows (and respective values are specified for eoi_for_machine_analysis_idc):

3: optimized for human viewing

2: suitable but not specifically optimized for human viewing

1: unsuitable for human viewing

0: unknown if the video is suitable for human viewing

- It is proposed to enable indicating temporal sublayers where optimization has been applied.

Option A: Multiple EOI SEI messages, each for a different sublayer range:

- When an EOI SEI message potentially persists for multiple pictures, the temporal sublayers to which the EOI SEI message applies are indicated as follows:

- A syntax element (eoi_num_sublayers) indicates the number of sublayers.

- If eoi_num_sublayers is equal to 7, it indicates that the EOI SEI message applies to all sublayers.

- Otherwise, eoi_num_sublayers indicates the number of sublayers to which the EOI SEI message applies and eoi_min_tid is present to indicate the smallest sublayer identifier to which the EOI SEI message applies.

- The semantics of eoi_persistence_flag equal to 1 is updated to concern only the temporal sublayers indicated by eoi_num_sublayers and eoi_min_tid.

- Constraints are added to avoid EOI SEI messages with conflicting syntax element values present in the same picture unit. It is allowed to have two EOI SEI messages with different SEI payload content in the same picture unit, when the EOI SEI messages apply different ranges of temporal sublayers.

Option B: A single EOI SEI message with a loop over sublayer ranges:

- When an EOI SEI message potentially persists for multiple pictures, the count of sublayer ranges is indicated in eoi_num_sublayer_ranges_minus1.

- For each sublayer range, the smallest sublayer identifier and the number of sublayers are indicated through eoi_min_tid_delta[ i ]  and eoi_num_sublayers[ i ], respectively.

- The optimization information is provided separately for each sublayer range in a loop. An array index [ i ] is added to the syntax elements related to the optimization information.

Version 2 of this document responds to the comments expressed in the first JVET review and some offline communication. Version 2 adds option 2B and clarifies the interpretation of option 2A.

# Human viewing and machine analysis indicators

## Problem statement

The EOI SEI message indicates the purpose for the optimization with the following two flags:

eoi_for_human_viewing_flag equal to 1 specifies that purposes for the applied optimization include human viewing. eoi_for_human_viewing_flag equal to 0 specifies that purposes for the applied optimization may or may not include human viewing.

eoi_for_machine_analysis_flag equal to 1 specifies that purposes for the applied optimization include machine analsysis. eoi_for_machine_analysis_flag equal to 0 specifies that purposes for the applied optimization may or may not include machine analysis.

It is asserted that the two flags are not sufficient to provide guidance for the following cases:

- When the video has not been optimized for human viewing (eoi_for_human_viewing_flag is equal to 0) and has been optimized for machine analysis (eoi_for_machine_analysis_flag is equal to 1), is the video still suitable for human viewing? The answer to this question would be important to avoid accidental displaying of machine-targeted bitstreams that look subjectively unpleasant.

- When the video has been optimized for human viewing (eoi_for_human_viewing_flag is equal to 1) and has not been optimized for machine analysis (eoi_for_machine_analysis_flag is equal to 0), is the video still suitable for machine analysis?

## Overview of the proposal

It is proposed to use a 2-bit indicator rather than a flag with values. The values for the eoi_for_human_viewing_idc are specified as follows (and respective values are specified for eoi_for_machine_analysis_idc):

3: optimized for human viewing

2: suitable but not specifically optimized for human viewing

1: unsuitable for human viewing

0: unknown if the video is suitable for human viewing

## Proposed specification text (relative to JVET-AF2032)

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_flag | u(1) |
| eoi_for_machine_analysis_flag | u(1) |
| eoi_for_human_viewing_idc | u(2) |
| eoi_for_machine_analysis_idc | u(2) |
| eoi_type | u(16) |
| if( EoiObjectBasedFlag ) |  |
| eoi_object_based_idc | ue(v) |
| if( EoiTemporalResamplingFlag ) { |  |
| eoi_temporal_resampling_type_flag | u(1) |
| eoi_num_int_pics | ue(v) |
| } |  |
| } |  |
| } |  |

eoi_for_human_viewing_flag equal to 1 specifies that purposes for the applied optimization include human viewing. eoi_for_human_viewing_flag equal to 0 specifies that purposes for the applied optimization may or may not include human viewing.

eoi_for_machine_analysis_flag equal to 1 specifies that purposes for the applied optimization include machine analsysis. eoi_for_machine_analysis_flag equal to 0 specifies that purposes for the applied optimization may or may not include machine analysis.

eoi_for_human_viewing_idc equal to 3 specifies that purposes for the applied optimization include human viewing. eoi_for_human_viewing_idc equal to 2 specifies that the video is suitable but not specifically optimized for human viewing. eoi_for_huma_viewing_idc equal to 1 specifies that the video is unsuitable for human viewing. eoi_for_human_viewing_idc equal to 0 specifies that it is unknown if the video is suitable for human viewing.

eoi_for_machine_analysis_idc equal to 3 specifies that purposes for the applied optimization include machine analysis. eoi_for_machine_analysis_idc equal to 2 specifies that the video is suitable but not specifically optimized for machine analysis. eoi_for_machine_analysis_idc equal to 1 specifies that the video is unsuitable for machine analysis. eoi_for_machine_analysis_idc equal to 0 specifies that it is unknown if the video is suitable for machine analysis.

# Indicating temporal sublayers where optimization has been applied

## Problem statement

Section 7.2 of JVET-AE2030 suggests that in the use case of coding video content for machine consumption the QP value for pictures on higher temporal sublayers can be higher than a customary QP for respective temporal sublayers for user viewing. For example, JVET-AB0275 proposed increasing the QP offset by 5 for frames with TemporalId greater than or equal to 2 compared to the QP settings of the random access common test conditions.

The EOI SEI message does not currently indicate to which temporal sublayers the optimizations have been applied. It is asserted that the lack of sublayer-specific optimization information may be problematic at least in the following cases:

- When the optimization has been applied to the highest temporal sublayer(s) only and the bitstream is later pruned to exclude these highest temporal sublayer(s), the information of the EOI SEI message is no longer correct.

- It is not known that a lower frame rate version of the content would be suitable for displaying, when the video has been optimized for machine consumption by increasing the QP value for the highest temporal sublayer(s) more than what is pleasant for user viewing. For example, it could be desirable to display a 30-Hz video for monitoring even if the machine analysis gets a 60-Hz video as input.

## Option A

## Overview of the proposal

The proposal of option A includes the following items:

- When an EOI SEI message potentially persists for multiple pictures, the temporal sublayers to which the EOI SEI message applies are indicated as follows:

- A syntax element (eoi_num_sublayers) indicates the number of sublayers.

- If eoi_num_sublayers is equal to 7, it indicates that the EOI SEI message applies to all sublayers.

- Otherwise, eoi_num_sublayers indicates the number of sublayers to which the EOI SEI message applies and eoi_min_tid is present to indicate the smallest sublayer identifier to which the EOI SEI message applies.

- The semantics of eoi_persistence_flag equal to 1 is updated to concern only the temporal sublayers indicated by eoi_num_sublayers and eoi_min_tid.

- Constraints are added to avoid EOI SEI messages with conflicting syntax element values present in the same picture unit. It is allowed to have two EOI SEI messages with different SEI payload content in the same picture unit, when the EOI SEI messages apply different ranges of temporal sublayers.

## Proposed specification text (relative to JVET-AF2032)

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_flag | u(1) |
| eoi_for_machine_analysis_flag | u(1) |
| eoi_type | u(16) |
| if( eoi_persistence_flag ) { |  |
| eoi_num_sublayers | u(3) |
| if( eoi_num_sublayers < 7 ) |  |
| eoi_min_tid | u(3) |
| } |  |
| if( EoiObjectBasedFlag ) |  |
| eoi_object_based_idc | ue(v) |
| if( EoiTemporalResamplingFlag ) { |  |
| eoi_temporal_resampling_type_flag | u(1) |
| eoi_num_int_pics | ue(v) |
| } |  |
| } |  |
| } |  |

eoi_persistence_flag specifies the persistence of the optimization information provided in this SEI message. eoi_persistence_flag equal to 0 specifies that the optimization information applies for the current picture only. eoi_persistence_flag equal to 1 specifies that the optimization information applies for the pictures in the temporal sublayers indicated by eoi_num_sublayers and eoi_min_tid, when present, among the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

…

eoi_num_sublayers equal to 7 specifies that this SEI message applies to all temporal sublayers. eoi_num_sublayers less than 7 specifies the number of temporal sublayers to which this SEI message applies. eoi_num_sublayers equal to 0 is reserved.

eoi_min_tid specifies the smallest temporal sublayer identifier value to which this SEI message applies. The value of eoi_min_tid shall be in the range of 0 to 6 − eoi_num_sublayers, inclusive. When eoi_persistence_flag is equal to 1 and eoi_min_tid is not present, eoi_min_tid is inferred to be equal to 0.

When eoi_persistence_flag is equal to 1, the variable eoiMaxSublayer, which is the greatest sublayer identifier value to which this SEI message applies, is derived as follows:

eoiMaxSublayer = eoi_min_tid + eoi_num_sublayers − 1

When eoi_persistence_flag is equal to 1, the semantics of eoi_for_human_viewing_flag and eoi_for_machine_analysis_flag apply to the sublayer representation of eoiMaxSubLayer and the semantics of eoi_type, eoi_object_based_idc (when present), eoi_temporal_resampling_type_flag (when present), and eoi_num_int_pics (when present) apply to the sublayers with a sublayer identifier in the range of eoi_min_tid to eoiMaxSublayer, inclusive.

…

When multiple encoder optimization information SEI messages are present in a particular PU, the following constraints apply for these SEI messages:

–	All of the SEI messages shall have the same value of eoi_cancel_flag.

–	When eoi_persistence_flag is present in the SEI messages, all the SEI messages shall have the same value of eoi_persistence_flag.

–	When any of the SEI messages has eoi_num_sublayers equal to 7, all the SEI messages shall have the same SEI payload content.

–	When an SEI message among the SEI messages has eoi_num_sublayers less than 7, the following constraints apply:

–	All the SEI messages with the same values of eoi_num_sublayers and eoi_min_tid, which are assigned to numSubLayersA and minTidA, respectively, shall have the same SEI payload content.

–	There shall be no such SEI message among the SEI messages for which both of the following are true:

–	eoi_num_sublayers is not equal to numSubLayersA or eoi_min_tid is not equal to minTidA.

–	eoi_min_tid is greater than minTidA and less than minTidA + numSubLayersA.

## Option B

### Overview of the proposal

The proposal of option B includes the following items:

- When an EOI SEI message potentially persists for multiple pictures, the count of sublayer ranges is indicated in eoi_num_sublayer_ranges_minus1.

- For each sublayer range, the smallest sublayer identifier and the number of sublayers are indicated through eoi_min_tid_delta[ i ]  and eoi_num_sublayers[ i ], respectively.

- The optimization information is provided separately for each sublayer range in a loop. An array index [ i ] is added to the syntax elements related to the optimization information.

### Proposed specification text (relative to JVET-AF2032)

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| if( eoi_persistence_flag ) { |  |
| eoi_num_sublayer_ranges_minus1 | ue(v) |
| for( i = 0; i <= eoi_num_sublayer_ranges_minus1; i++ ) { |  |
| eoi_min_tid_delta[ i ] | ue(v) |
| eoi_num_sublayers[ i ] | u(3) |
| } |  |
| } |  |
| for( i = 0; i <= eoi_num_sublayer_ranges_minus1; i++ ) { |  |
| eoi_for_human_viewing_flag[ i ] | u(1) |
| eoi_for_machine_analysis_flag[ i ] | u(1) |
| eoi_type[ i ] | u(16) |
| if( EoiObjectBasedFlag[ i ] ) |  |
| eoi_object_based_idc[ i ] | ue(v) |
| if( EoiTemporalResamplingFlag[ i ] ) { |  |
| eoi_temporal_resampling_type_flag[ i ] | u(1) |
| eoi_num_int_pics[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

eoi_persistence_flag specifies the persistence of the optimization information provided in this SEI message. eoi_persistence_flag equal to 0 specifies that the optimization information applies for the current picture only. eoi_persistence_flag equal to 1 specifies that the optimization information applies for the pictures in the temporal sublayers indicated by eoi_min_tid_delta[ i ] and eoi_num_sublayers[ i ] among the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

…

eoi_num_sublayer_ranges_minus1 plus 1 specifies the count of sublayer ranges for which this SEI message provides the optimization information. eoi_num_sublayer_ranges_minus1 shall be less than 8. When not present, eoi_num_sublayer_ranges_minus1 is inferred to be equal to 0.

eoi_min_tid_delta[ i ] is used to derive the smallest temporal sublayer identifier value for the i-th sublayer range to which this SEI message applies.

eoi_num_sublayers[ i ] specifies the count of sublayers in the i-th sublayer range. eoi_num_sublayers[ i ] equal to 0 is reserved.

When eoi_persistence_flag is equal to 1, the variables eoiMinSublayer[ i ], which is the smallest sublayer identifier value in the i-th sublayer range, and eoiMaxSublayer[ i ], which is the greatest sublayer identifier value in the i-th sublayer range, are derived as follows:

if( i  = =  0 ) {
	eoiMinSubLayer[ i ] = eoi_min_tid_delta[ i ]
	eoiMaxSubLayer[ i ] = eoi_min_tid_delta[ i ] + eoi_num_sublayers[ i ] − 1
} else {
	eoiMinSubLayer[ i ] = eoiMaxSubLayer[ i − 1 ] + 1 + eoi_min_tid_delta[ i ]
	eoiMaxSubLayer[ i ] = eoiMinSubLayer[ i ] + eoi_num_sublayers[ i ] − 1
}

It is a requirement of bitstream conformance that eoiMaxSubLayer[ i ] is less than 8.

eoi_for_human_viewing_flag[ i ] equal to 1 specifies that purposes for the applied optimization include human viewing. eoi_for_human_viewing_flag[ i ] equal to 0 specifies that purposes for the applied optimization may or may not include human viewing.

eoi_for_machine_analysis_flag[ i ] equal to 1 specifies that purposes for the applied optimization include machine analsysis. eoi_for_machine_analysis_flag[ i ] equal to 0 specifies that purposes for the applied optimization may or may not include machine analysis.

When eoi_persistence_flag is equal to 1, the semantics of eoi_for_human_viewing_flag[ i ] and eoi_for_machine_analysis_flag[ i ] apply to the sublayer representation of eoiMaxSubLayer[ i ].

Add the array index [ i ] in the semantics of the remaining eoi_ syntax elements and derived variables and specify that the semantics to concern the i-th sublayer range to which this SEI message applies (when eoi_persistence_flag is equal to 1) or the current picture (when eoi_persistence_flag is equal to 0).

## Examples

### Sublayer-specific QP adaptation

Section 7.2 of JVET-AE2030 suggests that in the use case of coding video content for machine consumption the QP value for pictures on higher temporal sublayers can be higher than a customary QP for respective temporal sublayers for user viewing. For example, JVET-AB0275 proposed increasing the QP offset by 5 for frames with TemporalId greater than or equal to 2 compared to the QP settings of the random access common test conditions. In this example, the information provided in the EOI SEI message can be set as follows:

- For the sublayer range with identifiers 2 and above:

  - Unsuitable for human viewing

  - Optimized for machine analysis

  - eoi_type indicating temporal quality optimization

### Sublayer-specific object-based optimization

Due to complexity constraints in the encoder, the object detection required for object-based optimization with spatial QP adaptation is performed only for the lowest temporal sublayer, while no preprocessing is performed for the higher sublayers. In this example, the information provided in the EOI SEI message can be set as follows:

- For the sublayer range with identifier 0 only:

  - May or may not be suitable for human viewing

  - Optimized for machine analysis

  - eoi_type indicating object-based optimization optimization

# References

- M. M. Hannuksela, J. Chen, S. Deshpande, Hendry, S. McCarthy (editors), "Technologies under consideration for future extensions of VSEI (version 2)", JVET-AF2032.

- J. Chen, C. Hollman, and S. Liu (editors), "Optimization of encoders and receiving systems for machine analysis of coded video content (draft 3)", JVET-AE2030-v1.

- C. Hollmann, J. Ström, P. Wennersten, L. Litwic, R. Sjöberg, M. Damghanian, “Information about Ericsson’s response to the CfP for Video Coding for Machines”, JVET-AB0275.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).