| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0102-v1 |
| --- | --- |

| Title: | AHG12: New context model parameters for low delay B condition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Yan Ye Jie Chen Xinwei Li | Tel: Email: | {ruling.lrl, yan.ye, jiechen.cj, sid.lxw}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

# Abstract

This contribution proposes to separate context model parameters of low delay B slices from those of non-low delay B slices. In addition to three sets of context model parameters for B-/P-/I-slice types, a fourth set of context model parameters is proposed and is trained using the bitstream generated by ECM-11.0 under low delay B condition. A SPS flag is signaled to indicate whether the original set or the newly proposed set of context model parameters are used to initialize the context model of B slices. It is reported that on top of ECM-11.0, the overall coding performance for {Y, U, V} is {x%, x%, x%} for AI, {x%, x%, x%} for RA, {x%, x%, x%} for LB and {x%, x%, x%} for LP.

# Introduction

In ECM-11.0, there are three sets of context model parameters for B-/P-/I-slice types, respectively. Each set of context model parameters contains its own initial probabilities, window sizes and adaptive weights. Besides, two adjustment weights are shared among three sets.

# Proposed methods

In this contribution, it is proposed to add a new set of context model parameters for B-slice type. The initial probabilities, window sizes and adaptive weights in this new set are trained using the bitstream generated by ECM-11.0 under low delay B condition. The two adjustment weights are shared among the three original sets and the new set of context model parameters. A SPS flag is signaled to indicate whether the original set or the new set of context model parameters are used to initialize the context model of B slices. At the encoder, the SPS flag is configurated according to the random access or low delay B conditions.

# Simulation results

The proposed methods are implemented on top of ECM-11.0 and are tested under CTC for enhanced compression tool testing [1]. Table 1 shows the simulation results of the proposed methods. It is noted that the runtime is not accurate since the anchor and the test are run on different clusters.

Table 1. Simulation results of proposed methods

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | 0.14% | 0.10% | 97.8% | 96.6% | 100.0% | 99.9% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.03% | 0.14% | 0.06% | 98.7% | 98.6% | 100.0% | 100.2% |
| Class C | -0.07% | 0.12% | 0.10% | 99.8% | 99.1% | 99.6% | 100.0% |
| Class E | -0.08% | -0.04% | 0.04% | 101.2% | 100.1% | 100.0% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.10% | -0.06% | 0.15% | 98.8% | 97.0% | 100.3% | 100.0% |
| Class F | -0.07% | -0.18% | 0.18% | 96.8% | 98.2% | 100.0% | 100.0% |
| Class TGM | -0.05% | -0.03% | -0.05% | 96.2% | 98.0% | 100.0% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.13% | 0.46% | 0.25% | 97.4% | 99.8% | 99.7% | 100.1% |
| Class C | -0.23% | 0.29% | 0.15% | 97.4% | 99.9% | 98.7% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.34% | 0.36% | 0.06% | 97.5% | 99.7% | 99.7% | 100.0% |
| Class F | -0.23% | -0.19% | -0.10% | 97.2% | 98.4% | 100.0% | 100.0% |
| Class TGM | -0.18% | -0.18% | -0.09% | 97.3% | 99.2% | 99.5% | 99.8% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.13% | 0.09% | -0.43% | 96.8% | 101.2% | 100.3% | 100.0% |
| Class E | -0.32% | 0.93% | -0.11% | 97.5% | 99.3% | 100.2% | 100.9% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.17% | 0.77% | -0.61% | 96.1% | 99.0% | 99.8% | 99.9% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.12% | 0.19% | -0.01% | 97.5% | 99.9% | 100.5% | 100.0% |
| Class E | -0.02% | 0.60% | 0.10% | 97.5% | 99.9% | 98.8% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.05% | 0.40% | -0.39% | 97.8% | 99.3% | 99.8% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

For comparison, Table 2 summarizes the results of EE2 Test 5.1, which retrains CABAC initialization parameters for all slices, and this proposal, which adds CABAC initialization for low-delay B slices.

Table 2. Summary results of EE2 Test 5.1 and the proposed methods

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| EE2 Test5.1 | -0.04% | 0.05% | 0.01% | -0.14% | 0.28% | 0.15% |
| Proposed methods |  |  |  |  |  |  |
|  | Low delay B |  |  | Low delay P |  |  |
|  | Y | U | V | Y | U | V |
| EE2 Test5.1 | -0.06% | 0.77% | 0.56% |  |  |  |
| Proposed methods |  |  |  |  |  |  |

# Conclusion

This contribution proposes a new set of context model parameters for low delay B condition. It is reported that x% BD-rate reduction can be achieved in low delay B condition. It is recommended to further study this proposal in the next round of EE2.

# References

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).