| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0103-v1 |
| --- | --- |

| Title: | EE2-1.16: IBC LIC with slope adjustment |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changyue Ma, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Che-Wei Kuo, Ning Yan, Xianglin Wang | Email: | machangyue@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2 Test-1.16 on IBC LIC with slope adjustment. On top of ECM-11.0, simulation results of Test 1.16 are reported as below:

For SCC content, the results compared with ECM-11.0:

AI:

Class F : {-0.14%, -0.25%, -0.11%, 100.2%, 98.2%}, Class TGM : {-0.16%, -0.10%, -0.20%, 100.0%, 99.8%};

RA:

Class F : {-0.20%, -0.36%, -0.25%, 100.2%, 99.8%}, Class TGM : {-0.12%, -0.13%, -0.12%, 100.2%, 99.8%};

Introduction

IBC LIC with slope adjustment was proposed in JVET-AF0116 [1], and established as Test 1.16 in JVET-AF EE2 [2].

EE2-1.16

The slope adjustment of IBC LIC is proposed, where an offset parameter chosen from the set {-1/2, -1/4, -1/8, -1/16, 1/16, 1/8, 1/4, 1/2} is signaled to update the slope parameter of IBC LIC. The proposed method is only applied in IBC AMVP mode and luma component. In addition, the proposed method is not applied to the CUs that apply multi-mode IBC-LIC.

Simulation results

Test 1.16 was implemented on top of ECM-11.0 [3] and tests were performed following ECM CTC [4]. The simulation results of Test 1.16 are summarized in the following tables.

Table 1: Performance of EE2-1.16

|  | AI |  |  |  |  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -0.14% | -0.25% | -0.11% | 100.2% | 98.2% | -0.20% | -0.36% | -0.25% | 100.2% | 99.8% |
| Class TGM | -0.16% | -0.10% | -0.20% | 100.0% | 99.8% | -0.12% | -0.13% | -0.12% | 100.2% | 99.8% |

Conclusion

In this contribution, the results of EE2 Test 1.16 on IBC LIC with slope adjustment are reported. It is recommended to adopt Test 1.16 into ECM.

References

- C. Ma, X. Xiu, W. Chen, H.-H. Jhu, C.-W. Kuo, N. Yan, X.Wang, “Non-EE2: slope adjustment for IBC LIC,” JVET-AF0117, October 2023.

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Strom, F. Wang, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AF2024, October 2023.

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October 2023.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).