| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0106-v2 |
| --- | --- |

| Title: | AHG 12: TIMD merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ramin G. Youvalari, Mohsen Abdoli | Email: | raminyouvalari@xiaomi.com mabdoli@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

This contribution proposes a new intra prediction method called as TIMD merge mode for improving the performance of ECM-11.0. The proposed method inherits the TIMD modes and its associated information (fusion flag, fusion weights and wide-angle conditions of TIMD modes) from previously TIMD or TIMD merge coded blocks. The difference of the proposed method compared to the TIMD mode of ECM-11.0 is related to the selection prcess of the intra modes for TIMD. The block prediction aspect of the proposed method is the same as in TIMD mode. The impact on coding efficiency and runtimes of proposed method over ECM-11.0 are reportedly {for Y, U, V, EncT, DecT}:

CTC classes: AI { -0.09%, 0.05%, 0.04%, 103.6%, 99.7%}, RA { -0.xx%, -0.xx%, -0.xx%, 10x%, 10x%}

Class F:    AI { -0.02%, 0.00%, 0.04%, 103.9%, 100.3%}, RA { -0.06%, -0.13%, -0.15%, 100%, 10x%}

Class TGM: AI { 0.01%, -0.01%, -0.02%, 104.1%, 99.9%}, RA { -0.01%, -0.04%, -0.03%, 101%, 10x%}

# Introduction

The TIMD process [1] is based on a template cost analysis in which several intra modes -- from a candidates list comprising MPMs, wide-angular, DC, horizontal, vertical, and extended modes are tested. The two intra modes (IPM1 and IPM2) providing the smallest SATD costs are selected and blended in a fusion process according to their respective template cost.

# Proposed Method

In this contribution it is proposed to inherit the TIMD modes from the previously TIMD coded blocks. For this, the adjacent and non-adjacent spatial neighboring blocks are scanned and if the scanned block is coded in TIMD or TIMD merge mode, then its TIMD information (prediction modes, fusion flag, fusion weights and wide-angle conditions of TIMD modes) are added to a TIMD merge list. Up to 10 TIMD pairs are added into the list.

Figure 1 shows the non-adjacent spatial neighboring blocks that are used in TIMD merge list generation.

Fig 1. Non-adjacent spatial neighboring candidates for TIMD merge mode

Table 1: Initial TIMD merge list

| Cand | Intra modes | Fusion weights |
| --- | --- | --- |
| 1 | (IPM1,1, IPM1,2) | (W1,1, W1,2) |
| 2 | (IPM2,1, IPM2,2) | (W2,1, W2,2) |
| 3 | (IPM3,1, IPM3,2) | (W3,1, W3,2) |
| . . . | . . . | . . . |
| 10 | (IPM10,1, IPM10,2) | (W10,1, W10,2) |

After forming the TIMD merge list, the merge candidates are sorted based on the SATD cost over the template of the current block. For the cost calculation, the SATD costs of the pair TIMD modes are normalized using their fusion weights. For example, if a merge candidate has two intra modes (IPM1, IPM2) with fusion weights (W1, W2), the candidate’s cost is calculated as:

Cost = (SATD(IPM1) * W1 + SATD(IPM2) * W2 ) >> shift

The template size and the template cost calculation aspects are the same as in TIMD mode.

After sorting the TIMD merge candidates, two best candidates from the list are selected to be used in encoder RDO.

In the encoder side, the two candidates are passed to the SATD-based sorting and the best performing candidate among the two is selected and used in full-RD.

# Bitstream Signalling

Usage of the mode is signaled with a CABAC coded PU level flag. The TIMD merge mode is used as a sub-mode of TIMD tool. Moreover, when the TIMD merge mode is enabled, a CABAC coded index is also signaled in order to indicate whether the first candidate, from the merge list, has been used or the second candidate.

# Simulation Results

Tables below summarize the coding efficiency and runtime impact of the proposed method in ECM-11.0 [2] using the JVET common test conditions [3].

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08 % | 0.03 % | 0.02 % | 102.8% | 99.7% | 99.7% | 100.3% |
| Class A2 | -0.10 % | 0.06 % | 0.10 % | 103.4% | 99.9% | 100.0% | 100.0% |
| Class B | -0.09 % | 0.06 % | 0.05 % | 103.7% | 99.6% | 100.1% | 100.2% |
| Class C | -0.09 % | 0.00 % | 0.02 % | 103.9% | 99.4% | 100.6% | 100.4% |
| Class E | -0.07 % | 0.11 % | 0.05 % | 104.1% | 100.3% | 99.7% | 100.1% |
| Overall | -0.09 % | 0.05 % | 0.04 % | 103.6% | 99.7% | 100.1% | 100.2% |
| Class D | -0.10 % | 0.09 % | 0.17 % | 104.7% | 100.4% | 99.5% | 99.8% |
| Class F | -0.02 % | 0.00 % | 0.04 % | 103.9% | 100.3% | 99.7% | 100.1% |
| Class TGM | 0.01 % | -0.01 % | -0.02 % | 104.1% | 99.9% | 100.0% | 99.9% |

|  | Random Access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class B | -0.03 % | 0.08 % | -0.05 % | 100.2% | 99.9% | #DIV/0! | #DIV/0! |
| Class C | -0.02 % | 0.01 % | -0.06 % | 100.5% | 100.0% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class D | -0.04 % | 0.17 % | -0.24 % | 100.9% | 100.0% | #DIV/0! | #DIV/0! |
| Class F | -0.06 % | -0.13 % | -0.15 % | 100.2% | 99.7% | #DIV/0! | #DIV/0! |
| Class TGM | -0.01 % | -0.04 % | -0.03 % | 101.2% | 100.0% | #DIV/0! | #DIV/0! |

# Conclusion

This contribution proposes TIMD merge mode that improves the intra prediction performance of ECM-11.0. The proposed method inherits TIMD information from the previously TIMD coded blocks intead of deriving them from current block’s template. The experimental results showed that the TIMD merge mode provides 0.09% average luma BD-Rate gain in AI and 0.xx% gain in RA configurations. It is suggested to study the proposed method in the next EE2 round.

# References

- Coban et.al, “Algorithm description of Enhanced Compression Model 11 (ECM 11)”, JVET-AF2025, Oct. 2023.

- ECM-11.0 reference software, https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).