| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0136-v2 |
| --- | --- |

| Title: | EE2-1.5: IntraTMP extension to LIC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, T. Dumas, K. Naser, Y. Chen, M. Radosavljević, T. Poirier | Tel: Email: | fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2 test 1.5 which was proposed in JVET-AF0275 [2]. The method proposed to extend the usage of Local Illumination Compensation to Intra-TMP prediction mode, in addition to the IBC and Inter coding modes.

A CU-level flag is signaled to indicate the use of LIC at luma block level. For screen content coding, the Intra-TMP LIC mode is extended to support top-only, left-only for computing single linear model parameters. MMLM is also supported for Intra TMP-LIC mode in screen content coding.

The impact on coding efficiency and runtimes of the test over ECM-11.0 is reportedly as follows for {Y, U, V, EncT, DecT, VmPeak}:

AI: { -0.11%, -0.05%, -0.08%, 103.7%, 100.6%, 99.4% }

RA: { -0.04% , -0.07% , -0.07% , 100.6% , 101.3% , 99.9% , 100.2% }

Furthermore, a second trade-off of IntraTMP-LIC is reported here and is used in combination with other tools (see tests 1.6, 1.7a and 1.7b). Its impact on ECM-11.0 performances is as follows:

AI: { -0.04% , 0.02% , -0.01% , 101.5% , 95.0% , 99.2% , 97.4% }

# Introduction

In ECM, Local Illumination Compensation (LIC) is supported in combination with IBC and Inter prediction modes, while it is not allowed with IntraTMP. On the other hand, convolutional template-based filtering is allowed for both IBC and IntraTMP modes. It is thus proposed to allow LIC with IntraTMP, which aligns IntraTMP and IBC in terms of possible prediction enhancements.

# Proposed method

It is proposed to allow the application of LIC to blocks in IntraTMP mode. Specifically:

- The LIC usage for an IntraTMP mode is signaled through a CU-level flag

- Usages of LIC and FLM (CCCM-like filtering) are mutually exclusive for a given CU.

- Usages of LIC together with fusion in intra TMP is allowed.

- Top-only and Left-only template usage for LIC model determination is allowed for screen content coding. For camera-captured coding, only the top-left template is employed.

- Multi Mode Linear Model (MMLM) is suppored similarly to IBC-LIC, for screen content coding.

- When LIC is used for a given CU, the Intra TMP search process employs MRSAD rather than SAD distortion function to search for block vector.

# Experimental results

The proposed test was run under JVET EE2 coding test conditions [1].

## IntraTMP-LIC as delivered in individual test

Below Table 1 shows the performances of test 1.5 as delivered in EE2 source code. Luma coding gain is -0.11% in AI test conditions, and -0.04% in RA. Encoding and decoding times are respectively 103.7% and 100.6% in AI. No runtime impact is observed in RA.

Table 1: experimental results of test 1.5

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |
| Class A1 | -0,06% |  | -0,01% |  | 0,08% |  | 105,2% |  | 100,4% |  | 101,1% |  | 94,6% |
| Class A2 | -0,18% |  | -0,15% |  | -0,13% |  | 103,9% |  | 101,3% |  | 99,8% |  | 96,2% |
| Class B | -0,12% |  | -0,08% |  | -0,12% |  | 103,1% |  | 100,0% |  | 96,3% |  | 94,5% |
| Class C | -0,07% |  | -0,06% |  | -0,12% |  | 103,0% |  | 100,4% |  | 100,1% |  | 87,6% |
| Class E | -0,15% |  | 0,06% |  | -0,10% |  | 103,9% |  | 101,7% |  | 101,5% |  | 90,6% |
| Overall | -0,11% |  | -0,05% |  | -0,08% |  | 103,7% |  | 100,6% |  | 99,4% |  | 92,6% |
| Class D | -0,03% |  | -0,04% |  | 0,04% |  | 101,7% |  | 98,5% |  | 99,7% |  | 91,1% |
| Class F | -0,17% |  | -0,18% |  | 0,04% |  | 102,0% |  | 101,7% |  | 96,8% |  | 90,1% |
| Class TGM | -0,04% |  | -0,04% |  | -0,07% |  | 99,4% |  | 97,7% |  | 101,0% |  | 95,0% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | -0,03% | -0,06% |  | -0,19% |  | 103,1% |  | 103,0% |  | 99,5% |  | 99,5% |  |
| Class A2 | -0,10% | -0,06% |  | -0,17% |  | 100,3% |  | 98,9% |  | 100,1% |  | 100,1% |  |
| Class B | -0,04% | 0,00% |  | 0,06% |  | 101,1% |  | 99,9% |  | 100,0% |  | 100,0% |  |
| Class C | 0,00% | -0,18% |  | -0,06% |  | 98,3% |  | 103,7% |  | 100,1% |  | 101,1% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0,04% | -0,07% |  | -0,07% |  | 100,6% |  | 101,3% |  | 99,9% |  | 100,2% |  |
| Class D | -0,04% | -0,10% |  | -0,10% |  | 99,5% |  | 97,8% |  | 100,0% |  | 98,8% |  |
| Class F | -0,19% | -0,20% |  | -0,21% |  | 100,5% |  | 102,5% |  | 100,2% |  | 100,4% |  |
| Class TGM | -0,07% | -0,15% |  | -0,07% |  | 101,3% |  | 106,9% |  | 100,7% |  | 100,9% |  |

## IntraTMP-LIC second trade-off as delivered in combined tests 1.6, 1.7a and 1.7b

Note that a restricted version IntraTMP-LIC tool is delivered in combination tests 1.6, 1.7a and 1.7b to keep encoding time increase limited over ECM-11.0.

The restriction consists in a reduced IntraTMP search process for both LIC and non-LIC intra TMP coding units. That is, a SAD-based search is performed for non-LIC blocks and a MRSAD-based search is performed for intra TMP CUs coded in LIC mode.

To limit the amount of BV search process, the sampling of the IntraTMP search area is increased from 3 to 4.

The following Table 2 shows the performances of restricted IntraTMP-LIC over ECM-11.0.

Table 2: performance of the fast version of IntraTMP-LIC mode

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,05% | -0,02% | -0,03% | 103,1% | 98,4% | 101,0% | 92,5% |
| Class A2 | -0,09% | -0,05% | -0,07% | 103,3% | 98,4% | 99,8% | 95,5% |
| Class B | -0,03% | 0,06% | -0,04% | 100,5% | 94,3% | 96,0% | 95,6% |
| Class C | -0,02% | -0,07% | 0,03% | 101,4% | 94,6% | 99,6% | 103,2% |
| Class E | 0,00% | 0,20% | 0,08% | 99,6% | 90,3% | 101,5% | 100,0% |
| Overall | -0,04% | 0,02% | -0,01% | 101,5% | 95,0% | 99,2% | 97,4% |
| Class D | -0,02% | 0,02% | 0,08% | 99,6% | 92,1% | 99,8% | 107,3% |
| Class F | -0,10% | -0,11% | -0,01% | 101,5% | 96,9% | 98,4% | 96,9% |
| Class TGM | 0,09% | 0,09% | 0,07% | 97,8% | 90,9% | 101,0% | 102,4% |

# Cited Reference

- JVET-AF2017 , “Common test conditions and evaluation procedures for enhanced compression tool testing”, M. Karczewicz, Y. Ye, Oct. 2023

- JVET-AF0275, “AHG12: Intra TMP extension to DIMD, LIC and SGPM”, F. Le Léannec, K. Naser, G. Rath, T. Dumas, P. Bordes, Y. Chen (InterDigital), Oct. 2023

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).