| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET- AG0141-v2 |
| --- | --- |

| Title: | AHG 12: Occurrence-based intra coding (OBIC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ramin G. Youvalari, Mohsen Abdoli | Email: | raminyouvalari@xiaomi.com mabdoli@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

This contribution proposes a new intra prediction method called as Occurrence-based Intra Coding (OBIC) for improving the performance of ECM-11.0. The proposed method derives the intra prediction modes based on the occurrence of the intra modes in the spatial neighborhood of the block. The proposed method uses blending of up to five intra modes, with highest occurrence, along with the planar mode to obtain the prediction of the block. The blending weights are decided based on each mode’s occurrence in the neighborhood.

The impact on coding efficiency and runtimes of proposed method over ECM-11.0 are reportedly {for Y, U, V, EncT, DecT}:

CTC classes: AI { -0.12%, 0.03%, -0.04%, 103.2%, 100.1%}, RA { -0.xx%, -0.xx%, -0.xx%, 10x%, 10x%}

Class F: 	AI { -0.02%, 0.03%, 0.10%, 102.3%, 101.4%}, RA { -0.08%, -0.22%, -0.30%, 101.2%, 10x%}

Class TGM: AI { -0.02%, -0.05%, -0.07%, 101.6%, 102.2%}, RA { 0.02%, 0.02%, 0.06%, 100.1%, 10x%}

# Introduction

DIMD mode in ECM [1] derives up to five intra modes from the reconstructed neighbor samples, and those five predictors are combined with the planar mode predictor with the weights derived from the histogram of gradients (HoG).

In this contribution, we porpose a new intra prediction method that, similar to DIMD, blends up to five angular preidctions with a planar prediction to generate the final block prediction.

# Proposed Method

The Occurrence-based intra coding (OBIC) method derives the intra prediction modes of the current block based on the sample-wise occurrence of the intra modes in the spatial neighborhood of the block. For this, adjacent and non-adjacent spatial neighboring blocks are checked and the intra prediction modes of the blocks are collected into an occurrence histogram. Instead of Histogram of Gradient (HoG) as in DIMD, the OBIC introduces the Histogram of oCcurrence (HoC), which consists of the intra modes and their sample-wise occurrences. The occurrence values are calculated based on the number of sampels that are coded in a certain intra prediction mode in that neighborhood. For example, if a uiWidth × uiHeight block is coded with an IPM mode, the occurrence of the mode in that particular block is calculated as:

HoC[IPM] += uiWidth * uiHeight;

Where uiWidth and uiHeight are the width and height of a spatial neighboring block.

The occurrences of the exsiting modes from the spatial neighborhood blocks are accumulated into the histogram.

Figure 1 shows the non-adjacent spatial neighboring blocks that are used in OBIC mode’s HoC generation.

Fig 1. Non-adjacent spatial neighboring candidates for OBIC mode

Up to 5 angular modes with the highest occurrence along with the planar mode are selected from the HoC and used for final prediction by blending the prediction of the selected modes.

Some blocks, mentioned below, use more than one intra mode for prediction. In such cases, all the intra modes of such blocks are selected and used when creating the OBIC histogram:

- DIMD: up to 5 angular modes

- TIMD: up to 2 modes

- SGPM: 2 modes

- OBIC: up to 5 angular modes

Moreover, the intra modes of following blocks are not considered when creating the histogram of OBIC mode:

- MIP block

- IntraTMP block

- IBC block

The blending weights are calculated similar to the DIMD mode, but instead of using gradient values from the template, the occurrence values are used for OBIC. Moreover, the planar mode’s weight is also decided similary to DIMD mode.

The OBIC mode is only used in luma blocks.

Fig 2. An example of the histogram of occurrences (HoC) of IPM modes in the spatial neighborhood of a CU

# Bitstream Signalling

Usage of the mode is signaled with a CABAC coded PU level flag. The OBIC mode is used as a sub-mode of DIMD and its flag is signaled after DIMD flag.

Moreover, the OBIC mode accounts for one additional RD check at the encoder side.

# Simulation Results

Tables below summarize the coding efficiency and runtime impact of the proposed method in ECM-11.0 [2] using the JVET common test conditions [3].

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.14 % | 0.04 % | -0.08 % | 102.3% | 100.3% | 100.3% | 100.2% |
| Class A2 | -0.14 % | 0.03 % | 0.06 % | 102.4% | 96.8% | 100.0% | 100.2% |
| Class B | -0.13 % | 0.01 % | -0.11 % | 103.5% | 98.4% | 99.7% | 99.6% |
| Class C | -0.11 % | -0.02 % | 0.02 % | 103.6% | 103.1% | 100.0% | 100.5% |
| Class E | -0.08 % | 0.16 % | -0.04 % | 103.7% | 102.4% | 100.5% | 99.7% |
| Overall | -0.12 % | 0.03 % | -0.04 % | 103.2% | 100.1% | 100.0% | 100.0% |
| Class D | -0.12 % | -0.07 % | 0.00 % | 107.1% | 102.3% | 100.5% | 100.4% |
| Class F | -0.02 % | 0.03 % | 0.10 % | 102.3% | 101.4% | 100.3% | 99.9% |
| Class TGM | -0.02 % | -0.05 % | -0.07 % | 101.6% | 102.2% | 100.4% | 99.7% |

|  | Random Access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class B | -0.04 % | -0.05 % | -0.11 % | 99.2% | 95.9% | #DIV/0! | #DIV/0! |
| Class C | -0.07 % | -0.16 % | -0.01 % | 100.0% | 102.2% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class D | -0.06 % | -0.33 % | -0.37 % | 100.5% | 108.4% | #DIV/0! | #DIV/0! |
| Class F | -0.08 % | -0.22 % | -0.30 % | 101.2% | 106.1% | #DIV/0! | #DIV/0! |
| Class TGM | 0.02 % | 0.02 % | 0.06 % | 100.1% | 106.9% | #DIV/0! | #DIV/0! |

# Conclusion

This contribution proposes occurrence-based intra coding (OBIC) mode that improves the intra prediction performance of ECM-11.0. The proposed method derives the intra modes of a block based on the occurrence of intra modes in the spatial neighborhood of the block. The experimental results showed that the OBIC mode provides 0.12% average luma BD-Rate gain in AI and 0.xx% luma BD-Rate gain in RA configurations. It is suggested to study the proposed method in the next EE2 round.

# References

- Coban et.al, “Algorithm description of Enhanced Compression Model 11 (ECM 11)”, JVET-AF2025, Oct. 2023.

- ECM-11.0 reference software, https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).