| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0144-v2 |
| --- | --- |

| Title: | Carriage of depth and alpha maps as HEVC single-layer bitstreams |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas Emmanouil Potetsianakis Evangelos Alexiou Ramin Ghaznavi-Youvalari Mohsen Abdoli Mary-Luc Champel | Tel: Email: | thomase@xiaomi.com emmanouil@xiaomi.com alexiou@xiaomi.com ramin.youvalari@xiaomi.com mabdoli@xiaomi.com champelmaryluc@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

For delivering XR experiences on XR head-mounted displays (HMD), it is argued that split rendering services recently emerged in which the XR scenes are pre-rendered by a remote entity and then transmitted as video to the HMD. Specific to AR applications, it is further claimed that those services transmit the pre-rendered views as well as the associated depth maps and alpha maps as multiple video bitstreams in different RTP streams, as defined in the 3GPP SA4 TS 26.565 under development.

However, it is also argued that the current HEVC specification merely enables the carriage of alpha planes and depth pictures as auxiliary pictures in a multi-layer bitstream. In addition, the presence of metadata describing the alpha planes and depth pictures is constrained to layers containing auxiliary pictures and thus cannot be present in a single-layer bitstream.

Therefore, it is proposed to enable split-rendering services by allowing the carriage of depth maps and alpha maps as HEVC single-layer bitstreams.

# Introduction

For delivering XR experiences on XR head-mounted displays (HMD), new services emerged wherein the XR scenes are pre-rendered by a remote entity, such as a smartphone or a server, and then transmitted as video to be finally composed and displayed on the HMD. Those services are commonly known as remote rendering services. Specific to AR applications, the AR scene cannot be fully remotely rendered since the AR scene needs to be blended with the user’s surrounding. If the AR device is an optical see-through (OST) device, the pre-rendered views are blended when displayed on top of the real scene directly perceived by the user. This is for the instance the case for AR glasses. If the AR device is a video see-through (VST) device, the pre-rendered views are alpha blended with a captured view of the real scene to form a final image which is then displayed. This is the case for smartphones and mixed-reality headsets.

In the 3GPP SA4 standardisation group, the APIs and protocols for such split rendering services are currently being standardized in the specification TS 26.565 “Split Rendering Media Service Enabler” [1]. To realise this service, the TS 26.565 defines multiple independent RTP streams to transmit the pre-rendered views (textures) and the auxiliary information (depth pictures and alpha planes).

However, the HEVC multi-layer extensions enable the carriage of alpha planes and depth pictures only as auxiliary pictures in dedicated layers. Those “auxiliary” layers must by definition be associated with a layer carrying primary pictures, i.e. non-auxiliar pictures. In addition, the metadata describing the alpha planes and depth pictures via SEI messages is constrained to be present only in layers containing auxiliary pictures.

Therefore, the current HEVC specification does not allow to carry alpha planes and depth pictures as well as the related metadata in a single layer bitstream.

This contribution aims thus at addressing the need of split rendering services by enabling the carriage of depth maps and alpha maps as HEVC single-layer bitstreams.

Compared to version 1, version 2 refines the proposed new code points in Table E.5.

# Use case: multi-stream HEVC-based split rendering

## Background on alpha maps in split rendering services

For VST AR devices, the transmitted video stream of pre-rendered views contains an alpha plane. This alpha map sequence is used to perform the alpha blending composition with the captured view of the real scene. For example, the OpenXR specification [2] which is a predominant standard for XR devices defines such a blend mode as XR_ENVIRONMENT_BLEND_MODE_ALPHA_BLEND.

- XR_ENVIRONMENT_BLEND_MODE_ALPHA_BLEND. The composition layers will be alpha-blended with the real world behind the display. The composited image will be interpreted as an RGBA image, with the composited alpha channel determining each pixel’s level of blending with the real world behind the display. This is the typical mode for an AR experience on a phone or headset that supports video passthrough.

In contrast, the alpha planes, if any, are discarded for OST AR devices since the transparency information comes from samples being more or less close to black. For example, the OpenXR specification [2] defines such a blend mode as XR_ENVIRONMENT_BLEND_MODE_ADDITIVE:

- XR_ENVIRONMENT_BLEND_MODE_ADDITIVE. The composition layers will be additively blended with the real world behind the display. The composited image will be interpreted as an RGB image, ignoring the composited alpha channel during the additive blending. This will cause black composited pixels to appear transparent. This is the typical mode for an AR experience on a see-through headset with an additive display, although this mode can also be supported on devices that support video passthrough.

## Background on depth maps in split rendering services

For both types of OST and VST AR devices, the video streams transmitted to the AR devices carry depth maps for each pre-rendering view picture. This depth information allows to adjust the pre-rendered views based on the latest pose estimation using techniques such as late stage reprojection [3]. For example, the OpenXR specification [2] defines the XrCompositionLayerDepthInfoKHR data structure for the application to submit the depth map of each view and “perform more accurate reprojections taking depth into account”.

# Design goals

To enable a single-layer bitstream carrying a depth map sequence or alpha map sequence, the following information must be defined:

- Information indicating that the bitstream carries a depth map sequence or an alpha map sequence.

- Information indicating the mapping used between the depth/alpha map (mono-channel) and the luma and chroma components.

- Information specifying how to interpret the sample values from the decoded pictures to reconstruct the depth/alpha maps.

# Proposed solutions

## Overview

In order to enable the above use case, this contribution presents possible alternative solutions for each design goal:

- Information indicating that the bitstream carries a depth map sequence or an alpha map sequence.

  - Option 1.1: Defining a content type parameter to identify depth map and alpha map sequences in a new SEI message.

  - Option 1.2: Defining code points in colour primaries for depth maps and alpha maps as colour primaries in the VUI.

- Information indicating the mapping used between the depth/alpha map (mono-channel) and the luma and chroma components.

  - Option 2.1: Defining a component mapping parameter in the new SEI message.

  - Option 2.2: Defining a code points in matrix coefficients for mono-channel (luma only) in the VUI.

- Information specifying how to interpret the sample values from the decoded pictures to reconstruct the depth/alpha maps.

  - Option 3.1: New SEI messages based on the multi-layer extension but defined for the single layer context (no reuse of annexes F and G)

  - Option 3.2: Modifications of existing SEI messages to be used in the single layer context

The remainder of this clause 4 provide specification text to support those options.

## Options 1.1 and 2.1: Depth and alpha maps signalled as a new concept of content types

### Description

When carrying a depth map or an alpha map sequence in a single layer bitstream, the indication of the type of sequence is lost compared to a multi-layer bitstream with auxiliary pictures type signalling, i.e. AUX_DEPTH and AUX_ALPHA.

Additionally, depth and alpha maps are by definition mono-channel 2D arrays but the internal format of the coded video picture in the bitstream may not be mono-channel. When the coded picture format is multi-channel, there is a need to specify how the mapping is done between the reconstructed depth maps (mono-channel) and the decoded pictures.

There can be different types of mapping which are:

- Only the luma component is used and the chroma components are to be discarded.

- The three components are jointly used to represent the depth map and the specific mapping needs to be specified.

The remainder of this section provides text specification to support this signalling.

### Content type information SEI message

#### Content type information SEI message syntax

| content_type_information( payloadSize ) { | Descriptor |
| --- | --- |
| cti_content_tpe_idc | u(4) |
| cti_component_mapping_idc | u(8) |
| } |  |

#### Content type information SEI message semantic

When present, the content type information SEI message may be included in any access unit. Preferably, when present, the SEI message is included for the purpose of random access in an access unit in which the coded picture with nuh_layer_id equal to targetLayerId is an IRAP picture.

The content type information SEI message applies to all the pictures with nuh_layer_id equal to targetLayerId from the access unit containing the content type information SEI message up to but excluding the next picture, in decoding order, associated with a content type information SEI message applicable to targetLayerId or to the end of the CLVS of the nuh_layer_id equal to targetLayerId, whichever is earlier in decoding order.

The content type information SEI message may then apply until a new message of the same type appears in the bitstream of up to the end of the layer.

The syntax element cti_content_type_idc specifies the content type represented by the current layer in the CVS associated with this SEI. The value of cti_content_type_idc shall be in the range of 0 to 15, inclusive.

Table  - Definition of cti_content_type_idc

| cti_content_type_idc value | Content type description |
| --- | --- |
| 0 | The current layer represents a depth map sequence. |
| 1 | The current layer represents an alpha map sequence. |
| 2-10 | Reserved |
| 11-15 | Unspecified and left for usage by external specifications. |

cti_channel_mapping_idc specifies the type of mapping between a content image format and a picture format used in the current layer.

Table  - Definition of cti_component_mapping_idc

| Value | Description |
| --- | --- |
| 0 | This value defines a one-to-one mapping between the luma component and the content image. |
| 1 | This value defines a one-to-one mapping between the luma component and the content image. If present, the chroma components are to be discarded and their sample values are set to 1 << ( BitDepthC − 1 ). |
| 2 | This value defines a mapping in which the three components (luma and two chroma) constitute different bit ranges of the content image sample values. The coded picture format is 4:4:4 which provides a bit precision of BitDepth + 2*BitDepthC bits, i.e. the bit depth of the luma component plus two times the bit depth of the chroma component. The mapping is defined as follows: Y: y(i,j) = (m(i,j) & 0xFF0000) >> 16 Cb: cb(i,j) = (m(i,j) & 0x00FF00) >> 8 Cr: cr(i,j) = m(i,j) & 0x0000FF With y(I,j), cb(I,j) and cr(I,j), respectively the samples of the Y, Cb and Cr components and m(I,j) the sample of the content image. |
| 3..255 | Reserved |

## Option 1.2 and 2.2: Depth and alpha as signalled as new colour primaries

### Description

In this option, the depth map and alpha maps are considered as a type of colour primaries and signalled via the VUI. The reconstruction of the maps based on the luma and chroma components is determined by the matrix coefficients parameter. This has the benefit of leveraging the existing handling of the VUI at the system level to carry over the information.

### Proposed changes

colour_primaries indicates the chromaticity coordinates of the source primaries as specified in Table E.3 in terms of the CIE 1931 definition of x and y as specified in ISO 11664-1.

When the colour_primaries syntax element is not present, the value of colour_primaries is inferred to be equal to 2 (the chromaticity is unspecified or is determined by the application). Values of colour_primaries that are identified as reserved in Table E.3 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders shall interpret reserved values of colour_primaries as equivalent to the value 2.

Table E.3 — Colour primaries interpretation using colour_primaries syntax element

| Value | Primaries |  |  |  | Informative remark |
| --- | --- | --- | --- | --- | --- |
| 0 | Reserved |  |  |  | For future use by ITU-T ¦ ISO/IEC |
| 1 | primary | x | y |  | Rec. ITU-R BT.709-6 Rec. ITU-R BT.1361-0 conventional colour gamut system and extended colour gamut system (historical) IEC 61966-2-1 sRGB or sYCC IEC 61966-2-4 SMPTE RP 177 (1993) Annex B |
|  | green | 0.300 | 0.600 |  |  |
|  | blue | 0.150 | 0.060 |  |  |
|  | red | 0.640 | 0.330 |  |  |
|  | white D65 | 0.312 7 | 0.329 0 |  |  |
| … |  |  |  |  |  |
| 22 | primary | x |  | y | EBU Tech. 3213-E (1975) |
|  | green | 0.295 |  | 0.605 |  |
|  | blue | 0.155 |  | 0.077 |  |
|  | red | 0.630 |  | 0.340 |  |
|  | white D65 | 0.312 7 |  | 0.329 0 |  |
| 23 | Depth map |  |  |  | This signal is a depth picture |
| 24 | Alpha map |  |  |  | This signal is an alpha map |
| 25..255 | Reserved |  |  |  | For future use by ITU-T ¦ ISO/IEC |

Table E.5 — Matrix coefficients interpretation using the matrix_coeffs syntax element

| Value | Matrix | Informative remark |
| --- | --- | --- |
| 0 | Identity | The identity matrix. Typically used for GBR (often referred to as RGB); however, may also be used for YZX (often referred to as XYZ) IEC 61966-2-1 sRGB SMPTE ST 428-1 (2006) See Formulae (E-31) to (E-33) |
| 1 | KR = 0.212 6; KB = 0.072 2 | ITU-R Rec. BT.709-6 ITU-R Rec. BT.1361-0 conventional colour gamut system and extended colour gamut system (historical) IEC 61966-2-1 sYCC IEC 61966-2-4 xvYCC709 SMPTE RP 177 (1993) Annex B See Formulae (E-28) to (E-30) |
| … |  |  |
| 14 | ICTCP | Rec. ITU-R BT.2100-2 ICTCP See Formulae (E-62) to (E-64) for transfer_characteristics value 16 (PQ) See Formulae (E-65) to (E-67) for transfer_characteristics value 18 (HLG) |
| 15 | IPT-C2 | Society of Motion Picture and Television Engineers ST 2128 See Formulae (E-75) to (E-77) |
| 16 | YCgCo-Re | See Formulae (E-41) to (E-55) |
| 17 | YCgCo-Ro | See Formulae (E-41) to (E-55) |
| 18 |  | 1st colour component |  | A monochromatic representation with only the first colour component in the original representation present in the coded image. Typically used for Green only; however, may also be used for the Y tristimulus value (luminance), when the colour primaries are set to XYΖ, or to indicate that only the first component of the decoded image shall be interpreted for images not representing texture. |  |
| 19 |  | 2nd colour component |  | A monochromatic representation with only the second colour component in the original representation present in the coded image. Typically used for Red only; however, may also be used for the X tristimulus value, when the colour primaries are set to XYΖ, or to indicate that only the first component of the decoded image shall be interpreted for images not representing texture. |  |
| 20 |  | 3rd colour component |  | A monochromatic representation with only the third colour component in the original representation present in the coded image. Typically used for Blue only; however, may also be used for the Z tristimulus value, when the colour primaries are set to XYΖ, or to indicate that only the first component of the decoded image shall be interpreted for images not representing texture. |  |
| 21 |  | Luma only KR = 0.2126; KB = 0.0722 |  | A monochromatic representation of the luma component of the Rec. ITU-R BT.709-6 representation. |  |
| 22 |  | Luma only KR = 0.2627; KB = 0.0593 |  | A monochromatic representation of the luma component of the Rec. ITU-R BT.2020-2 non-constant luminance representation. |  |
| 23 | I only - ICTCP | A monochromatic representation of the I component of ICTCP representation. |
| 24..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

## Option 3.1: New SEI messages based on the multi-layer extension but refined for single layer context

### Description

The depth map information SEI message, presented in 4.2.2, is inspired by the existing depth representation information SEI message but redefined for the context of depth map sequence in a single layer bitstream. Compared to the existing depth representation information SEI message, the case of disparity is dropped since it only makes sense in the context of multiple views and this is, by definition, not possible to have multiples views in a single layer context.

Regarding the alpha map signalling, the SEI message parameter syntax could be reused as is in a new message while adapting the semantic for the single layer context. For brevity purposes, the text has been omitted in this contribution.

### Depth map information SEI message

#### Depth map information SEI message syntax

| depth_map_info( payloadSize ) { | Descriptor |
| --- | --- |
| z_near_flag | u(1) |
| z_far_flag | u(1) |
| depth_representation_type | ue(v) |
| if( z_near_flag ) |  |
| depth_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| if( z_far_flag ) |  |
| depth_rep_info_element( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| } |  |

#### Depth map information SEI message semantic

When present, the depth map information SEI message shall be associated with one or more layers with cti_content_tpe_idc value equal to 0.

When present, the depth map information SEI message may be included in any access unit. Preferably, when present, the depth map information SEI message is included for the purpose of random access in an access unit in which the coded picture with nuh_layer_id equal to targetLayerId is an IRAP picture.

The depth map information SEI message applies to all the pictures with nuh_layer_id equal to targetLayerId from the access unit containing the depth map information SEI message up to but excluding the next video picture, in decoding order, associated with a depth map information SEI message applicable to targetLayerId or to the end of the CLVS of the nuh_layer_id equal to targetLayerId, whichever is earlier in decoding order.

Table 3 – Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each decoded luma sample value of a picture represents an inverse of Z value that is uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 represents the inverse of ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents the inverse of ZNear (specified below). |
| 1 | Each decoded luma sample value of a picture represents a Z value uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 corresponds to ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents ZNear (specified below). |
| 2 | Each decoded luma sample value of a picture represents an Euclidean distance from the camera to the object in the scene. |
| 3.15 | Reserved |

## Option 3.2: Modifications of existing SEI messages to be used in single layer context

### Description

In this option, the current SEI messages syntax and semantics are modified to be allow their usage in the context of single layer bitstream. This has the advantage to not duplicate functionality across the specification.

Note that the way to express that a layer contains pictures carrying depth maps depends on the option presented above. As a result, the proposed text below remains generic when it comes to refer to such layer type.

For brevity, only the modifications for the Depth representation information SEI message are presented. Similar changes to allow the single layer context would need to be made if this option is chosen.

### Proposed text with tracked changes

      - Depth representation information SEI message semantics

        - General

The syntax elements in the depth representation information SEI message specify various parameters for auxiliary pictures of type AUX_DEPTH or non-auxiliary pictures carrying depth maps. Those parameters are signalled for the purpose of processing decoded pictures prior to rendering on a 3D display, such as view synthesis. Specifically, depth or disparity ranges for depth pictures are specified.

When present, the depth representation information SEI message shall be associated with either:

- one or more layers with AuxId value equal to AUX_DEPTH

- a layer with primary pictures carrying depth maps

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the depth representation information SEI message applies.

When present, the depth representation information SEI message may be included in any access unit. It is recommended that, when present, the SEI message is included for the purpose of random access in an access unit in which the coded picture with nuh_layer_id equal to targetLayerId is an IRAP picture.

For an auxiliary picture with AuxId[ targetLayerId ] equal to AUX_DEPTH, an associated primary picture, if any, is a picture in the same access unit having AuxId[ nuhLayerIdB ] equal to 0 such that ScalabilityId[ LayerIdxInVps[ targetLayerId ] ][ j ] is equal to ScalabilityId[ LayerIdxInVps[ nuhLayerIdB ] ][ j ] for all values of j in the range of 0 to 2, inclusive, and 4 to 15, inclusive.

The information indicated in the SEI message applies to all the pictures with nuh_layer_id equal to targetLayerId from the access unit containing the SEI message up to but excluding the next picture, in decoding order, associated with a depth representation information SEI message applicable to targetLayerId or to the end of the CLVS of the nuh_layer_id equal to targetLayerId, whichever is earlier in decoding order.

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

When associated with primary pictures carrying depth maps, it is a requirement of bitstream conformance that d_min_flag shall be equal to 0.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

When associated with primary pictures carrying depth maps, it is a requirement of bitstream conformance that d_max_flag shall be equal to 0.

depth_representation_type specifies the representation definition of decoded luma samples of auxiliary pictures as specified in Table G.3. In Table G.3, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

The variable maxVal is set equal to ( 1 << ( 8 + bit_depth_luma_minus8 ) ) − 1, where bit_depth_luma_minus8 is the value included in or inferred for the active SPS of the layer with nuh_layer_id equal to targetLayerId.

Table G.3 — Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each decoded luma sample value of an auxiliary picture represents an inverse of Z value that is uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 represents the inverse of ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents the inverse of ZNear (specified below). |
| 1 | Each decoded luma sample value of an auxiliary picture represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| 2 | Each decoded luma sample value of an auxiliary picture represents a Z value uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 corresponds to ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents ZNear (specified below). |
| 3 | Each decoded luma sample value of an auxiliary picture represents a nonlinearly mapped disparity, normalized in range from 0 to maxVal, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[ i ]. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| Other values | Reserved for future use |

When associated with primary pictures carrying depth maps, it is a requirement of bitstream conformance that depth_representation_type shall be equal to 0 or  2.

disparity_ref_view_id specifies the ViewId value against which the disparity values are derived.

NOTE 1	disparity_ref_view_id is present only if d_min_flag is equal to 1 or d_max_flag is equal to 1 and is useful for depth_representation_type values equal to 1 and 3.

The variables in the x column of Table G.4 are derived from the respective variables in the s, e, n, and v columns of Table G.4 as follows:

—	If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1 )s * 2e − 31 * ( 1 + n ÷ 2v ).

—	Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE 1	The above specification is similar to that found in IEC 60559:1989.

Table G.4 — Association between depth parameter variables and syntax elements

| x | S | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of a luma sample width of the coded picture with ViewId equal to ViewId of the auxiliary picture.

The units for the ZNear and ZFar values, when present, are identical but unspecified.

depth_nonlinear_representation_num_minus1 plus 2 specifies the number of piece-wise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

depth_nonlinear_representation_model[ i ] for i ranging from 0 to depth_nonlinear_representation_num_minus1 + 2, inclusive, specify the piece-wise linear segments for mapping of decoded luma sample values of an auxiliary picture to a scale that is uniformly quantized in terms of disparity. The values of depth_nonlinear_representation_model[ 0 ] and depth_nonlinear_representation_model[ depth_nonlinear_representation_num_minus1 + 2 ] are both inferred to be equal to 0.

NOTE 2	When depth_representation_type is equal to 3, an auxiliary picture contains nonlinearly transformed depth samples. The variable DepthLUT[ i ], as specified below, is used to transform decoded depth sample values from the nonlinear representation to the linear representation, i.e. uniformly quantized disparity values. The shape of this transform is defined by means of line-segment approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first ( 0, 0 ) and the last ( maxVal, maxVal ) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to maxVal, inclusive, with spacing depending on the value of nonlinear_depth_representation_num_minus1.

The variable DepthLUT[ i ] for i in the range of 0 to maxVal, inclusive, is specified as follows:

for( k = 0; k <= depth_nonlinear_representation_num_minus1 + 1; k++ ) {
          pos1 = ( maxVal * k ) / (depth_nonlinear_representation_num_minus1 + 2 )
          dev1 = depth_nonlinear_representation_model[ k ]
          pos2 = ( maxVal * ( k + 1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 )
          dev2 = depth_nonlinear_representation_model[ k + 1 ]	(G-10)

          x1 = pos1 − dev1
          y1 = pos1 + dev1
          x2 = pos2 − dev2
          y2 = pos2 + dev2

          for( x = Max( x1, 0 ); x <= Min( x2, maxVal ); x++ )
                           DepthLUT[ x ] = Clip3( 0, maxVal, Round( ( ( x − x1 ) * ( y2 − y1 ) ) ÷ ( x2 − x1 ) + y1 ) )
}

When depth_representation_type is equal to 3, DepthLUT[ dS ] for all decoded luma sample values dS of an auxiliary picture in the range of 0 to maxVal, inclusive, represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive.

# Conclusion

In this contribution, we report on the current standardisation of split rendering services (TS 26.565 [1]) in which video bitstreams are transmitted to XR devices to transport texture, alpha map and depth map sequences as separate streams. To this end, the contribution argues that there is a need to support the transport of alpha maps and depth maps in a single layer context, at least, in the HEVC specification.

We recommend documenting the preferred approach from the proposed alternatives in the next revised draft specification.

# Patent rights declaration(s)

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- 3GPP TS 26.565 Split Rendering Media Service Enabler

- The OpenXR™ Specification, The Khronos® OpenXR Working Group, Version 1.0.33, https://registry.khronos.org/OpenXR/specs/1.0/html/xrspec.htm

- Late stage reprojection - Azure Remote Rendering | Microsoft Learn, https://learn.microsoft.com/en-us/azure/remote-rendering/overview/features/late-stage-reprojection