| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, Online, 17-26 January2024 | Document: JVET-AG0152-v2 |
| --- | --- |

| Title: | EE2-1.3: SGPM with IntraTMP and IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser, Ya Chen, Fabrice Le Léannec | Email: | karam.naser@interdigital.com, |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution presents the experimental results of EE2-1.3 test on SGPM mode with IntraTMP and IBC (previously proposed in JVET-AF0226). It proposes further including block-vector based prediction obtained from IntraTMP or IBC modes of the neighboring blocks in the candidate list of SGPM. The simulation results are reported as follows:

AI: -0.05%	-0.03%	0.00%	EncT 99.7% DecT 100.0%

RA: -0.01%	 0.04%	 0.01%	EncT 99.4% DecT 99.1%

In SC classes (classF and TGM)

AI: -0.76%	-0.74%	-0.78%	EncT 97.2% DecT 99.1%

RA: -0.50%	-0.54%	-0.64%	EncT 99.8% DecT 98.7%

# Introduction

SGPM is an intra mode similar to the inter GPM mode, where 2 regular intra predictors are used instead of inter predictors. In this mode, a candidate list is built with each entry containing one partition split and two intra prediction modes as shown in Figure 1.

Figure 1 Spatial GPM candidates.

# Proposal

In this EE test, it is proposed extending the idea of geometric prediction combing block-vector based prediction and regular intra prediction from IBC-GPM to SGPM.  That is, SGPM mode tests block vectors of neighboring CU’s in addition to regular prediction modes. Specifically, the following is proposed:

- Obtain block vectors of all merge candidates

- Select the best block vector according to SATD template cost

- Test up to 6 best block vectors (if available) inside SGPM candidate list construction.

The difference with ECM-11 SGPM design is depicted in the Figure 2 below. It is about generalizing the two intra predictors from regular to regular or block-vector based:

Figure 2 modified SGPM process.

Identifiers intra_pred_0 and intra_pred_1 correspond to be regular prediction or block-vector based obtained from neighboring IntraTMP/IBC information.

# Experimental results

The proposal was implemented in ECM-11.0 and tested with CTC-AI and RA conditions. The following results are obtained:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.14% | 0.00% | 99.8% | 98.2% | 102.7% | 102.2% |
| Class A2 | -0.08% | 0.01% | 0.03% | 101.5% | 100.9% | 100.8% | 102.7% |
| Class B | -0.04% | 0.03% | -0.06% | 97.9% | 97.9% | 100.3% | 101.1% |
| Class C | -0.04% | -0.10% | 0.03% | 100.6% | 102.0% | 102.3% | 100.3% |
| Class E | -0.09% | 0.00% | 0.03% | 99.8% | 101.6% | 103.4% | 100.4% |
| Overall | -0.05% | -0.03% | 0.00% | 99.7% | 100.0% | 101.7% | 101.2% |
| Class D | -0.03% | -0.14% | -0.09% | 101.1% | 104.2% | 99.6% | 100.1% |
| Class F | -0.42% | -0.45% | -0.46% | 99.2% | 101.5% | 99.7% | 100.6% |
| Class TGM | -1.11% | -1.03% | -1.10% | 95.2% | 96.8% | 101.1% | 101.0% |
| Overall SC | -0.76% | -0.74% | -0.78% | 97.2% | 99.1% | 100.4% | 100.8% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.14% | 0.11% | 99.7% | 98.5% | 99.5% | 100.4% |
| Class A2 | -0.05% | 0.04% | 0.02% | 99.6% | 97.9% | 100.2% | 100.1% |
| Class B | 0.00% | 0.16% | -0.06% | 99.7% | 98.9% | 100.2% | 100.0% |
| Class C | -0.01% | 0.03% | 0.03% | 98.6% | 100.7% | 100.0% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01% | 0.04% | 0.01% | 99.4% | 99.1% | 100.0% | 100.1% |
| Class D | -0.07% | 0.10% | -0.31% | 100.3% | 99.5% | 100.3% | 100.0% |
| Class F | -0.38% | -0.25% | -0.43% | 99.6% | 100.2% | 99.9% | 99.2% |
| Class TGM | -0.62% | -0.83% | -0.85% | 100.0% | 97.3% | 100.2% | 100.3% |
| Overall SC | -0.50% | -0.54% | -0.64% | 99.8% | 98.7% | 100.0% | 99.7% |

# Conclusion

This contribution presents the results of EE2-1.3 test on SGPM mode with IntraTMP and IBC. This results in 0.05% gain in AI with higher gains in SC classes, without increasing the coding time.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).