| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0154-v2 |
| --- | --- |

| Title: | EE2-1.1: Decoder Derived Cross-Component Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Po-Han Lin, Vadim Seregin, Jian-Liang Lin, Marta Karczewicz | Tel: Email: | yjchang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In EE2 test 1.1a, a decoder derived cross-component prediction (DDCCP) mode with a mode flag signalling is applied to derive the best CCP mode by comparing the template costs between the evaluated CCPs. In EE2 test 1.1b, on top of test 1.1a, decoder-derived CCP fusion candidates are further introduced to derive CCP fusion modes selected based on the template costs. A fusion candidate is the combination of two CCP modes selected from the existing CCP mode lists reordered by template costs. The experimental results over ECM-11.0 are summarized as follows:

Test 1.1a:

|  | BD-rate Y | BD-rate U | BD-rate V | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra | -0.05% | 0.37% | 0.41% | 100.9% | 101.3% | 100.0% | 100.0% |

Test 1.1b:

|  | BD-rate Y | BD-rate U | BD-rate V | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra | -0.09% | -0.16% | -0.12% | 100.7% | 101.3% | 100.0% | 99.9% |
| Random access | -0.03% | -0.06% | -0.01% | 100.1% | 99.1% | 99.7% | 100.0% |

- Introduction

Cross-component linear model (CCLM) prediction mode is applied to VVC, where the chroma samples are predicted based on the reconstructed luma samples around the collocated positions. ECM [1] further adopts various types of cross-component prediction (CCP) modes, which include slope adjustment of CCLM, gradient LM (GLM), convolutional cross-component intra prediction mode (CCCM), no-subsampled CCCM, CCCM with multiple downsampling filters (CCCMwMDF), gradient and location based CCCM (GLCCCM), block-vector guided CCCM (BVG-CCCM), local-boosting CCP (LBCCP), and CCP merge (CCPmerge, also called non-local CCP). The newly introduced CCP modes improve the overall performance, especially in chroma components. The current CCP design causes an inefficient signaling while the best CCP mode for a chroma CU in a specific case is the one using longest bits. JVET-AF0176 provides a way to resolve the issue by signaling decoder derived flags to derive the best CCP mode using the template costs. The methods in JVET-AF0176 were investigated in EE2-1.1, and the results are reported in this contribution.

- Proposal

A decoder derived cross-component prediction (DDCCP) is proposed with a DDCCP flag signalled. When this DDCCP flag is true, it requires to construct a CCP mode list, and derive the best CCP mode by using the template (as shown in Figure 1) to calculate the template costs to compare the reconstructed samples and the prediction values generated by the evaluated CCP mode. The CCP mode list is constructed by single model CCLM, single model CCCM, multi-model CCCM, single model GLCCCM, single model CCCM applied with LBCCP, and multi-model CCCM applied with LBCCP.

It is further proposed to introduce various decoder-derived CCP fusion candidates. A fusion candidate is the combination of two CCP modes selected from the existing CCP mode lists reordered by template costs. A DDCCP fusion flag is signalled to select CCP fusion mode derived from an existing reordered CCP mode list.

Figure 1. The template adjacent to the current chroma CU

- Simulation results

There are two EE tests in 1.1 as follows:

| # | Test |
| --- | --- |
| 1.1a | Decoder derived CCP mode |
| 1.1b | Test 1.1a with decoder derived CCP fusion mode |

The tests are performed on top of reference software ECM-11.0. The simulations are run under Common Test Conditions for ECM [2]. The results are shown in the following tables.

Table 1. Simulation results of Test 1.1a.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | 0.51% | 0.41% | 100.5% | 101.2% | 100.0% | 100.0% |
| Class A2 | -0.04% | 0.28% | 0.29% | 100.8% | 101.3% | 100.0% | 100.0% |
| Class B | -0.04% | 0.48% | 0.44% | 101.3% | 101.0% | 100.0% | 99.9% |
| Class C | -0.04% | 0.26% | 0.43% | 100.9% | 101.7% | 100.0% | 100.0% |
| Class E | -0.08% | 0.28% | 0.47% | 101.0% | 101.2% | 100.0% | 100.0% |
| Overall | -0.05% | 0.37% | 0.41% | 100.9% | 101.3% | 100.0% | 100.0% |
| Class D | -0.06% | 0.38% | 0.43% | 101.2% | 101.8% | 99.9% | 99.8% |
| Class F | -0.04% | 0.23% | 0.68% | 100.3% | 100.4% | 100.0% | 100.0% |

Table 2. Simulation results of Test 1.1b.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.11% | -0.53% | -0.40% | 100.1% | 100.6% | 100.0% | 99.8% |
| Class A2 | -0.14% | -0.26% | -0.35% | 100.7% | 101.5% | 100.0% | 99.8% |
| Class B | -0.06% | -0.10% | -0.25% | 100.7% | 101.6% | 100.0% | 99.9% |
| Class C | -0.05% | -0.01% | 0.13% | 101.2% | 101.3% | 100.0% | 100.0% |
| Class E | -0.11% | -0.01% | 0.26% | 100.8% | 101.2% | 100.0% | 100.0% |
| Overall | -0.09% | -0.16% | -0.12% | 100.7% | 101.3% | 100.0% | 99.9% |
| Class D | -0.05% | 0.20% | 0.06% | 102.0% | 100.6% | 99.8% | 99.7% |
| Class F | -0.08% | -0.08% | 0.19% | 102.7% | 102.0% | 100.0% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | -0.40% | 0.00% | 100.2% | 99.9% | 100.1% | 100.3% |
| Class A2 | *-0.05% | 0.03% | -0.07% | 100.6% | 97.8% | 100.3% | 100.0% |
| Class B | -0.01% | 0.10% | -0.08% | 100.2% | 99.7% | 100.0% | 100.0% |
| Class C | -0.04% | -0.07% | 0.10% | 99.6% | 98.9% | 98.5% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.06% | -0.01% | 100.1% | 99.1% | 99.7% | 100.0% |
| Class D | -0.01% | 0.20% | 0.05% | 101.5% | 100.6% | 99.8% | 100.1% |
| Class F | -0.01% | -0.19% | -0.24% | 100.0% | 100.1% | 100.0% | 100.2% |

* 1 rate point data copied from anchor

- References

[1] M. Coban, R.-L. Liao, K. Naser, J. Ström, and L. Zhang, “Algorithm description of Enhanced Compression Model 11 (ECM 11),” JVET-AF2025, Dec. 2023.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).