| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0161-v1 |
| --- | --- |

| Title: | Crosscheck of EE1-2.3: Further complexity reduction on the joint LOP.2 (JVET-AG0163) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Informational |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy | Email: | dmytror@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results for EE1-2.3 (JVET-AG0163), in which a combination of new backbone block for LOP filter and a new training strategy are jointly tested. EE1-2.3 reports bd-rate change of {-0.03%, 1.83%, 1.54%} under AI and {-0.25%, 2.62%, 2.11%} under RA, against the NNVC7.1 LOP2 anchor (nnIntra ON). Reported complexity reduction is 3% in kMAC/pixel, number of convolution layers is reduced by 13%. Inference cross-check confirmed the results reported by the EE1-2.3 proponent.

For the training cross-check, two trainings were completed. The proposed network change was trained with an anchor training strategy (ee2.3_training0) and in a combination with a modified training strategy (ee2.3_training1), after proponent provided modified configuration files.

Floating point results of the anchor training (ee2.3_training0), demonstrates that EE1-2.3 network change provides bd-rate change of {0.1%, 2.5%, 2.7%} under AI and {0.1%, 2.8%, 2.6%} under RA comparing to the LOP2 anchor (integer model). Change in the training strategy (ee2.3_training1) reportedly provides additional bd-rate change of {-0.2%, -0.6%, -0.4%} in AI and {-0.3%, -0.2%, -0.4%} in RA, comparing to ee2.3_training0 results, both in floating point models.

The quantized model of the EE1-2.3 combing both, network change and training method test results in bd-rate change of {-0.03%	2.12%	2.22%} in AI and {-0.12%, 2.81%, 2.16%} in RA configurations, comparing to LOP2 anchor (nnIntra OFF). With nnIntra enabled, this quantized model provides bd-rate change of {-0.02%, 2.25%, 2.29%} in AI and {-0.13%, 3.16%, 2.23% } in RA configurations, comparing to NNVC7.1 LOP2 anchor (nnIntra ON).

# Introduction

In contribution JVET-AG0163, a new backbone block architecture is proposed for the LOP.2 ILF, shown in Figure 1. The skip connection location in BB is modified, which allows two consecutive 1x1 convolutions to be fused in a convolution layer. Introduced change provides complexity reduction of 3% in kMAC/pixel terms and reduces number of convolution layers by 13%.

In addition to the network change, proponent of JVET-AG0163 also introduced new training strategy, employing batch size 64 (instead of 128 in LOP2 anchor training)  and learning rate of 0.0008, instead of 0.0004 in anchor training.

Figure 3. The LOP model architecture proposed in JVET-AG0163.

# Cross-check of the inference testing

Proponents of the JVET-AG0163 provided floating point and integer models for testing.

Cross-checker successfully reproduced inference testing results reported by the proponent for both, floating point and int16 models. BD-rate change of the provided int16 model tested in the CTC against the NNVC7.1 LOP2 anchor (nnIntra ON) is shown in Table 1.

Table 1. Cross-checker’s inference results with EE1-2.3 int16 model provided by the proponent.

|  | AI |  |  | Run time |  | RA |  |  | Run time |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.01% | 1.81% | 0.47% | 95% | 88% | -0.30% | 2.68% | 1.42% | 100% | 96% |
| Class A2 | -0.02% | 1.17% | 1.45% | 95% | 91% | -0.33% | 2.38% | 2.06% | 100% | 101% |
| Class B | -0.05% | 1.90% | 1.23% | 94% | 89% | -0.17% | 2.54% | 2.26% | 101% | 103% |
| Class C | 0.02% | 2.53% | 1.96% | 98% | 88% | -0.27% | 2.84% | 2.46% | 102% | 102% |
| Class E | -0.06% | 1.45% | 2.67% | 98% | 90% |  |  |  |  |  |
| Overall | -0.03% | 1.83% | 1.54% | 96% | 89% | -0.25% | 2.62% | 2.11% | 101% | 101% |
| Class D | 0.11% | 2.60% | 2.55% | 99% | 90% | -0.25% | 2.98% | 2.47% | 101% | 102% |

# Cross-check of the training

For the training cross-check, two training were completed.

The first training (ee2.3_training0) was completed for the proposed network change was trained with an anchor training strategy. The second training (ee2.3_training1) was completed for the network change in a combination with a modified training strategy, after the proponent provided modified configuration files.

Assessment of the validation cost of the ee2.3_training1 suggests that modified parameters of the training lead validation cost to drop faster in the range 0..30 epochs, however stagnates after for Luma component and degrade for chroma. Validation cost exhibit higher level of fluctuation (instability).

Figure 3. EE2.3_training 1 validation cost

Inference results of ee2.3_training0, floating point model, comparing to LOP2 (integer model) are shown in Table 2. Inference results of ee2.3_training1, floating point model, comparing to LOP2 (integer model) are shown in Table 3. Table 4 shows bd-rate change resulting from the change in training strategy.

Table 2. Cross-checker’s comparative results of ee2.3_training0 float mode vs. LOP2 integer model.

|  | AI |  |  | Run time |  | RA |  |  | Run time |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.15% | 2.09% | 1.44% | 101% | 101% | 0.18% | 1.60% | 0.64% | 102% | 105% |
| Class A2 | 0.07% | 1.84% | 2.41% | 102% | 106% | 0.02% | 3.06% | 2.89% | 100% | 98% |
| Class B | 0.05% | 2.33% | 2.07% | 102% | 108% | 0.11% | 2.99% | 3.10% | 102% | 105% |
| Class C | 0.08% | 3.03% | 3.27% | 104% | 108% | 0.05% | 3.23% | 3.34% | 102% | 106% |
| Class E | 0.14% | 3.12% | 4.39% | 103% | 100% |  |  |  |  |  |
| Overall | 0.09% | 2.50% | 2.68% | 102% | 105% | 0.09% | 2.79% | 2.63% | 102% | 104% |
| Class D | 0.04% | 3.44% | 3.76% | 107% | 108% | 0.02% | 3.43% | 3.62% | 104% | 105% |

Table 3. Cross-checker’s comparative results of ee2.3_training1 float mode vs. LOP2 integer model.

|  | AI |  |  | Run time |  | RA |  |  | Run time |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.06% | 1.95% | 2.90% | 102% | 118% | -0.24% | 1.70% | 1.81% | 105% | 128% |
| Class A2 | -0.13% | 1.16% | 1.38% | 102% | 122% | -0.31% | 2.75% | 2.05% | 104% | 131% |
| Class B | -0.09% | 1.73% | 1.66% | 101% | 117% | -0.19% | 2.92% | 2.01% | 104% | 129% |
| Class C | -0.12% | 2.32% | 2.82% | 102% | 129% | -0.15% | 2.68% | 2.87% | 103% | 112% |
| Class E | -0.22% | 2.29% | 2.92% | 105% | 100% |  |  |  |  |  |
| Overall | -0.12% | 1.89% | 2.29% | 102% | 117% | -0.22% | 2.58% | 2.21% | 104% | 124% |
| Class D | -0.15% | 2.18% | 2.78% | 103% | 134% | -0.18% | 2.68% | 2.53% | 104% | 125% |

Table 4. Cross-checker’s comparative results of ee2.3_training1 vs. ee2.3_training0, all in float model.

|  | AI |  |  | Run time |  | RA |  |  | Run time |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.21% | -0.13% | 1.47% | 101% | 117% | -0.42% | 0.10% | 1.18% | 103% | 122% |
| Class A2 | -0.21% | -0.66% | -1.00% | 100% | 115% | -0.34% | -0.29% | -0.80% | 104% | 133% |
| Class B | -0.14% | -0.58% | -0.39% | 100% | 108% | -0.30% | -0.06% | -1.04% | 102% | 123% |
| Class C | -0.20% | -0.68% | -0.43% | 98% | 119% | -0.20% | -0.52% | -0.44% | 101% | 106% |
| Class E | -0.35% | -0.81% | -1.40% | 102% | 100% |  |  |  |  |  |
| Overall | -0.21% | -0.58% | -0.36% | 100% | 111% | -0.31% | -0.20% | -0.38% | 102% | 120% |
| Class D | -0.19% | -1.22% | -0.94% | 96% | 124% | -0.21% | -0.74% | -1.09% | 101% | 119% |

The resulting model of ee2.3_training1 was quantized and tested with NNVC7.1. Table 5 reports simulation results vs LOP2 ILF (nnIntra OFF) and Table 6 reports simulation results vs NNVC7.1 LOP2 anchor (nnIntra ON)

Table 5. Cross-checker’s results of ee2.3_training1 int16 model vs. LOP2 anchor (nnIntra OFF)

|  | AI |  |  | Run time |  | RA |  |  | Run time |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.06% | 2.20% | 2.60% | 96% | 99% | -0.09% | 1.89% | 1.76% | 97% | 95% |
| Class A2 | -0.04% | 1.57% | 1.32% | 95% | 93% | -0.16% | 2.76% | 1.69% | 104% | 96% |
| Class B | -0.03% | 2.05% | 1.75% | 97% | 98% | -0.10% | 3.36% | 2.11% | 97% | 97% |
| Class C | -0.06% | 2.38% | 2.69% | 105% | 86% | -0.14% | 2.84% | 2.88% | 104% | 99% |
| Class E | -0.08% | 2.35% | 2.89% | 102% | 109% |  |  |  |  |  |
| Overall | -0.03% | 2.12% | 2.22% | 99% | 96% | -0.12% | 2.81% | 2.16% | 103% | 94% |
| Class D | -0.09% | 2.27% | 2.84% | 112% | 90% | -0.22% | 2.76% | 2.79% | 103% | 103% |

Table 6. Cross-checker’s results of ee2.3_training1 int16 model vs. NNVC7.1 LOP2 anchor (nnIntra ON).

|  | AI |  |  | Run time |  | RA |  |  | Run time |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.07% | 2.31% | 2.51% | 99% | 99% | -0.09% | 2.30% | 1.71% | 98% | 97% |
| Class A2 | -0.02% | 1.75% | 1.42% | 96% | 95% | -0.21% | 3.36% | 1.96% | 96% | 98% |
| Class B | -0.03% | 2.16% | 1.85% | 97% | 97% | -0.10% | 3.68% | 2.11% | 97% | 98% |
| Class C | -0.05% | 2.46% | 2.74% | 98% | 83% | -0.14% | 2.99% | 2.99% | 100% | 98% |
| Class E | -0.07% | 2.56% | 3.07% | 100% | 94% |  |  |  |  |  |
| Overall | -0.02% | 2.25% | 2.29% | 98% | 93% | -0.13% | 3.16% | 2.23% | 98% | 98% |
| Class D | -0.09% | 2.33% | 2.84% | 104% | 93% | -0.22% | 2.65% | 2.84% | 99% | 101% |

# Conclusion

This contribution reports the cross-check results for EE1-2.3 (JVET-AG0163 [1]). Inference cross-check confirmed the results reported by the EE1-2.3 proponent. For the training cross-check, two trainings were completed. The proposed network change was trained with an anchor training strategy (ee2.3_training0) and in a combination with a modified training strategy (ee2.3_training1), after proponent provided modified configuration files.

Floating point results of the anchor training (ee2.3_training0), demonstrates that EE1-2.3 network change provides bd-rate change of {0.1%, 2.5%, 2.7%} under AI and {0.1%, 2.8%, 2.6%} under RA comparing to the LOP2 anchor (integer model). Change in the training strategy (ee2.3_training1) reportedly provides additional bd-rate change of {-0.2%, -0.6%, -0.4%} in AI and {-0.3%, -0.2%, -0.4%} in RA, comparing to ee2.3_training0 results, both in floating point models.

The quantized model of the EE1-2.3 combing both, network change and training method test results in bd-rate change of {-0.03%	2.12%	2.22%} in AI and {-0.12%, 2.81%, 2.16%} in RA configurations, comparing to LOP2 anchor (nnIntra OFF).