| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0162_v1 |
| --- | --- |

| Title: | EE1-1.5: Report on implementation of HOP In-loop filter with Transformer blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yun Li Dmytro Rusanovskyy Marta Karczewicz | Tel: Email: | yli30@qti.qualcomm.com dmytror@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A unified filter architecture for the High-performance Operation Point (HOP) was defined at JVET-AD0380. Updated training strategy and configuration were presented in JVET-AG0041, and complete inference results for HOP training are reported in JVET-AF0014.

Based on the architecture of the HOP network, Transformer blocks were introduced in JVET-AF0158, and the inference was performed by using the Pytorch inference engine. This model has improved the coding gain significantly compared to HOP. EE1-1.5 aimed to integrate HOP with transformer with SADL support. However, it was identified, that some operators in the transformer need to be approximated when the integer quantization is concerned, and they can not be introduced into the SADL implementation directly.

In this document, the progress of transformer implementation with SADL support is reported. At this meeting, simplified attention mechanism of the transformer block was implemented. The filter architecture of EE1-1.5 was modified and constrained to the complexity and memory requirements of the HOP architecture. The training was conducted by using the NNVC7.1 HOP training guidelines, and SADL-int16 quantized model was used for the inference.

Complexity of the model in kMAC/pixel: 475 (block-wise) with 1.44 M parameters. The proposed filter (int16 model) with NNIntra ON provides:

{-13.1%, -16.1%, -17.1%} for AI test configuration, and {-13.6%, -19.1%, -19.3%} for RA test configuration, versus the VTM.

Compared to the NNVC7.1 HOP anchor (NNIntra ON), the proposed filter provides: {-0.7%, -5.5%, -4.7%} for the AI configuration, and {-0.04%, -7.6%, -5.7%} for the RA configuration.

the performance of RA may be improved by applying a new two-stage RA data extraction, and the coding gain for Y and UV components can be rebalanced by using a new weighting in the loss function for the training.

It is proposed to continue EE1 study of the proposed method and the full Transformer architecture in the next round of EE.

# Introduction

A unified filter architecture for the High-performance Operation Point (HOP) was defined at JVET-AD0380. Updated training strategy and configuration were presented in JVET-AG0041, and complete inference results for HOP training are reported in JVET-AF0014.

Based on the architecture of the HOP network, Transformer blocks were introduced in JVET-AF0158 [1]. This model has improved the coding gain significantly compared to HOP. However, some operators of the Transformer architecture, e.g., Softmax, LayerNorm, were found to be non-hardware friendly and required approximation in SADL int16 implementation. For example, the exponent operator in Softmax and the square root operator in the LayerNorm cannot be quantized directly without an approximation. In addition, they are non-element-wise operators. These attributes can lead to problems that requires an extra step for an efficient hardware implementation.

At this meeting, simplified attention mechanism of the transformer block with int16 SADL support was implemented and tested in EE1-1.5.

- Proposed method

Block diagram of the EE1-1.5 experiment is shown in Figure 1.

Figure 1 HOP with transformer blocks, JVET-AF0158.

At this meeting, simplified attention mechanism of the transformer block with int16 SADL support was implemented and tested in EE1-1.5. Figure 2 shows the block diagram of the current implementation, with non-implemented blocks of the Transformer being shown in shadow-grey color. Furthermore, flipping [2], and 1x1 convolution for IPB and BS are utilized.

In order to simplify the above-mentioned quantization problem and preserve the self-attention mechanism in the filtering system, a Transformer block from prior arts [3][4] is modified and shown in Fig. 1, where Fig. 1(a) is the original self-attention Transformer module with a Feed-forward Network (FFN), and Fig. (b) is the Attention block derived from the Transformer module with all normalization layers and FFN removed.

Figure 2 Simplified Attention block from the Transformer: (a) The complete Transformer; (b) Simplified Attention block implemented with int16 SADL.

The resulting Attention block was integrated into the end of the Residual Backbone Block (RBB), which is referred to as HOP Attention Block (HOP AB), illustrated in Fig. 3.

Figure 3 An Attention block is integrated into the backbone block of HOP network as the HOP attention block.

In this proposal, only two HOP Attention Blocks are utilized to replace the 8th and the 15th HOP backbone block, respectively, whereas the rest of the HOP backbone blocks remain unchanged.  To adjust the complexity to that of HOP, the number of residual blocks is increased to 26 due to the rank of the decomposition being reduced to 48. This configures the model to the complexity of 475 kMAC/pixel (block-wise), and the number of parameters is 1.44 M.

The information for the inference and the training stage is presented in Table 1 and 2, respectively.

Table1. Network information for the inference

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | HW environment: |  |  |
|  | GPU Type |  | N/A |
|  | Framework: |  | SADL |
|  | Number of GPUs per Task |  | 0 |
|  | Total Parameter Number |  | 1.44M /model |
|  | Parameter Precision (Bits) |  | 32 or 16 |
|  | Memory Parameter (MB) |  |  |
|  | Multiply Accumulate (kMAC/pixel) block (frame) based |  | 475 (375 frame-wise) |
|  | Total Conv. Layers |  | 180 |
|  | Total FC Layers |  | 0 |
|  | Total Memory (MB) | Float |  |
|  |  | Int16 | 2.87/model |
|  | Batch size: |  | 1 |
|  | Patch size |  | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage |  |  |
|  | Other information: |  |  |

Table 2. Network information for the training

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla A100 |
|  | Framework: | PyTorch v3.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | ~280h |
|  | Training data information: | DIV2K BVI TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

- Simulation results

The method was integrated in the NNVC7.1 and tested following the EE1 CTC [5]. The proposed method has been trained following the EE1-0 training procedure (Stage 3 HOP training only) specified in JVET-AE0191 and updated for EE1 testing as specified in JVET-AG0041. The training is being conducted with a target of 20 epochs as specified by the guidance.

The proposed method was tested with NNVC7.1 reference software with a fixed block size of 128x128, and the trained model is quantized to uniform11-bit for the parameters with SADL. In addition, the SADL is also modified for running the Attention module.

Summary of the inference results are reported below, complete results in CTC template are provided along this document.

Comparative results to the NNVC7.1 HOP2 filter of EE1-0 are shown in Table 3. NN Intra was enabled for both the test and the anchor. Comparison against VTM is shown in Table 4.

As observed in Tables 3, the proposed filter outperforms the HOP anchor with NNIntra ON by {-0.7%, -5.5%, -4.7%} for the AI configuration and {-0.04%, -7.6%, -5.7%} for the RA configuration, respectively. New round of data extraction is likely needed to improve the performance of RA.

Compared to VTM, shown in Table 4, the proposed method provides {-13.1%, -16.1%, -17.1%} for the AI configuration, and {-13.6%, -19.1%, -19.3%} for the RA configuration, respectively.

Table 3 Proposed method (int16 model) vs NNVC7.1 HOP, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.71% | -3.57% | -3.86% | 0.18% | -6.26% | -4.12% |
| Class A2 | -0.69% | -7.23% | -11.66% | 0.01% | -8.39% | -15.66% |
| Class B | -0.76% | -7.40% | -2.86% | -0.13% | -13.57% | -4.43% |
| Class C | -0.53% | -2.23% | -2.23% | -0.14% | -0.48% | -0.83% |
| Class E | -0.61% | -6.62% | -4.80% |  |  |  |
| Overall | -0.66% | -5.46% | -4.68% | -0.04% | -7.58% | -5.66% |
| Class D | -0.18% | -2.03% | -1.96% | 0.15% | -1.21% | -0.76% |
| Class F | -0.57% | -2.63% | -1.58% | -0.17% | -0.68% | -1.26% |

Table 4 Proposed method (int16 model) vs VTM, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -13.44% | -11.48% | -18.43% | -15.13% | -13.65% | -22.03% |
| Class A2 | -12.94% | -16.82% | -19.89% | -14.51% | -18.92% | -24.19% |
| Class B | -11.68% | -17.23% | -13.71% | -12.41% | -24.02% | -16.02% |
| Class C | -12.59% | -14.40% | -16.89% | -13.34% | -17.15% | -17.54% |
| Class E | -16.14% | -20.18% | -18.78% |  |  |  |
| Overall | -13.13% | -16.06% | -17.08% | -13.62% | -19.09% | -19.26% |
| Class D | -11.88% | -13.19% | -16.66% | -14.20% | -17.00% | -18.08% |
| Class F | -9.38% | -13.28% | -12.17% | -8.45% | -12.22% | -11.79% |

- Conclusion

In this contribution progress of EE1-1.5 test, SADL implementation of HOP+Transformer filter is reported. Simplified Attention mechanism of the Transformer was implemented with int16 SADL and introduced into the HOP architecture combined with the lower rank decomposition and flipping to improve the performance of HOP.

Compared to NNVC-7.1 HOP anchor (NNIntra ON), the EE1-1.5 filter (int16 model) provides: AI: {-0.7%, -5.5%, -4.7%} and RA: {-0.04%, -7.6%, -5.7%}. Compared to VTM, it provides: AI: {-13.1%, -16.1%, -17.1%}, and RA: {-13.6%, -19.1%, -19.3%}.

The performance of RA may be improved by using a new two-stage RA data extraction, and the coding gain for Y and UV components can be rebalanced by using a new weighting in the loss function for the training.

It is proposed to continue studying the proposed method and the full Transformer module in the next round of EE.

- References

- EE1-1.5 HOP In-loop filter with transformer blocks, JVET-AF0158, Y. Li, D. Rusanovskyy, T. Ryder, S. Eadie, M. Karczewicz.

- EE1-1.2 Complexity-performance tradeoff of decomposition, JVET-AE0164, D. Rusanovskyy, Y. Li, M. Karczewicz.

- S. Khan, M. Naseer, M. Hayat, S.W. Zamir, F. S. Khan, M. Shah, “Transformers in vision: A Survey”, arXiv:2101.01169v5, 2022

- H. Zhang, I. Goodfellow, D. Metaxas, A. Odena, “Self-attention generative adversarial networks”, in international conference on machine learning, 2019

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).