| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0164-v1 |
| --- | --- |

| Title: | EE2-2.10: GPM with affine prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Zhipin Deng2, Li Zhang1 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

GPM with affine prediction is tested in EE2-2.10, where a GPM partition can be predicted by affine motion compensation (AMC) or non-affine motion compensation (non-AMC), indicated by a flag. It is reported that on top of ECM-11.0, simulation results of the proposed method are as below:

RA in average: -0.17 % (Y), -0.19 % (U), -0.17 % (V), 101.1% (EncT), 100.8% (DecT).

LB in average: -0.10 % (Y), 0.31% (U), 0.20 % (V), 101.7% (EncT), 100.7% (DecT).

RA on the TGM class: -0.24% (Y), -0.18% (U), -0.16% (V), 100.3% (EncT),	101.3% (DecT).

LB on the TGM class: -0.51% (Y), -0.39% (U), -0.39% (V), 101.5% (EncT),	101.0% (DecT).

# Introduction

The GPM mode in ECM splits a CU into two partitions separated by a straight splitting line. The motion compensation for each GPM partition is performed to the whole CU, and a sample-based weighted blending process is used to generate the final prediction. Each partition of a GPM CU is merge-predicted with its own merge candidate index. In ECM, affine prediction is disallowed to predict a GPM partition.

# Proposed method

In this contribution, GPM is further extended to enable affine motion compensation (AMC). Therefore, a GPM partition can be predicted by AMC inter-prediction, non-AMC inter-prediction or intra-prediction. In addition, a GPM partition predicted by AMC can be combined with the other GPM partition predicted by AMC, non-AMC, or intra-prediction.

When AMC is applied, a uni-prediction affine merge candidate list is constructed from the subblock-based merge candidate list after discarding sub-TMVP candidates, similar to the uni-prediction merge candidate list construction for GPM in VVC. AMC is performed for a GPM partition using the control point motion vectors (CPMVs) of a merge candidate in the uni-prediction affine merge candidate list. The length of the uni-prediction affine merge candidate list is signalled in SPS. When ARMC is applicable, the uni-prediction affine merge candidate list is reordered according to the template costs.

A gpm_affine_flag is signaled for each GPM partition to indicate whether AMC is applied for the GPM partition. A merge candidate index for the GPM partition is signaled using individual arithmetic context models depending on whether AMC or non-AMC is applied.

In the current implementation, AMC is not allowed for GPM-MMVD and GPM-TM.

# Experimental results

The proposed method has been implemented on top of the ECM-11.0 [1]. Results under RA/LDB configurations are presented in Table 1. The simulations were performed in accordance with the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method on ECM-11.0

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.31% | -0.23% | 100.9% | 100.2% |  |  |  |  |  |
| Class A2 | -0.22% | -0.07% | -0.21% | 101.7% | 100.0% |  |  |  |  |  |
| Class B | -0.15% | -0.23% | -0.10% | 101.2% | 101.6% | -0.04% | 0.67% | 0.48% | 101.8% | 99.3% |
| Class C | -0.20% | -0.15% | -0.18% | 100.6% | 100.9% | -0.01% | -0.20% | -0.12% | 101.5% | 102.0% |
| Class E |  |  |  |  |  | -0.29% | 0.40% | 0.15% | 101.8% | 101.5% |
| Overall | -0.17% | -0.19% | -0.17% | 101.1% | 100.8% | -0.10% | 0.31% | 0.20% | 101.7% | 100.7% |
| Class D | -0.15% | -0.24% | -0.26% | 100.7% | 101.3% | -0.17% | -0.32% | 0.02% | 101.9% | 100.6% |
| Class F | -0.11% | -0.01% | 0.00% | 100.1% | 101.5% | -0.13% | -0.11% | -0.16% | 99.2% | 101.2% |
| Class TGM | -0.24% | -0.18% | -0.16% | 100.3% | 101.3% | -0.51% | -0.39% | -0.39% | 101.5% | 101.0% |

# Conclusions

GPM-affine is presented in this contribution. Simulation results indicate 0.17%/0.10% luma coding gains in RA/LDB configurations with 101.1%/100.8% and 101.7%/100.7% complexity increase. It is recommended to adopt this contribution in ECM.

# References

[1] ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).