| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–27 January 2024 | Document: JVET-AG0169-v1 |
| --- | --- |

| Title: | AHG9: Comments on use of NNPF SEI messages in the SEI processing order SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen Olena Chubach Yu-Wen Huang Shawmin Lei | Email: | {lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the semantic text for indicating NNPF SEI messages in the SPO SEI message.

# Introduction

The draft of the SEI processing order SEI message is updated as JVET-AF2027 from the last JVET meeting. The current design for the SPO SEI message specifies that when an SEI processing order SEI message with a particular value of po_id is present in any access unit of a CVS, an SEI processing order SEI message with that particular value of po_id shall be present in the first access unit of the CVS in decoding order.

The NNPF SEI messages can be indicated with other SEI messages for a preferred processing order specified in the SEI processing order (SPO) SEI message. The NNPF SEI messages consist of the NNPF characteristic (NNPFC) SEI message and the NNPF activation (NNPFA) SEI message.

# Proposal

The NNPFC SEI messages for a base NNPF or the update NNPFs may be present in a CVS, while the NNPFA SEI messages can selectively activate that the target NNPF is the base NNPF (when nnpfa_target_base_flag equal to 1) with nnpfc_id equal to nnpfa_target_id or the update NNPF (when nnpfa_target_base_flag equal to 0) specified by the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id that precedes the first VCL NAL unit of the current picture in decoding order and is not a repetition of the NNPFC SEI message that contains the base NNPF.

When NNPF SEI messages are indicated in the SPO SEI message, however, it needs to further clarify the mechanism for updating NNPF and activating NNPF. This contribution proposes the following semantic text (highlighted in green) for indicating NNPF SEI messages in the SPO SEI message.

po_sei_wrapping_flag[ i ] equal to 1 specifies that one or more processing order nesting SEI messages with both of the following constraints should be present:

- pon_target_po_id[ j ] with any value of j is equal to po_id.

- There is a k-th loop entry in the SEI processing order nesting SEI message such that the payloadType of the k-th nested SEI message is equal to po_sei_payload_type[ i ] and pon_processing_order[ k ] is equal to po_sei_processing_order[ i ].

When po_sei_wrapping_flag[ i ] is equal to 0, an SEI message with payloadType equal to po_sei_payload_type[ i ] (and, when po_sei_prefix_flag[ i ] equal to 1, prefix data that matches the values of po_sei_prefix_data_bit[ i ][ j ]) should be present outside of the processing order nesting SEI message. However, if po_sei_wrapping_flag[ i ] is equal to 0 and no SEI message is present with payloadType equal to po_sei_payload_type[ i ] or po_sei_wrapping_flag[ i ] is equal to 0 and po_sei_prefix_flag[ i ] is equal to 1 and no SEI message is present with with payloadType equal to po_sei_payload_type[ i ] that has prefix data that matches the values of po_sei_prefix_data_bit[ i ][ j ], the following applies:

- If po_sei_importance_flag[ i ] is equal to 1, the decoder should ignore the entire SEI processing order SEI message.

- Otherwise, the decoder should ignore all data associated with the loop variable value of i.

NOTE 2 – po_sei_wrapping_flag[ i ] equal to 1 enables SEI messages to be carried within the processing order nesting SEI message to prevent such SEI messages from being incorrectly interpreted by decoders that do not process the SEI processing order SEI message. Thus, po_sei_wrapping_flag[ i ] equal to 1 is intended to be used when po_sei_wrapping_flag[ i ] equal to 0 can lead to unintended results being produced by such decoders.

When both the NNPFC and NNPFA SEI messages are indicated as individual SEI messages with the same processing order number in the processing chain specified by the SPO SEI message with a po_id, the following applies:

- When po_sei_wrapping_flag[ i ] is equal to 1, both the i-th NNPFC and NNPFA SEI messages are the processing-order-nested SEI messages in the processing order nesting SEI message and there shall not be any other NNPFC SEI messages with the nnpfc_id present in a CVS. The NNPF with a nnpfc_id in the NNPFC SEI message is a base NNPF and the NNPFA SEI message activates the target NNPF with nnpfa_target_id equal to nnpfc_id, which is this base NNPF.

- Otherwise (when po_sei_wrapping_flag[ i ] is equal to 0), both the i-th NNPFC and NNPFA SEI messages are present in the SPO SEI message with a po_id. The NNPF with a nnpfc_id in the i-th indicated NNPFC SEI message is a base NNPF and one or more NNPFC SEI messages for the nnpfc_id of the target NNPF may be present in a CVS. The NNPFA SEI message can be set to activate the base NNPF with nnpfa_target_id equal to nnpfc_id or the update NNPF with nnpfa_target_id equal to nnpfc_id specified by the last NNPFC SEI message preceding the SPO SEI message with that particular value of po_id in any access unit of a CVS.

- The NNPFC SEI message payload shall precede the NNPFA SEI message in the SPO SEI message or in the processing order nesting SEI message.

po_sei_importance_flag[ i ] indicates the degree of importance determined by the encoder for the type of SEI message with index i.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).