| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0175-v1 |
| --- | --- |

| Title: | EE1-1.3: Separate models for HOP filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Chaoyi Lin, Junru Li, Kai Zhang, Li Zhang (Bytedance) Franck Galpin (InterDigital) Dmytro Rusanovskyy (Qualcomm) Renjie Chang (Tencent) | Email: | yue.li@bytedance.com Franck.galpin@interdigital.com dmytror@qti.qualcomm.com renjiechang@tencent.com |
| Source: | Bytedance, InterDigital, Qualcomm, Tencent |  |  |

_____________________________

- Abstract

This contribution presents the test result of EE1-1.3, where separate models from JVET-AF0183 are directly used for filtering luma and chroma in HOP filter without retraining. Inference process is aligned with HOP.2 in NNVC-7.1. BD-rate changes on top of NNVC-7.1 HOP.2 anchor (NN intra + HOP.2) are reportedly summarized as below:

RA: 0.15%, -2.40%, -1.13%, AI: 0.08%, -1.85%, -1.29%

kMAC/pixel: 473 (366 + 107)

Introduction

In JVET-AF0183 [1], considering that luma and chroma have different characteristics, separate models are designed to filter luma and chroma components, as shown in Fig. 1 and Fig. 2. In addition, luma and chroma are allowed to select different filtering parameters.

Fig. 1. Proposed HOP luma model in JVET-AF0183.

Fig. 3. Proposed HOP chroma model in JVET-AF0183.

Proposed method

This EE test directly deploys the separate models from JVET-AF0183 into NNVC-7.1 and inherits the inference interface of HOP.2, i.e., luma and chroma share the same filtering parameters.

Note that the improved training procedure of HOP.2, e.g. larger learning rate and smaller batch size, has not been applied to train the separate models.

Inference

SADL is used for performing the inference of the proposed NN filters in VTM. Both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 1.

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1.42M |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | Total Parameters Number × 2 |
|  | Multiply Accumulate (kMAC/pixel) | 473 |
| Optional | Total Conv. Layers | 154 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144×144, 272×272 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

HOP stage 3 training is applied. The network information in the training stage is provided in Table 2.

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: A100-SXM-80GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | 200h |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | NNVC, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |

Experimental results

The proposed method is integrated into NNVC-7.1 and tested according to the common test conditions defined in [2].

Compared with NNVC-7.1 HOP.2 anchor (nn intra + hop.2), the proposed method brings on average {0.15%, -2.40%, -1.13%} and {0.08%, -1.85%, -1.29%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 HOP.2 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.48% | -4.35% | -0.13% | 89% | 86% |
| Class A2 | -0.01% | -3.38% | -1.17% | 91% | 87% |
| Class B | 0.06% | -3.05% | -2.40% | 90% | 87% |
| Class C | 0.12% | 0.60% | -0.26% | 90% | 84% |
| Class E |  |  |  |  |  |
| Overall | 0.15% | -2.40% | -1.13% | 90% | 86% |
| Class D | 0.04% | -1.15% | -0.62% | 90% | 86% |
| Class F | 0.00% | 0.20% | 0.39% | 90% | 88% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-7.1 HOP.2 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.14% | -3.84% | -0.74% | 91% | 87% |
| Class A2 | 0.01% | -2.41% | -0.16% | 94% | 89% |
| Class B | -0.03% | -1.37% | -1.97% | 96% | 84% |
| Class C | 0.10% | -0.41% | -0.89% | 95% | 83% |
| Class E | 0.22% | -2.00% | -2.35% | 87% | 85% |
| Overall | 0.08% | -1.85% | -1.29% | 93% | 85% |
| Class D | 0.12% | -1.85% | -1.56% | 97% | 91% |
| Class F | -0.06% | -1.33% | -0.99% | 94% | 89% |

Conclusions

This contribution presents a separate models-based design for HOP. The proposed filtering method brings obviously higher chroma gains compared with existing single model-based HOP, at the cost of a slight luma performance drop. It is asserted that the coding gain of separate models may be further improved, if the refined training procedure used in HOP.2 is applied to the training of the separate models. It is therefore suggested to continue studying the proposed method in next round of EE.

References

- Y. Li, K. Zhang, L. Zhang, EE1-1.1.4.c/d: Separate models for HOP, JVET-AF0183, Hannover, DE, October 2023

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AF2016, Hannover, DE, October 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).