| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0192-v2 |
| --- | --- |

| Title: | AHG9: Miscellaneous NNPF items related to VUI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | xujizheng@bytedance.com yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the VSEI text:

Item 1: Fix a typo when vui_chroma_sample_loc_type_frame is mentioned in the VSEI text.

Item 2: Fix two bugs for the constraints on nnpfc_matrix_coeffs and add two NOTEs in the semantics of nnpfc_matrix_coeffs to clarify complicated cases.

Item 3: Fix an asserted bug on the inference of nnpfc_full_range_flag.

Item 4: Add a missing inference and a NOTE on nnpfc_chroma_sample_loc_type_frame.

Item 5: Enable sending of nnpfc_matrix_coeffs and nnpfc_full_range_flag when nnpfc_out_format_idc is not equal to 1.

Item 6: Add a NOTE about the aspect ratio of NNPF output pictures when ResolutionResamplingFlag is equal to 1.

Item 7: Enable sending the overscan information for NNPF output pictures.

In the v2 document, updated text changes for item 3, which were agreed during the meeting discussion, are added.

# Introduction

In the following, added or modified parts are highlighted in turquoise and deleted parts are highlighted in yellow strikethrough in red fonts. Unless indicated otherwise, the text changes are based on the document JVET-AE2006-v2.docx.

# Item 1: On the typo of vui_chroma_sample_loc_type_frame

## Problem statement

In the latest published VSEI specification, in the semantics of vui_chroma_sample_loc_type_frame, it is described as:

vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field, and vui_chroma_sample_loc_type_bottom_field, when present, specify the location of chroma samples as follows:

—	If GeneralProgressiveSourceFlag is equal to 1, GeneralInterlacedSourceFlag is equal to 0, and ChromaFormatIdc is equal to 1 (4:2:0 chroma format), vui_chroma_sample_loc_type_frame specifies the location of chroma samples for both fields of each frame of the CLVS as shown in Figure 1.

—	Otherwise, if ChromaFormatIdc is equal to 1 (4:2:0 chroma format), vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field specify the location of chroma samples for each top field and bottom field of the CLVS, respectively, as shown in Figure 1.

—	Otherwise (ChromaFormatIdc is not equal to 1), the values of the syntax elements chroma_sample_loc_type, vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field shall be ignored.

## Proposed change

The following text changes are based on the latest VSEI v3 published standard specification:

—	Otherwise (ChromaFormatIdc is not equal to 1), the values of the syntax elements vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field shall be ignored.

# Item 2: On the semantics of nnpfc_matrix_coeffs

## Problem statement

The semantics of nnpfc_matrix_coeffs are described as:

nnpfc_matrix_coeffs describes the equations used in deriving luma and chroma signals from the green, blue, and red, or Y, Z, and X primaries. Its semantics apply to the pictures resulting from applying the NNPF specified in this SEI message and are as specified for MatrixCoefficients in Rec. ITU-T H.273 | ISO/IEC 23091-2 with BitDepthY and BitDepthC being equal to outTensorBitDepthY and outTensorBitDepthC, respectively.

When nnpfc_matrix_coeffs is not present in the NNPFC SEI message, the value of nnpfc_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

nnpfc_matrix_coeffs shall not be equal to 0 unless both of the following conditions are true:

–	nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8.

–	nnpfc_out_order_idc is equal to 2, outSubHeightC is equal to 1, and outSubWidthC is equal to 1.

nnpfc_matrix_coeffs shall not be equal to 8 unless one of the following conditions is true:

–	nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8 + 1, nnpfc_out_order_idc is equal to 2, outSubHeightC is equal to 1, and outSubWidthC is equal to 1.

In the above two constraints, nnpfc_out_tensor_luma_bitdepth_minus8 and nnpfc_out_tensor_chroma_bitdepth_minus8 may not exist, which should not be used in such a case. It is proposed to use nnpfc_out_tensor_luma_bitdepth_minus8 and nnpfc_out_tensor_chroma_bitdepth_minus8 when they both are present.

In addition, when not present, nnpfc_matrix_coeffs is inferred to be equal to vui_matrix_coeffs. When vui_matrix_coeffs is equal to 0 or 8, it is easy for an encoder to violate one of the two constraints. It is suggested to add two NOTEs to clarify such cases.

## Proposed change

When nnpfc_matrix_coeffs is not present in the NNPFC SEI message, the value of nnpfc_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

nnpfc_matrix_coeffs shall not be equal to 0 unless allboth of the following conditions are true:

–	Both nnpfc_out_tensor_chroma_bitdepth_minus8 and nnpfc_out_tensor_luma_bitdepth_minus8 are present.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8.

–	nnpfc_out_order_idc is equal to 2, outSubHeightC is equal to 1, and outSubWidthC is equal to 1.

NOTE: When all of the above three conditions are true and vui_matrix_coeffs is equal to 0, nnpfc_matrix_coeffs is expected to be present.

nnpfc_matrix_coeffs shall not be equal to 8 unless one of the following conditions is true:

–	Both nnpfc_out_tensor_chroma_bitdepth_minus8 and nnpfc_out_tensor_luma_bitdepth_minus8 are present; and nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8.

–	Both nnpfc_out_tensor_chroma_bitdepth_minus8 and nnpfc_out_tensor_luma_bitdepth_minus8 are present; and nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8 + 1, nnpfc_out_order_idc is equal to 2, outSubHeightC is equal to 1, and outSubWidthC is equal to 1.

NOTE: When one of the above two conditions is true and vui_matrix_coeffs is equal to 8, nnpfc_matrix_coeffs is expected to be present.

# Item 3: On the inference of nnpfc_full_range_flag

## Problem statement

In the semantics of nnpfc_separate_colour_description_present_flag, it is described that the scaling and offset values applied in association with the matrix coefficients for the picture resulting from the NNPF is the same as implied by the VUI parameters when they are not signalled. According to such semantics, nnpfc_full_range_flag is inferred to be vui_full_range_flag, when nnpfc_full_range_flag is not present.

However, in the semantics of nnpfc_full_range_flag, the value is inferred to be equal to 0, when nnpfc_full_range_flag is not present. It conflicts with the description of nnpfc_separate_colour_description_present_flag.

## Proposed change

It is suggested to infer the value of nnpfc_full_range_flag from vui_full_range_flag, following the inferences of other syntax elements for colour description, i.e. nnpfc_colour_primaries, nnpfc_transfer_characteristics and nnpfc_matrix_coeffs. The text changes are:

nnpfc_full_range_flag indicates the scaling and offset values applied in association with the matrix coefficients as specified by nnpfc_matrix_coeffs. Its semantics are as specified for the VideoFullRangeFlag parameter in Rec. ITU-T H.273 | ISO/IEC 23091-2. When not present, the value of nnpfc_full_range_flag is inferred to be equal to vui_full_range_flag0.

During the meeting discussion, the following text changes are agreed:

nnpfc_full_range_flag indicates the scaling and offset values applied in association with the matrix coefficients as specified by nnpfc_matrix_coeffs. Its semantics are as specified for the VideoFullRangeFlag parameter in Rec. ITU-T H.273 | ISO/IEC 23091-2. When vui_full_range_flag is not present and nnpfc_out_format_idc is equal to 1, the value of nnpfc_full_range_flag is inferred to be equal to vui_full_range_flag0.

# Item 4: On the inference of nnpfc_chroma_sample_loc_type_frame

## Problem statement

In some cases, the location of chroma samples cannot be signalled. For example, when ColourizationFlag is equal to 1, nnpfc_out_colour_format_idc is equal to 1, meaning NNPF output pictures are in the 4:2:0 chroma format, and nnpfc_chroma_loc_info_present_flag is equal to 0, nnpfc_chroma_sample_loc_type_frame is not present. The inference of nnpfc_chroma_sample_loc_type_frame is missing in the current semantics, leading to undetermined chroma sample location.

## Proposed change

It is suggested adding the inference value for nnpfc_chroma_sample_loc_type_frame when it is not present. In addition, similar to vui_chroma_sample_loc_type_frame, a NOTE is added to clarify how to set the value of chroma sample location for BT.2020 and BT.2010 recommendations. The text is as follows:

nnpfc_chroma_sample_loc_type_frame, when not equal to 6 and nnpfc_out_colour_format_idc is equal to 1, specifies the location of chroma samples of the output pictures, as shown in Figure 1. nnpfc_chroma_sample_loc_type_frame equal to 6 and nnpfc_out_colour_format_idc equal to 1 indicates that the location of the chroma samples is unknown or unspecified or specified by other means not specified in this document. The value of nnpfc_chroma_sample_loc_type_frame shall be in the range of 0 to 6, inclusive. When nnpfc_chroma_sample_loc_type_frame is not present and nnpfc_out_colour_format_idc is equal to 1, the value of nnpfc_chroma_sample_loc_type_frame is inferred to be 6, which indicates that the location of the chroma samples is unknown or unspecified or specified by other means not specified in this document.

When nnpfc_out_colour_format_idc is equal to 1 (4:2:0 chroma format) and the output pictures are intended for interpretation according to Rec. ITU-R BT.2020 or Rec. ITU-R BT.2100, nnpfc_chroma_loc_info_present_flag should be equal to 1, and nnpfc_chroma_sample_loc_type_frame should be equal to 2.

# Item 5: On signalling of nnpfc_matrix_coeffs and nnpfc_full_range_flag

## Problem statement

In the current design, nnpfc_matrix_coeffs and nnpfc_full_range_flag are signalled only when nnpfc_out_format_idc is equal to 1, i.e. the NNPF output is of integer. However, nnpfc_matrix_coeffs and nnpfc_full_range_flag are properties of the colour space of output signals. Even the NNPF output is of real number, nnpfc_matrix_coeffs and nnpfc_full_range_flag should still be meaningful.

## Proposed change

It is suggested to enable sending nnpfc_matrix_coeffs and nnpfc_full_range_flag regardless of the NNPF output format. Below shows the proposed text changes:

8.28.2.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_matrix_coeffs | u(8) |
| nnpfc_full_range_flag | u(1) |
| } |  |
| } |  |
| ... |  |

# Item 6: On the aspect ratio of NNPF output pictures

## Problem statement

In an NNPF, when ResolutionResamplingFlag is equal to 1, the aspect ratio may be different from that of the cropped decoded pictures. However, the aspect ratio can be derived from that of the cropped decoded picture and scaling factors of resolution resampling.

## Proposed change

To clarify that the aspect ratio of the NNPF output pictures can be derived from exiting information, it is suggested to add the following NOTE:

NOTE: The aspect ratio of the NNPF output pictures can be derived from the aspect ratio of the cropped decoded pictures and the values of nnpfc_pic_width_num_minus1, nnpfc_pic_width_denom_minus1, nnpfc_pic_height_num_minus1, nnpfc_pic_height_denom_minus1.

# Item 7: On signalling of preferred display method for NNPF output

## Problem statement

In an NNPF, when ResolutionResamplingFlag is equal to 1, the aspect ratio may change, which may influence the preferred display method for the resulted video.

One example is video resolution adaptation, which make the video suitable for a specific resolution. Let's assume that the preferred display method for the cropped decoded video is overscan. After resolution adaption, the display area exactly matches the area of the pictures. So the preferred display method of the output video by the NNPF can be different from that of the decoded video.

The other example is AI image expander, shown in figure 1. An AI image expander can create new image area based on the original image. Because the new area is generated, the preferred display method for the new image can be set to overscan in case the monitor resolution is different.

Figure 1. Illustration of AI image expander. The blue area denotes the original image, an AI image expander may expand the image horizontally with new generated content.

In above examples, it is desired to indicate a preferred display method for the processed video.

## Proposed change

It is suggested to signal the preferred display information for the NNPF output picture. Below shows the text changes:

8.28.2.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( ResolutionResamplingFlag ) |  |
| nnpfc_scan_type_info_present_flag | u(1) |
| if( nnpfc_scan_type_info_present_flag ) |  |
| nnpfc_scan_type_idc | u(2) |
| ... |  |

8.28.2.2 Neural-network post-filter characteristics SEI message semantics

......

nnpfc_scan_type_info_present_flag equal to 1 specifies that the nnpfc_scan_type_idc is present. When nnpfc_scan_type_info_present_flag equal to 0 specifies that the nnpfc_scan_type_idc is not present. When not present, nnpfc_scan_type_info_present_flag is inferred to be equal to 0.

nnpfc_scan_type_idc equal to 0 indicates that the preferred display method for the pictures output by the NNPF is unknown or unspecified or specified by external means. nnpfc_scan_type_idc equal to 1 indicates that the pictures output by the NNPF are suitable for display using overscan. nnpfc_scan_type_idc equal to 2 indicates that the pictures output by the NNPF contain visually important information in the entire region out to the edges of the picture, such that the pictures output by the NNPF should not be displayed using overscan. Instead, they should be displayed using either an exact match between the display area and the edges, or using underscan. As used in this paragraph, the term "overscan" refers to display processes in which some parts near the borders of the pictures are not visible in the display area. The term "underscan" describes display processes in which the entire pictures are visible in the display area, but they do not cover the entire display area. For display processes that neither use overscan nor underscan, the display area exactly matches the area of the pictures. The value of nnpfc_scan_type_idc shall not be equal to 2. When not present, the value of nnpfc_scan_type_idc is inferred to be equal to 0.

.....

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).