| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0198_v2 |
| --- | --- |

| Title: | Non-EE2: Coding Information-based ALF Classification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Yang Wang1, Zhipin Deng1, Lei Zhao1, Na Zhang1, Mehdi Salehifar2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In this contribution, a coding information-based classification method is proposed for Luma Adaptive Loop Filter (ALF). In the proposed method, the prediction mode and the partitioning information are introduced into the Luma-ALF classification process to build up an additional rule.

On top of ECM-11.0, simulation results of the proposed method are reported as below:

AI:  0.01%, -0.01%, -0.01%, 100.1%%, 100.3%.

RA: -0.03%%, -0.01%%, -0.01%, 100.2%, 100.4%.

LB: -0.10%, -0.07%, 0.12%, 100.5%, 100.1%.

Introduction

In Luma-ALF classification design of ECM-11.0, there are totally 3 classifiers which are texture-based, band-based and residual-based classifiers. The encoder collects statistical data for each classifier and selects the most suitable classifier for each CTU. In current classification design, coding information such as the prediction mode and the partitioning information are not well considered and utilized.

Proposed Method

In this contribution, an additional rule for Luma-ALF classification is proposed. By using the prediction mode and partitioning information jointly, each classification unit is classified into 2 noise levels by the new added classification rule. For each classification unit (2x2), it will be classified into noise level 1 if it is located at a boundary position or coded by MODE_INTRA. Otherwise, the unit will be classified into noise level 0. The boundary position is defined as CU/TU boundaries.

Then the generated noise levels are further combined with the existing classifiers to output the final classification results. The class number of existing classifier is reduced to 12 from 25. For the band and residual classifier, the 0 to 1023 are quantized into 12 classes. For the texture classier, the 12 classes are generated by 3 directions multiply with 4 activities. This step is done by a LUT to map 25 classes into 12 classes. The 12 classes are further combined with the proposed 2 noise levels. Finaly, for each classifier, totally 12 x 2 = 24 classes are generated.

The proposed method is always applied and no signaling is required. In such a design, the number of classifiers is kept as 3 and no additional encoder selection is introduced.

Simulation Results

The proposed method was implemented on top of the ECM-11.0 [1]. Simulations were performed following the common test conditions [2].

More detailed results could be found in the attached excel sheets. The results compared to ECM-11.0 are shown in Table 1.

Table 1 Performance of the Proposed Method

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.02% | -0.01% | -0.01% | 101.3% | 99.6% | 100.3% | 100.9% |
| Class A2 | 0.02% | -0.01% | -0.01% | 98.7% | 100.7% | 100.3% | 100.6% |
| Class B | 0.01% | -0.01% | -0.01% | 99.9% | 100.1% | 99.9% | 101.1% |
| Class C | 0.00% | 0.00% | 0.00% | 101.0% | 100.0% | 100.3% | 100.9% |
| Class E | 0.00% | -0.01% | -0.01% | 99.9% | 101.1% | 100.2% | 100.6% |
| Overall | 0.01% | -0.01% | -0.01% | 100.1% | 100.3% | 100.2% | 100.9% |
| Class D | 0.00% | 0.00% | 0.00% | 99.5% | 100.1% | 100.6% | 100.5% |
| Class F | 0.03% | -0.02% | -0.02% | 100.4% | 101.8% | 100.4% | 101.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.05% | 0.04% | 99.9% | 99.6% | 100.2% | 100.6% |
| Class A2 | 0.00% | 0.03% | -0.08% | 99.9% | 100.8% | 100.5% | 100.1% |
| Class B | -0.01% | 0.02% | -0.01% | 100.3% | 101.1% | 100.9% | 101.0% |
| Class C | -0.10% | -0.03% | 0.02% | 100.5% | 99.9% | 100.0% | 101.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.01% | -0.01% | 100.2% | 100.4% | 100.4% | 100.8% |
| Class D | -0.01% | 0.04% | -0.05% | 101.1% | 100.9% | 100.1% | 100.5% |
| Class F | -0.04% | -0.04% | -0.09% | 100.6% | 100.2% | 100.2% | 101.8% |
|  | Low Delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | 0.33% | 0.65% | 100.2% | 100.7% | 101.0% | 100.8% |
| Class C | -0.12% | -0.10% | -0.20% | 101.0% | 100.6% | 100.2% | 100.7% |
| Class E | -0.15% | -0.70% | -0.34% | 100.2% | 98.6% | 100.8% | 100.5% |
| Overall | -0.10% | -0.07% | 0.12% | 100.5% | 100.1% | 100.7% | 100.7% |
| Class D | -0.05% | -0.18% | -0.41% | 101.8% | 100.6% | 99.6% | 100.4% |
| Class F |  |  |  |  |  |  |  |

Conclusions

In this contribution, an additional classification rule with coding information is proposed for Luma-ALF. The proposed method can achieve promising coding performance with negligible encoding and decoding time increase. It is recommended to include the proposed method into the next round of EE2.

Reference

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AF2017, DE, October 2023.

Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).