| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0225 |
| --- | --- |

| Title: | Crosscheck of JVET-AG0091 (EE2-1.8: Auto-relocated block vector prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | yo-kidani@kddi.com |
| Source: | KDDI Corporation (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-AG0091, which reports testing results of the Test-1.8 method (auto-relocated block vector prediction) in the context of Exploration Experiments 2 (JVET-AF2024). The cross-checker confirms that the software provided by the proponents matches the algorithm description of JVET-AG0091. All the testing results in this cross-check report match those provided by the proponent.

Summary

JVET-AG0091 [1] proposes an auto-relocated block vector prediction (AR-BVP) and reports its testing results over ECM-11.0 [2]. The AR-BVP are derived as new BV candidates for IBC merge/AMVP candidate list construction. Specifically, the AR-BVP can be derived by recursively traced BVs based on the pre-derived BV candidates by other BV derivation methods such as spatial BVP, temporal BVP, and HMVP. AR-BVP candidates are inserted after HMVP derivation. JVET-AG0091 investigates the three tests of AR-BVP’s BV tracing paths: Test 1.8a is 1; Test 1.8b is 2; and Test 1.8c is no constraint.

The cross-checker carefully analyzed the software provided by the proponents [3] where the AR-BVP is implemented on top of ECM-11.0 [2]. Then, we confirmed that the software implementation perfectly matches the algorithm description of JVET-AG0091.

The cross-checker conducted simulations using the provided software according to the EE2 description [4] and the common test condition for enhanced compression tool testing [5]. The cross-check results are provided in the accompanying Excel sheets. The cross-checker could exactly reproduce the results provided by the proponent and reported in JVET-AG0091. The cross-checker compiled the provided software on a 64-bit Ubuntu environment. The experiments were performed on the cluster PCs.

Conclusion

The cross-checker has verified the description and results reported in JVET-AG0091. It is confirmed that the software provided by the proponent matches the proposal description, and the testing results produced by the cross-checker match the reported results.

From a cross-checker’s point of view, JVET-AG0091 proposes a straightforward yet effective extension to the IBC AMVP/merge list construction. The proposed method provides reasonable coding gain without increasing encoding and decoding runtime (EncT/DecT) for both camera-captured and screen content. Comparing the results of Tests 1.8a, 1.8b, and 1.8c results, the trade-off between coding gain and EncT/DecT is almost the same. On the other hand, 1.8a is the most straightforward design among them.

Accordingly, the Test 1.8a’s AR-BVP method is encouraged to be adopted in the next ECM version.

Reference

- N. Zhang, K. Zhang, L. Zhang, “EE2-1.8: Auto-relocated block vector prediction,” JVET-AG0091, January 2024.

- EE2-1.8, https://vcgit.hhi.fraunhofer.de/ecm/jvet-af-ee2/ECM/-/tree/TEST_1.8

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-11.0

- V. Seregin, et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AF2024, October 2023.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October 2023.