| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0226-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AG0130 (EE1-4.1: Unified CNN-based super resolution for resampling-based video coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jiedong Ye, Xu Li, Yiqing Zhu, Qiong Liu | Email: | ye_jd@hust.edu.cn |
| Source: | Huazhong University of Science and Technology |  |  |

_____________________________

- Abstract

This contribution reports cross-check results of EE1-4.1. It proposes a unified SR architecture, which is designed to be aligned to the unified in-loop filter architecture and complexity.

The cross-checked rate-distortion results and encoding/decoding runtimes of JVET-AG0130 are reported. In AI and RA,configurations, the cross-checked rate-distortion results match exactly those presented in JVET-AG0130.

Introduction

JVET-AF0143[1] proposes a unified SR architecture, which is designed to be aligned to the unified in-loop filter architecture and complexity. They proposed two unified SR models, high complexity HOP and low complexity LOP.

Our cross-check consists of two main parts: 1. Training and inference. Train HOP and LOP then quantize the model from float to int, and perform inference with int models. 2. Inference only. Directly use their trained and quantized HOP and LOP models to perform inference with int models.

A total of 4 models are included. Trained by us: HOP(int), LOP(int); Trained by them: HOP(int), LOP(int).

Training status

The dataset used is consistent with JVET-AF0143 and consists of 200 BVI-DVC sequences at 1080p, 20 BVI-DVC sequences at 4K, 20 TVD sequences at 4K, and 800 DIV2K images (rotated and flipped for enhancement). The QP used is: 22, 27, 32, 37 for DIV2K (without 42); 22, 27, 32, 37, 42 for BVI-DVC and TVD.

A total of 44 epochs were trained, using the model from the last epoch for quantization and inference.

All results

Only the SR tool is activated during the inference process, the rest of the NN tools are disabled.

HOP trained by us

Table 1 Results of the HOP for RA configurations

|  | Random Access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -x.xx% | -x.xx% | -x.xx% |
| Class A2 | -x.xx% | -x.xx% | -x.xx% |
| avg. A1&A2 | -x.xx% | -x.xx% | -x.xx% |

Table 2 Results of the HOP for AI configurations

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -x.xx% | -x.xx% | -x.xx% |
| Class A2 | -x.xx% | -x.xx% | -x.xx% |
| avg. A1&A2 | -x.xx% | -x.xx% | -x.xx% |

LOP trained by us

Table 3 Results of the LOP for RA configurations

|  | Random Access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -5.03% | -6.15% | -3.77% |
| Class A2 | -2.23% | 3.89% | 5.22% |
| avg. A1&A2 | -3.63% | -1.13% | 0.73% |

Table 4 Results of the LOP for AI configurations

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -3.25% | -5.72% | -4.33% |
| Class A2 | -2.71% | -6.17% | 4.00% |
| avg. A1&A2 | -2.98% | -5.95% | -0.17% |

HOP trained by them

Table 5 Results of the HOP for RA configurations

|  | Random Access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -x.xx% | -x.xx% | -x.xx% |
| Class A2 | -x.xx% | -x.xx% | -x.xx% |
| avg. A1&A2 | -x.xx% | -x.xx% | -x.xx% |

Table 6 Results of the HOP for AI configurations

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -x.xx% | -x.xx% | -x.xx% |
| Class A2 | -x.xx% | -x.xx% | -x.xx% |
| avg. A1&A2 | -x.xx% | -x.xx% | -x.xx% |

LOP trained by them

Table 7 Results of the LOP for RA configurations

|  | Random Access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -x.xx% | -x.xx% | -x.xx% |
| Class A2 | -x.xx% | -x.xx% | -x.xx% |
| avg. A1&A2 | -x.xx% | -x.xx% | -x.xx% |

Table 8 Results of the LOP for AI configurations

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -x.xx% | -x.xx% | -x.xx% |
| Class A2 | -x.xx% | -x.xx% | -x.xx% |
| avg. A1&A2 | -x.xx% | -x.xx% | -x.xx% |

Reference

- C. Lin, et al. AHG11: Unified CNN super resolution for resampling-based video coding. JVET-AF0143, 32nd Meeting, Hannover DE. Oct. 13-20 2023.