| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0232-v2 |
| --- | --- |

| Title: | AHG9: Support of non-parallel MPI layers in the MPII SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li1, Ye-Kui Wang2, Yiling Xu1, Kaifa Yang1 1Shanghai Jiaotong University, China 2Bytedance Inc., US | Tel: Email: | yueli983@sjtu.edu.cn yekui.wang@bytedance.com yl.xu@sjtu.edu.cn sekiroyyy@sjtu.edu.cn |
| Source: | Shanghai Jiaotong University, Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an extension to the multiplane image information (MPII) SEI message in the VSEI TuC (JVET-AF2032), such that the multiplane image layers can be either parallel or non-parallel to each other.

In v2 of this document, some changes were made for optimizing the number of bits used for signalling of the normal vectors of multiplane image (MPI) layers for the case when the MPI layers might be non-parallel. And some text errors were fixed.

# Introduction

JVET-AE0066 [1] and JVET-AF0167 [2] propose a method to signal multiplane image information (MPII) in an SEI message, and it was included in the technologies under consideration (TuC) for future extension of VSEI (JVET-AF2032) [3]. However, the MPII SEI message in JVET-AF2032 does not support multiplane images with non-parallel layers.

## Summary of MPII SEI message

As noted in JVET-AF2032, the MPII SEI message conveys information that can be used by a decoder to reconstruct an MPI picture from a decoded output picture containing packed MPI texture and opacity layers. The MPII SEI message supports different packing arrangements of texture and opacity layers including side-by-side, top-and-bottom, and temporal interleaving. The MPII SEI message can also convey depth information of each MPI layer.

## MPI with non-parallel layers

Multiplane image (MPI) is an effective and efficient representation for view synthesis, which consists of a set of parallel RGBA layers, each containing color and transparency information. However, standard MPI is sensitive to discretization due to slanted surfaces in the scene. As show in Figure 1, When all layers are parallel to the source image plane, slanted surfaces may be split across MPI layers, leading to discretization artifacts in novel views.

| - original view | - novel view |
| --- | --- |

Figure 1 Discretization artifacts in standard MPI

To solve the above problem, [4] proposed the Structural Multiplane Image (S-MPI) representation, which consists of a set of posed RGBα images with geometries approximating the 3D scene as shown in Figure 2.

Figure 2 Standard MPI v.s. Structural MPI [4]

The standard MPI can be represented by a set of parallel RGBA layers with corresponding depth information as shown in Figure 3(a). Each layer can be formulated as (Ci, Ai, di) where Ci and Ai denote RGB and alpha transparency maps of the i-th layer and di denotes depth of i-th layer. Differently, besides RGBA layers and corresponding layer depth, the Structural MPI comprises the normal vectors of MPI layers as shown in Figure 3(b). Each layer in Structural MPI can be formulated as (Ci, Ai, ni, di) where Ci and Ai denote RGB and alpha transparency maps of the i-th layer, di denotes depth/distance between the i-th layer and camera position, and ni denotes the normal vector of the i-th layer.

| - Standard MPI representation [1] | - MPI with non-parallel layers [4] |
| --- | --- |

Figure 3 The structures of Standard MPI and MPI with non-parallel layers

With non-parallel MPI layers, improved performance in view reconstruction and synthesis were reported in [4].

# Proposal

This contribution proposes an extension to the MPII SEI message, for which the latest text is in JVET-AF2032, to support the structure of MPI with non-parallel layers. The text changes are provided below with modification highlighted in yellow.

| multiplane_image_information( payloadSize ) { | Descriptor |
| --- | --- |
| mpii_num_layers_minus1 | ue(v) |
| mpii_layers_parallel_flag | u(1) |
| mpii_layer_depth_equal_distance_flag | u(1) |
| if( mpii_layer_depth_equal_distance_flag ) { |  |
| depth_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| depth_rep_info_element( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| } else |  |
| for( i = 0; i <= mpii_num_layers_minus1; i++ ) |  |
| depth_rep_info_element( ZSign[ i ], ZExp[ i ], ZMantissa[ i ], ZManLen[ i ] ) |  |
| if( !mpii_layers_parallel_flag ) |  |
| normal_vector_len_idc | u(2) |
| for( i = 0; i <= mpii_num_layers_minus1; i++ ) { |  |
| fronto_parallel_flag[ i ] | u(1) |
| if( !fronto_parallel_flag[ i ] ) { |  |
| normal_vector_x[ i ] | i(v) |
| normal_vector_y[ i ] | i(v) |
| } |  |
| } |  |
| mpii_texture_opacity_interleave_flag | u(1) |
| if( mpii_texture_opacity_interleave_flag = = 0 ) |  |
| mpii_texture_opacity_arrangement_flag /* 0: Top-and-Bottom, 1: Side-by-Side */ | u(1) |
| mpii_picture_num_layers_in_height_minus1 | ue(v) |
| } |  |

The multiplane image information (MPII) SEI message specifies the multiplane image (MPI) scene representation information that may be used for view synthesis.

When an MPII SEI message is present in any AU of a CLVS, an MPII SEI message shall be present in the first AU of the CLVS and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an MPII SEI message is output that follows the current picture in output order.

NOTE 1 – This SEI message can work together with the multiview acquisition information SEI message for view synthesis. The multiview acquisition information SEI message specifies the intrinsic and extrinsic parameters for current camera view. When multiple views are available, the reconstructed novel views can be rendered from multiplane images of nearby views.

Use of this SEI message requires the definition of the following variables:

–	Cropped decoded output picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	A cropped decoded picture array decPicCurr0[ cIdx ][ x ][ y ], with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..( cIdx = = 0 ) ? CroppedWidth : CroppedWidth / SubWidthC − 1, y = 0..( cIdx = = 0 ) ? CroppedHeight : CroppedHeight / SubHeightC − 1.

–	In output order a temporally following cropped decoded picture array decPicCurr1[ cIdx ][ x ][ y ], with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..( cIdx = = 0 ) ? CroppedWidth : CroppedWidth / SubWidthC − 1, y = 0..( cIdx = = 0 ) ? CroppedHeight : CroppedHeight / SubHeightC − 1.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

mpii_num_layers_minus1 plus 1 specifies the number of texture and opacity layers for the MPI representation.

mpii_layers_parallel_flag equal to 1 indicates that all layers are parallel. mpii_layers_parallel_flag equal to 0 indicates that all layers may or may not be parallel.

mpii_layer_depth_equal_distance_flag equal to 1 indicates that equal distance is used to generate MPI layers and depth parameter for each layer Z[ i ] can be derive using nearest depth value ZNear and farthest depth value ZFar.

The depth value for i-th MPI layer, Z[ i ], is derived as follows:

Z[ i ] = i * ( ZFar − Znear ) ÷ (mpi_num_layers_minus1 ) + ZNear	(xx)

mpii_layer_depth_equal_distance_flag equal to 0 indicates that the depth information for each layer follows next in the SEI message.

The variables in the x column of Table xx are derived from the respective variables in the s, e, n and v columns of Table xx as follows:

–	If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1)s * 2( e − 31 ) * ( 1 + n ÷ 2v ).

–	Otherwise (e is equal to 0), x is set equal to ( −1)s * 2−( 30 + v ) * n.

NOTE 2 – The above specification is similar to that found in IEC 60559:1989.

Table xx – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| Z[ i ] | ZSign[ i ] | ZExp | ZMantissa | ZManLen |

NOTE 3 – In some applications, disparity is used instead of depth (the disparity value D and depth value Z relationship is D = 1 ÷ Z). Corresponding to Equation (xx), the disparity value for the i-th MPI layer is D[ i ] = i * ( DFar − Dnear ) ÷ (mpi_num_layers_minus1 ) + DNear.

normal_vector_len_idc specifies the number of bits, denoted by the variable lenInbits, used for coding of the normal_vector_x[ i ] and normal_vector_y[ i ] syntax elements. The value of lenInbits is derived to be equal to 2normal_vector_len_idc + 2.

fronto_parallel_flag[ i ] equal to 1 indicates that the i-th MPI layer is parallel to the image plane and the normal vector of the i-th MPI layer is (0.0, 0.0, 1.0). fronto_parallel_flag[ i ] equal to 0 indicates that the i-th MPI layer is not parallel to the image plane and the normal vector of the i-th layer can be derived from normal_vector_x[ i ] and normal_vector_y[ i ].

normal_vector_x[ i ] and normal_vector_y[ i ] are used to specify the x and y components, respectively, of the normal vector of the i-th MPI layer. normal_vector_x[ i ] and normal_vector_y[ i ] are coded using lenInbits bits. The values of normal_vector_x[ i ] and normal_vector_y[ i ] shall be in the range of −2lenInbits − 2 to 2lenInbits − 2, inclusive.

When mpii_layers_parallel_flag is equal to 0, the three components of the normal vector of the i-th MPI layer, normalVectorX[ i ], normalVectorY[ i ], and normalVectorZ[ i ], are derived as follows:

if( fronto_parallel_flag[ i ] ) {
	normalVectorX[ i ] = 0.0
	normalVectorY[ i ] = 0.0
	normalVectorZ[ i ] = 1.0
} else {
	normalVectorX[ i ] = normal_vector_x[ i ] ÷ 2lenInbits − 2

normalVectorY[ i ] = normal_vector_y[ i ] ÷ 2lenInbits − 2

normalVectorZ[ i ] = Sqrt( 1 − normalVectorX[ i ]2 − normalVectorY[ i ]2 )
}

mpii_texture_opacity_interleave_flag equal to 1 indicates decoded output pictures correspond to temporally interleaved texture and opacity constituent pictures in output order as illustrated in Figure XX. mpii_texture_opacity_interleave_flag equal to 0 indicates decoded output pictures correspond to spatially packed texture and opacity constituent pictures as illustrated in Figures XX and XX.

mpii_texture_opacity_arrangement_flag equal to 0 indicates decoded output pictures represent texture and opacity constituent pictures in a top-bottom packing arrangement as illustrated in Figure XX. mpii_texture_opacity_arrangement_flag equal to 1 indicates decoded output pictures represent texture and opacity constituent pictures in a side-by-side packing arrangement as illustrated in Figure XX.

For each specified picture packing arrangement scheme, there are two constituent pictures that are referred to as picture 0 and picture 1. When mpii_texture_opacity_interleave_flag is equal to 0, the constituent picture associated with the upper-left sample of the decoded picture is considered to be constituent picture 0 and the other constituent picture is considered to be constituent picture 1. When mpii_texture_opacity_interleave_flag is equal to 1, the first decoded picture in the current CLVS is constituent picture 0 and the next decoded picture in output order is constituent picture 1 and the display time of the constituent picture 0 should be delayed to coincide with the display time of constituent picture 1. The two constituent pictures form the spatially packed texture and opacity picture of a MPI, with picture 0 being associated with the spatially packed texture picture and picture 1 being associated with the spatially packed opacity picture.

mpii_picture_num_layers_in_height_minus1 plus 1 specifies the number of spatially packed layers in height for picture 0 and picture 1. The variable hLayers is set equal to mpii_picture_num_layers_in_height_minus1 + 1 and the variable wLayers is set equal to (mpii_num_layers_minus1 + 1) / hLayers.

Let variable fWidth and fHeight specify the width and height of picture 0 and picture 1, respectively, and are derived as follows:

–	If mpii_texture_opacity_interleave_flag is equal to 1, the following applies:

fWidth = CroppedWidth

fHeight = CroppedHeight

–	Otherwise (mpii_texture_opacity_interleave_flag is equal to 0)

–	If mpii_texture_opacity_arrangement_flag is equal to 0, the following applies:

fWidth = CroppedWidth , fHeight = CroppedHeight / 2

–	Otherwise (mpii_texture_opacity_arrangement_flag is equal to 1), the following applies:

fWidth = CroppedWidth / 2 , fHeight = CroppedHeight

Let variable cWidth = fWidth / subWidthC and variable cHeight = fHeight / subHeightC.

Let array picture0[ cIdx ][ x ][ y ] specify samples in picture 0 and array picture1[ cIdx ][ x ][ y ] specify samples in picture 1, with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..( cIdx = = 0 ) ? fWidth: cWidth − 1, y = 0..( cIdx = = 0 ) ? fHeight : cHeight − 1 and are derived as follows:

–	If mpii_texture_opacity_interleave_flag is equal to 1, the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr1[ cIdx ][ x ][ y ]

–	Otherwise (mpii_texture_opacity_interleave_flag is equal to 0)

–	Let variable cW = ( cIdx = = 0 )? fWidth : cWidth

–	Let variable cH = ( cIdx = = 0 )? fHeight : cHeight

–	If mpii_texture_opacity_arrangement_flag is equal to 0, the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y + cH ]

–	Otherwise (mpii_texture_opacity_arrangement_flag is equal to 1), the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x + cW ][ y ]

Let variable layerWidth and layerHeight specify the width and height for decoded MPI layer, respectively. The variables are derived as follows:

layerWidth = fWidth / wLayers

layerHeight = fHeight / hLayers

The reconstruction of MPI process is described as follows:

The outputs of this process are:

–	a 4D MPI texture layer array recTextureLayer[ i ][ cIdx ][ w ][ h ] with i = 0..mpii_num_layers_minus1, cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, w = 0..( cIdx = = 0 ) ? layerWidth : layerWidth / SubWidthC − 1, and h = 0..( cIdx = = 0 ) ? layerHeight : layerHeight / SubHeightC − 1.

–	a 3D MPI opacity layer array recOpacityLayer[ i ][ w ][ h ] with i = 0..mpii_num_layers_minus1, x = 0..layerWidth − 1, and y = 0..layerHeight − 1.

The array recTextureLayer and array recOpacityLayer are derived as follows:

for( i = 0; i  <=  mpii_num_layers_minus1; i++ ) {
	k = i % wLayers
	m = ( i − k ) / hLayers
	for( cIdx = 0; cIdx < ChromaFormatIdc = = 0 ) ? 1 : 3; cIdx++ )
		for( h = 0; h < ( cIdx = = 0 ) ? layerHeight : layerHeight / SubHeightC ; h++ )
			for( w = 0; w < ( cIdx = = 0 ) ? layerWidth : layerWidth / SubWidthC ; w++ ) {
				u = k * ( cIdx  = =  0 ) ? layerWidth : layerWidth / SubWidthC + w
				v = m * ( cIdx  = =  0 ) ? layerHeight : layerHeight / SubHeightC + h
				recTextureLayer[ i ][ cIdx ][ w ][ h ] = picture0[ cIdx ][ u ][ v ]
			}
	for( h = 0; h < layerHeight; h++ )
		for( w = 0; w < layerWidth; w++ )
			recOpacityLayer[ i ][ w ][ h ] = picture1[ 0 ][ k * layerWidth + w ][ m * layerHeight + h ]
}

# References

- Taoran Lu, Peng Yin, Guan-Ming Su, Dae Yeol Lee, Tsung-wei Huang, Sean McCarthy, Walt Husak, and Gary J. Sullivan, “AHG9: Multiplane Image Information SEI”, JVET-AE0066, July 2023

- Taoran Lu, Peng Yin, Sejin Oh, Sean McCarthy, Walt Husak, and Gary J. Sullivan, “AHG9: On the Proposed Multiplane Image Information SEI Message”, JVET-AF0167, October 2023

- M. M. Hannuksela, J. Chen, S. Deshpande, S. McCarthy, “Technologies under consideration for future extensions of VSEI (draft 2)”, JVET-AF2032, Sept. 2023

- Mingfang Zhang, Jinglu Wang, Xiao Li, Yifei Huang, Yoichi Sato, and Yan Lu, “Structural Multiplane Image: Bridging Neural View Synthesis and 3D Reconstruction”, CVPR 2023

# Patent rights declaration(s)

Shanghai Jiaotong University and Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).