| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0237-v3 |
| --- | --- |

| Title: | EE2-3.3 related: Fix on LFNST/NSPT index signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Jie Zhao Seung-Hwan Kim 10680 Treena St. St# 550 San Diego, CA 92131, USA | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com jaehyun.lim@lge.com jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In EE2-3.3 (JVET-AG0061: Utilizing LFNST/NSPT for inter coding), it is expected that LFNST/NSPT index signalling for inter coding is aligned with intra case and performed only when last non-zero coefficient of Luma component is not at DC position. However, in current EE2-3.3 SW, an LFNST/NSPT index can be signalled even when last coefficient of Luma component is at DC position. In this contribution, a fix for this problem is proposed.

The BD-rate and complexity changes relative to ECM-11.0:

- RA*:  -0.30%/0.04%/0.11% for Y/U/V with 102% EncT and 100% DecT

- LD:  -0.86%/0.06%/0.01% for Y/U/V with 104% EncT and 101% DecT.

The BD-rate and complexity changes relative to EE2-3.3:

- RA*:  0.01%/-0.19%/0.02% for Y/U/V with 100% EncT and 100% DecT

- LD:  0.01%/-0.46%/-0.35% for Y/U/V with 100% EncT and 100% DecT.

The above two LD results were obtained by enabling LFNST/NSPT for both test and anchor.

* QP-27 results of ParkRunning copied from their anchor

Problem Statement

LFNST/NSPT for inter coding is applied to the Luma component only when the Luma component satisfies DC condition (i.e., last non-zero coefficient is not at DC position). However, in the current EE2-3.3 SW, DC condition checking process considers both Luma and Choma. Further, it is hard coded that the DC condition is always true for Chroma components. As a result, an LFNST/NSPT index, which is zero, is signalled unnecessarily even when last non-zero coefficient of Luma component lies only in DC position. The problematic code part is shown in Figure 1.

void CABACWriter::residual_coding( …… ) { ...... if (cuCtx && tu.mtsIdx[compID] != MTS_SKIP && tu.blocks[compID].height >= 4 && tu.blocks[compID].width >= 4) { const int lfnstLastScanPosTh = CU::isIntra(*(tu.cu)) ? (isLuma(compID) ? LFNST_LAST_SIG_LUMA : LFNST_LAST_SIG_CHROMA) : (isLuma(compID) ? LFNST_LAST_SIG_LUMA_INTER : LFNST_LAST_SIG_CHROMA_INTER); // LFNST_LAST_SIG_LUMA_INTER = 1, LFNST_LAST_SIG_CHROMA_INTER = 0 cuCtx->lfnstLastScanPos ¦= cctx.scanPosLast() >= lfnstLastScanPosTh; } ...... }

Figure 1 Problematic code part related to LFNST/NSPT index signalling in EE2-3.3 SW

Proposed fix

A simple and straightforward fix for the problem pointed out in Section 1 is proposed in the following code (Figure 2):

if (cuCtx && tu.mtsIdx[compID] != MTS_SKIP && tu.blocks[compID].height >= 4 && tu.blocks[compID].width >= 4) { const int lfnstLastScanPosTh = isLuma( compID ) ? ( CU::isIntra(*(tu.cu)) ? LFNST_LAST_SIG_LUMA : LFNST_LAST_SIG_LUMA_INTER ) : LFNST_LAST_SIG_CHROMA; cuCtx->lfnstLastScanPos ¦= ( CU::isIntra( *( tu.cu ) ) ¦¦ isLuma( compID ) ) ? ( cctx.scanPosLast() >= lfnstLastScanPosTh ) : false; }

Figure 2 The proposed fix for the problem found in Figure 1

In Figure 2, you can find that “cuCtx->lfnstLastCanPos” is not updated by Chroma components when LFNST/NSPT is utilized in inter coding, because false value is directly used instead of checking DC condition as in Luma component. Due to this fix, the following modification in InterSearch.cpp (Figure 3) should be accompanied, because it is inappropriate to set unallowable cost value (i.e. MAX_DOUBLE) to Chroma components of which DC condition is always false.

..... if (compID == COMPONENT_Y && tu.mtsIdx[compID] > MTS_SKIP && !cuCtx.mtsLastScanPos) { currCompCost = MAX_DOUBLE; } //else if (cu.lfnstIdx && !cuCtx.lfnstLastScanPos) else if( cu.lfnstIdx && ( isLuma( compID ) && !cuCtx.lfnstLastScanPos ) ) { currCompCost = MAX_DOUBLE; } else { ......

Figure 3 The encoder fix aligned with the proposed fix shown in Figure 2

Experimental results

The proposed fix has been implemented based on ECM-11.0 reference software according to common test conditions defined in JVET-AF2017 [2]. The simulation results for LD configuration were obtained by enabling LFNST/NSPT for both test and anchor.

* QP-27 results of ParkRunning copied from their anchor

The changes of BD-rate and complexity over EE2 Test-3.3 are reported below:

* QP-27 results of ParkRunning copied from their anchor

Conclusion

In this contribution, the problem in EE2-3.3 SW which unnecessary LFNST/NSPT index signalling can occur in inter coding was pointed out, and a simple and straightforward fix for that was proposed. The unnecessary signalling is due to the fact that Chroma components can change value of the variable indicating whether last non-zero coefficient in Luma component is located at DC position or not. It is recommended that the proposed fix should be adopted in next ECM.

References

[1] F. Wang, J. Gan, Y. Yu, H. Yu, and D. Wang, “EE2-3.3: Utilizing LFNST/NSPT for inter coding”, JVET-AG0061, Jan. 2024.

[2] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).