| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0313-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AG0111 (EE1-2.2: Content-adaptive LOP filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Zhenyu Dai | Email: | daizhenyu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution describes the crosscheck of JVET-AG0111[1] : “EE1-2.2: Content-adaptive LOP filter”. Training crosscheck has been performed for class A1, classA2 and class D. The LOP1 NN loop filter models were trained (overfitted) and tested for each random-access segment of the CTC contents using the EE1-2.2 software based on the training and inference procedures shared by the proponents.

Using JVET-AG0111 as the anchor, the crosschecker trained EE1-2.2 results for RA class D are reported to have a minor deviation of 0.01%(Y) -0.05%(U) -0.06%(V). The training crosscheck of class A1 and class A2 are still running at this moment.

Introduction

JVET-AG0111 contribution proposes content-adaptive approach for neural network loop-filter by means of overfitting one of the models of the LOP1 filter in NNVC-7.1. The content adaptation models are overfitted for each random-access segment. The weight-update was computed and compressed, generating NNR/NNC bitstream using the NCTM software and subsequent patch provided for NCTM software. The compressed NNR stream is signalled through NN APS params for each RA segment.

Results

The inference was done by enabling LOP1 filter and NN Intra pred filter. The inference results for the models overfitted for each RA segment trained by the crosschecker are reported in Table 1 and Table 2 with NNVC-6.0 and JVET-AG0111 i.e., proponents results as anchor.

Table 1 EE1-2.2 RA cross check results over NNVC-6.0 (LOP1+NN IntraPred)

|  | Random access Main 10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C |  |  |  |
| Class E |  |  |  |
| Overall |  |  |  |
| Class D | 0.08% | 0.10% | 0.70% |
| Class F |  |  |  |

Table 2 EE1-2.2 RA cross check results over JVET-AG0111

|  | Random access Main 10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over JVET-AG0111 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C |  |  |  |
| Class E |  |  |  |
| Overall |  |  |  |
| Class D | 0.01% | -0.05% | -0.06% |
| Class F |  |  |  |

Conclusion

Based on the current results shared above, it is the opinion of the crosschecker that the proponents pass the training crosscheck.

Reference

- R. Yang, M. Santamaria, F. Cricri, M. M. Hannuksela, H. Zhang, J. Lainema, A. Hallapuro, D. Bugdayci Sansli (Nokia). EE1-2.2: Content-adaptive LOP filter. JVET-AG0111. January 2024.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. Common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AF2016, 32nd Meeting, Hannover, 13–20 October 2023.