| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0318-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AG0123 (EE2-1.10: Bilateral Filtering for Intra Prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Vladislav Shchukin | Email: | vladislav.shchukin@ericsson.com |  |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

The contribution reports the results of crosscheck of JVET-AG0123 proposed by Bytedance Inc. In JVET-AF0123 authors proposed bilateral filtering for intra prediction samples before they are used for prediction. Suggested bilateral filter is similar to the one used on loop-filtering stage in ECM-11.0.

# Verification

Source code with changes on the top of ECM-11.0 is taken from the corresponding git branch for EE2-1.10 (authored by Bytedance).

The BD-rate results for finished classes match the results reported in JVET-AG0123. The simulation results are presented below.

In the following tables, all reported runtimes (both EncT and DecT) are reliable. To estimate EncT/DecT in similar conditions, the anchor and the test are simulated on the same machine in parallel, and the total number of launched simulations on each machine is at most the number of physical cores on this machine. To avoid irrelevant costs during DecT estimation, the following settings of decoder are switched off: checking of MD5 sums, writing of YUV-output. EncT is averaged over 2 launches, DecT is averaged over 10 launches.

Simulation environment: Xeon Gold 6254 + 40GB RAM, Xeon Gold 6354 + 50GB RAM or similar; GCC 12.2.0; OS: Red Hat Enterprise Linux Server 7.9; AVX2.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.10% | -0.05% | -0.05% | 103.4% | 102.6% | 100.0% | 99.7% |
| Class A2 | -0.14% | 0.05% | 0.03% | 103.2% | 101.6% | 99.6% | 100.1% |
| Class B | -0.08% | -0.03% | -0.02% | 103.4% | 102.5% | 98.9% | 99.9% |
| Class C | -0.18% | -0.16% | -0.14% | 103.2% | 102.7% | 100.0% | 100.0% |
| Class E | -0.11% | 0.07% | 0.10% | 103.7% | 102.9% | 100.0% | 100.0% |
| Overall | -0.12% | -0.03% | -0.02% | 103.4% | 102.5% | 99.6% | 99.9% |
| Class D | -0.18% | -0.05% | -0.14% | 102.7% | 103.4% | 100.0% | 100.0% |
| Class F | -0.15% | -0.48% | -0.20% | 101.8% | 102.5% | 100.0% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.01% | 0.24% | 0.02% | 100.6% | 99.9% | 100.0% | 99.9% |
| Class C | -0.02% | -0.10% | 0.17% | 100.5% | 99.9% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.06% | -0.01% | 0.04% | 100.4% | 100.1% | 100.1% | 100.0% |
| Class F | 0.02% | -0.02% | -0.01% | 100.3% | 100.5% | 99.8% | 101.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.07% | 0.08% | -0.58% | 100.2% | 99.5% | 100.6% | 100.4% |
| Class E | -0.20% | 1.56% | -0.18% | 100.0% | 100.4% | 101.3% | 100.1% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.15% | 0.22% | -1.29% | 100.2% | 100.0% | 100.4% | 100.0% |
| Class F |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.