| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0332-v2 |
| --- | --- |

| Title: | AHG9: Test material for the multi-plane image information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Walt Husak, Sean McCarthy, Taoran Lu | Email: | {wjh,sean.mccarthy}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution provides information for test material for testing of the multi-plane image information SEI message described in the TuC for future extensions of VSEI.

# Description of the test content

MPI raw data test material, “GuitarMan”, is provided to facilitate the testing of the VTM software to support the multiplane image information (MPII) SEI message. The test material represents MPI video intended to be displayed as described in Table 1.

Table 1 “GuitarMan” MPI test material

| frame width | 384 |
| --- | --- |
| frame height | 768 |
| number of frames | 30 |
| frame rate (fps) | 30 |
| number of MPI layers per frame | 16 |
| MPI texture video format | YUV 4:2:0 10-bit standard range |
| MPI opacity video format | YUV 4:0:0 10-bit standard range |

The test material is provided in a zipped file containing the following:

1) MPI texture YUV: GuitarMan_texture_384x768_10b_420_std.yuv

The MPI texture YUV file contains 480 pictures in YUV 4:2:0 10-bit standard range format where each picture represents a single MPI texture layer per frame of displayed video (16 MPI texture layers per frame x 30 frames = 480 pictures). For example, frames 0~15 represent MPI texture layers 0 ~15 in the frame 0 of the displayed video. Frames 16~31 represent MPI texture layers 0~15 in the frame 1 of the displayed video.

2) MPI opacity YUV: GuitarMan_opacity_384x768_10b_P400_std.yuv

The MPI opacity YUV file contains 480 pictures in YUV 4:0:0 10-bit standard range format where each picture represents a single MPI opacity layer per frame of displayed video (16 MPI opacity layers per frame x 30 frames = 480 pictures). For example, frames 0~15 represent MPI opacity layers 0 ~15 in the frame 0 of the displayed video. Frames 16~31 represent MPI opacity layers 0~15 in the frame 1 of the displayed video.

3) MPI layer depth array: GuitarMan_depth_info.txt

4) Example cfg file (multiplane_image_info_SEI_GuitarMan_interleaved_example.cfg) to enable the MPII SEI message using VTM. The example employs 2 rows x 8 columns layer arrangement and temporal interleaved packing for texture and opacity.

Preprocessing is needed to convert the MPI raw data into packed video for VTM encoding. Example scripts and instructions are provided with the software implementation under \vvc-software-vtm-tuc\cfg\sei_vui\multiplane_image_info\

# License

A license agreement is attached to this input document.

Any questions or comments can be referred to:

Walt Husak

+1 818 823 2800

wjh@dolby.com

Please send your license agreement with [GuitarMan_MPI] in the subject line to wjh@dolby.com.

# Patent rights declaration(s)

Dolby Laboratories, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).