| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0002-v1 |
| --- | --- |

| Title: | JVET AHG report: Draft text and test model algorithm description editing (AHG2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JVET |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | B. Bross, C. Rosewarne, F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye |  | benjamin.bross@hhi.fraunhofer.de chris.rosewarne@canon.com.au frank@bossentech.com adrian.browne@sony.com seunghwan3.kim@lge.com shanl@global.tencent.com ohm@ient.rwth-aachen.de g-j-sullivan@outlook.com alexismt@apple.com yekui.wang@bytedance.com yan.ye@alibaba-inc.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports on the work of the JVET ad hoc group on Draft text and test model algorithm description editing (AHG2) between the 33rd meeting Online, (17 – 27 January 2024) and the 34th meeting in Rennes, FR, (17 – 24 April 2024).

# Mandates

At the 33rd meeting of the ITU-T/ISO/IEC Joint Video Experts Team (JVET), an ad hoc group on Draft text and test model algorithm description editing (AHG2) was established with the following mandates:

- Produce and finalize draft text outputs of the meeting (JVET-AG2027 and JVET-AG2034).

- Collect reports of errata for the VVC, VSEI, HEVC, AVC, CICP, and the published related technical reports and produce the JVET-AG1004 errata output collection.

- Coordinate with the test model software development AhG to address issues relating to mismatches between software and text.

- Collect and consider errata reports on the texts.

# Ad hoc group activity

## Output documents produced

### JVET-AH1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

Incorporated items at the JVET-AG meeting:

- For VVC:

  - In subclause 7.4.2.4.5, the first bullet item, replace "subpicture level values" with "subpicture index values". (reported by Rickard Sjöberg – thanks!)

- For VSEI:

  - Fix a typo when vui_chroma_sample_loc_type_frame is mentioned in the VSEI text. (JVET-AG0192)

  - Fix two bugs for the constraints on nnpfc_matrix_coeffs and add two NOTEs in the semantics of nnpfc_matrix_coeffs to clarify complicated cases. (JVET-AG0192)

  - Fix an asserted bug on the inference of nnpfc_full_range_flag. (JVET-AG0192)

### JVET-AH2027 SEI processing order and processing order nesting SEI messages in VVC (draft 7)

This document contains the draft text for changes to the Versatile Video Coding (VVC) standard (Rec. ITU-T H.266 | ISO/IEC 23090-3), to specify the SEI processing order (SPO) and processing order nesting (PON) SEI messages.

Changes that have been integrated:

- Item 2 option 1 from JVET-AG0052v4.

- Delete “However, if po_sei_wrapping_flag[ i ] is equal to 0 and no SEI message is present …” (and NOTE 2). (Documented under JVET-AG0053.) (NOTE 2 seemed unrelated to the deletion, providing a clarification of po_sei_wrapping_flag[ i ] and hence was kept.)

- Agreed: An SEI message being present within a PON should be equivalent, from a decoder perspective, to being present outside of a PON. (Documented under JVET-AG0053.)

- Delete “The persistence of all the SEI messages included in the same processing order nesting SEI message shall be the same.” (Documented under JVET-AG0053.)

- Add a clarification of the persistence issue (activation/deactivation/persistence). (Documented under JVET-AG0053.)

- Add a clarifying note in the semantics to describe the flexibility supported when multiple PON SEI messages are sent. (Documented under JVET-AG0165.)

- Add (editors to develop text): If NNPFC is in a PON, the corresponding NNPFA and any NNPFC updating for that NNPF shall be in a PON (not necessarily the same PON). (Documented under JVET-AG0169.)

- Add (editors to develop text): For all SEI messages (not just NNPFC), there should be a constraint that there should not be “activation” outside of a PON and then later a deactivation or some update inside of a PON. (Documented under JVET-AG0169.)

- Use of the film grain characteristics SEI message as a part of a processing order indicated by an SEI processing order SEI message (JVET-AG0180).

### JVET-AH2034 SEI messages for VSEI version 4 (Draft 1)

This document contains the draft text for changes to the versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7), to specify additional SEI messages, including encoder optimization information, source picture timing information , object mask information, and modality information SEI messages and updates to the neural-network post-filter characteristics SEI message.

Changes yet to be integrated: (none)

Changes that have been integrated:

Changes incorporate: JVET-AG0070, JVET-AG0081, JVET-AG-0082v2, JVET-AG0086, JVET-AG0089, JVET-AG0148, JVET-AG0188, JVET-AG0322

- NNPFC SEI message: Temporal extrapolation purpose (JVET-AG0089).

- Source picture timing information SEI message: Include with the following modifications

  - various aspects from JVET-AG0070

    - (Proposal 1)_Editorial clarification of NOTE 1

    - (Proposal 2) Forbid the value 0 for spti_num_units_in_elemental_interval (option 2)

    - (Proposal 3)Modify semantic to fix bug in handling the case spti_source_type_present_flag equal 0

  - option 1 of JVET-AG0188

  - Modification from JVET-AG0082v2

- Encoder optimization information SEI message:

  - 2-bit indicators eoi_for_human_viewing_idc and eoi_for_machine_analysis_idc (JVET-AG0086, item 1)

  - new optimization type from JVET-AG0081

- Object mask information SEI message: Include with modifications from JVET-AG0148

- Modality information SEI message:

- NNPFC SEI message application information signaling

- payloadType values for the added SEI message (VVC text).

# Related input contributions

The following input contributions were noted as relevant to the work of this ad hoc group:

- JVET-AH0046 On MV-HEVC profiles

- JVET-AH0169 Video CICP problem reports and proposed actions

# Remaining VVC bug tickets

Carried over:

- #1594 Mismatch between VVC spec and VTM for sample generation in CCLM process

- #1607 Wrong sign (+ instead of -) in Fig 16 Flowchart for decoding a decision in the ITU text

- #1609 NoBackwardPredFlag derivation ambiguity

- #1617 Not initialized NumCtusInSlice[0] to 0.

- #1618 [Multilayer Profiles] Potential Mismatch of VTM22.0 & Specification Related To Derivation Process For Merge Motion Vector Difference

New:

- #1624 Incorrect indexing in computation of motion vector offset

- #1628 Derivation of ModeTypeCondition should say "one or more"

- #1629 mtt_split_cu_vertical_flag context uses undefined variable chType

- #1630 Missing equations for applying AmvrShift

- #1631 Should "Motion vector storing process for geometric partitioning mode" store HpelIfIdx?

- #1632 Incorrect indexing used for choosing matrix intra sample prediction

- #1634 Matrices QStateTransTable,levelScale,AlfFixFiltCoeff,AlfClassToFiltMap are incorrectly transposed

- #1635 Incorrect inference for tu_y_coded_flag

# Recommendations

The AHG recommends to:

- Approve JVET-AH1004, JVET-AH2027, and JVET-AH2034 documents as JVET outputs,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification text and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC text specification and Test Model documents,

- Ensure that, when considering changes to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC specification text is made available in a timely manner,

- Review bug tickets, and other AHG2 related inputs and act on them if found to be necessary.