| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0012-v1 |
| --- | --- |

| Title: | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | M. Karczewicz Y. Ye L. Zhang B. Bross X. Li R. Chernyak K. Naser Y. Yu | Tel: Email: | martak@qti.qualcomm.com yan.ye@alibaba-inc.com lizhang.idm@bytedance.com benjamin.bross@hhi.fraunhofer.de xlxiangli@google.com chernyak@global.tencent.com karam.naser@interdigital.com yue.yu@oppo.com |
| Source: | AHG 12 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on enhanced compression beyond VVC capability (AHG12) between the 33rd meeting, by teleconference, 17–26 January 2024, and the 34th meeting, Rennes, FR, 17–24 April 2024.

# Introduction

The mandates given to the AHG are:

- Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC.

- Discuss and propose refinements to the ECM12 algorithm description JVET-AG2025.

- Coordinate with AHG7 to study the performance and complexity tradeoff of these video coding tools.

- Coordinate with AHG6 on ECM software development.

- Support AHG6 in generating anchors according to the test conditions in JVET-AF2017.

- Analyse the results of exploration experiments described in JVET-AG2024 in coordination with the EE coordinators.

- Coordinate with AHG11 to study the interaction with neural network-based coding tools.

- Define testing conditions and prepare bitstreams for subjective evaluation of ECM by expert viewing at the next meeting in coordination with AG 5, AHG4, and AHG6.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). A few emails were exchanged regarding EE2.

# Activities

The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-AG2024). The combined improvements of the ECM-12.0 over VTM-11.0ecm12.0 anchor for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -11.49% | -23.82% | -31.29% | 974.2% | 461.4% |
| Class A2 | -17.87% | -30.14% | -33.45% | 971.5% | 477.6% |
| Class B | -12.19% | -28.03% | -26.89% | 886.8% | 469.7% |
| Class C | -12.19% | -18.16% | -19.27% | 904.0% | 433.4% |
| Class E | -16.00% | -26.76% | -24.38% | 1115.2% | 773.6% |
| Overall | -13.66% | -25.28% | -26.60% | 954.3% | 501.3% |
| Class D | -10.16% | -15.74% | -16.30% | 902.9% | 469.7% |
| Class F | -27.76% | -37.20% | -37.31% | 640.3% | 509.9% |
| Class TGM | -40.85% | -48.69% | -47.92% | 529.9% | 537.3% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -23.81% | -29.45% | -38.33% | 1043.8% | 886.0% |
| Class A2 | -27.31% | -37.48% | -41.49% | 963.3% | 1047.8% |
| Class B | -22.31% | -36.57% | -34.80% | 847.1% | 890.3% |
| Class C | -23.79% | -28.57% | -29.15% | 863.0% | 904.4% |
| Class E |  |  |  |  |  |
| Overall | -24.01% | -33.20% | -35.34% | 910.7% | 922.8% |
| Class D | -24.62% | -29.48% | -30.46% | 849.7% | 1013.5% |
| Class F | -30.29% | -39.15% | -39.87% | 691.2% | 554.5% |
| Class TGM | -38.31% | -45.25% | -45.36% | 667.0% | 497.8% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -18.68% | -41.71% | -39.50% | 884.5% | 750.4% |
| Class C | -20.33% | -32.55% | -33.40% | 775.5% | 848.1% |
| Class E | -17.84% | -33.16% | -31.46% | 724.5% | 526.3% |
| Overall | -19.02% | -36.52% | -35.46% | 805.3% | 715.3% |
| Class D | -22.12% | -35.14% | -34.68% | 749.7% | 881.7% |
| Class F | -26.41% | -40.45% | -40.19% | 657.3% | 577.2% |
| Class TGM | -36.79% | -46.01% | -46.24% | 632.4% | 505.1% |

The rate reduction for natural sequences over VTM in RA configuration for {Y, U, V} increased from ECM-11.0’s {-22.56%, -31.91%, -33.67%} to ECM-12.0’s {-24.01%, -33.20%, -35.34%}. For SCC sequences (class TGM) the rate reduction for RA configuration increased from ECM-11.0’s { -36.82%, -43.33%, -43.43%} to ECM-12.0’s { -38.31%, -45.25%, -45.36%}.

# Contributions

In addition to 35 EE2 contributions, 51 (comparing to 43 last meeting) EE2-related and AHG12-related contributions were received. The EE2-related and AHG12-related contributions can be subdivided as follows:

## Intra (9)

JVET-AH0058, “Non-EE2: derived MIP modes with fusion”, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AH0072, “AHG12: Eliminating division operations in DIMD and TIMD”,	 Z. Fan, T. Chujoh, T. Ikai (Sharp)

JVET-AH0082, “Non-EE2: GPM with CCCM for intra chroma prediction”, Y. Huo, Y. Liu (Transsion)

JVET-AH0083, “AHG12: Additional TIMD mode with different cost metric”, D. Bugdayci Sansli, S. Blasi, J. Lainema (Nokia)

JVET-AH0095, “AHG12: Regularized EIP”, Hongdong Qin, Jacek Konieczny, Keqin Ding, Zhuowei Xu (TCL)

JVET-AH0105, “AHG12: Multiple Reference Area Decoder-side Intra Mode Derive”, J. Fan, J. Huo, Z. Zhang, Y. Ma, F. Yang (Xidian Univ.), M. Li (OPPO)

JVET-AH0210, “Non-EE2: Modifications to Matrix-Based Intra Prediction”, H. Wang, B. Ray, V. Seregin, M. Karczewicz, P. Garus, S. Eadie (Qualcomm)

JVET-AH0211, “Non-EE2: Intra merge mode”,	Y.-J. Chang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AH0274, "EE2 related: TIMD Merge Mode with Enhanced Block Vector Deployment", K. Naser, Y. Chen, F. Le Léannec, M. Radosavljević, K. Reuzé (InterDigital)

## IntraTMP and IntraBC (6)

JVET-AH0059, “AHG12: Temporal BV for IBC merge list construction”, L. Xu, Y. Yu, H. Yu, D. Wang (OPPO), N. Zhang, K. Zhang, L. Zhang (Bytedance)

JVET-AH0071, “EE2-related: IntraTMP extension to TIMD”, J. Fu, Z. Li, J. Zhang, C. Jia, S. Ma (PKU), Y. Gao, C. Huang (ZTE)

JVET-AH0131, “AhG12: Fix on BV Filtering for IntraTMP Fusion”, S. Kim, D. Jun (Dong-A University)

JVET-AH0151, “EE2-related: IntraTMP merge candidates clustering based on refinement window”, D. Ruiz Coll, J.-K. Lee, (Ofinno)

JVET-AH0156, “AHG12: Neural Network-based Intra Prediction”, F. Urban, T. Dumas, F. Galpin (Interdigital)

JVET-AH0203, “Non-EE2: On BVG-CCCM”, K. Zhang, Z. Deng, L. Zhang (Bytedance)

## Inter (14)

JVET-AH0091, “Non-EE2: Geometry partitioning mode with inter prediction and intra block copy”, Y. Wang, K. Zhang, L. Zhang (Bytedance)

JVET-AH0116, “EE2-3.1/2 related: Additional chained motion vector prediction candidates”, N. Zhang, K. Zhang, L. Zhang (Bytedance)

JVET-AH0141, “AHG12: Sharp motion compensation filter for bi-prediction”, J. Samuelsson-Allendes (Sharp)

JVET-AH0144, “AHG12: Hierarchical Subblock based DMVR”, K. Andersson, C. Hollmann (Ericsson)

JVET-AH0145, “AHG12: Follow up on normative DMVR fix for ECM as in JVET-AG0125 (Parallel friendly use of boundary distortion for DMVR)”, K. Andersson (Ericsson)

JVET-AH0168, “Template Matching Picture Boundary Padding”, Nicolas Neumann, Mathias Wien

JVET-AH0178, “Non-EE2: High Accuracy Sample Based Bi-Directional Optical Flow”,	M. Salehifar, Y. He, K. Zhang, L. Zhang (Bytedance)

JVET-AH0178, “Non-EE2: High Accuracy Sample Based Bi-Directional Optical Flow”,	M. Salehifar, Y. He, K. Zhang, L. Zhang (Bytedance)

JVET-AH0193, “EE2-related: Chroma LIC derivation with template costs”, TM. Bae S. Deshpande

JVET-AH0202, “EE2 related: Adaptive weighting for Inter CCP merge mode with zero luma CBF”, X. Li (Google), Z. Deng, K. Zhang, L. Zhang (Bytedance)

JVET-AH0208, “Non-EE2: affine candidates derived from temporal collocated picture”, C. Ma, X. Xiu, C.-W. Kuo, W. Chen, X. Wang (Kwai)

JVET-AH0213, “Non-EE2: MVP extension”, Z. Zhang, J.-L Lin, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AH0215, “Non-EE2: Adaptive Cost Function Selection in Merge Mode”, K. Cui, Z. Zhang, Y. Zhang, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AH0222, “Non-EE2: GPM-Affine with MMVD and TM”, Y. Wang, K. Zhang, L. Zhang (Bytedance)

## In Loop Filters (6)

JVET-AH0060, “Non-EE2: On residual adjustments for adaptive loop filters”, Z. Dai, N. Song, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AH0061, “Non-EE2: CCALF with Chroma SAO input”, Z. Dai, N. Song, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AH0062, “Non-EE2: CCALF with Chroma Fixed Filter Input”, N. Song, Z. Dai, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AH0163, “AHG12: ALF syntax modification”, F. Galpin, G. Boisson, P. Bordes, F. Lefebvre, I. Marzuki (InterDigital)

JVET-AH0170, “Non-EE2: ALF residuals scaling”, P. Bordes, F. Galpin, G. Boisson (InterDigital)

JVET-AH0275, "Non-EE2: CCALF with chroma ALF outputs",	N. Hu, M. Karczewicz, V. Seregin (Qualcomm)

## Transform (8)

JVET-AH0103, “Non-EE2: Low-delay configurations for LFNST/NSPT”, M. Koo, Y. Ahn, J. Nam, J. Zhao, J. Lim, S. Kim (LGE)

JVET-AH0181, “AHG12: Adaptation of intra/inter LFNST/NSPT for low-delay configuration”, C. Bonnineau, F. Le Leannec, K. Naser, S. Puri, I. Marzuki, R. Utida (InterDigital)

JVET-AH0182, “Non-EE2: On intra and inter LFNST/NSPT for low delay configuration”, Y. Kidani, H. Kato, K. Kawamura (KDDI)

JVET-AH0184, “EE2-4.1 related: On Multiple Transform Set Selection for LFNST/NSPT”, C. Bonnineau, S. Puri, K. Naser, F. Le Leannec (InterDigital)

JVET-AH0218, “EE2-4.1 related: Alternative intra mode derivation for LFNST/NSPT”, L. Zhao, K. Zhang, L. Zhang (Bytedance)

JVET-AH0219, “Non-EE2: Improvements on Inter LFNST/NSPT”, L. Zhao, K. Zhang, L. Zhang (Bytedance)

JVET-AH0237, “EE2-4.1 related: Alternative intra mode derivation for LFNST/NSPT kernel selection”, M. Coban, M. Karczewicz, V. Seregin

JVET-AH0307, "AHG12: Multiple Transform Set Selection for Intra MTS", C. Bonnineau, K. Naser, S. Puri, F. Le Léannec

## Coefficients and Entropy Coding (3)

JVET-AH0089, “AhG12: On entropy models training”, F. Galpin, F. Lo Bianco, C. Salmon-Legagneur (InterDigital)

JVET-AH0153, “EE2-6.3 related: Combination of EE2-6.1 and EE2-6.2 on entropy coding extension – retraining”,	F. Galpin, F. Lo Bianco, C. Salmon-Legagneur, M. Balcilar (InterDigital), R.-L. Liao, Yan Ye, Jie Chen, Xinwei Li (Alibaba), P. Nikitin, M. Karczewicz (Qualcomm)

JVET-AH0223, “AHG12: Context retraining for ECM-12.0”, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)

## Partitioning (2)

JVET-AH0152, “Ahg12: adaptive dual-tree coding in B slices”,	F. Le Leannec, K. Naser, T. Dumas, T. Poirier, F. Urban, E. Francois (InterDigital)

JVET-AH0155, “AHG12: Restricting BT CUs to apply QT-like partitioning structure”, W. Ahmad, P. Wennersten, K. Andersson (Ericsson)

## Quantization (2)

JVET-AH0063, “AHG12: On Shifting Quantization Center”, Y. Yu, H. Yu, L. Xu, J. Gan, F. Wang, Z. Xie, D. Wang (OPPO)

JVET-AH0117, “AHG12: Modification of chroma QP mapping table for ECM”, Y. Wang, K. Zhang, L. Zhang (Bytedance), J. Chen, R.-L. Liao, Y. Ye (Alibaba), V. Seregin, M. Karczewicz (Qualcomm)

## Other (1)

JVET- AH0180, " AHG12: Modified CTC for encoding time reduction under low-delay configuration ", R. Utida, S. Puri, I. Marzuki, C. Bonnineau (InterDigital)

# Recommendations

The AHG recommends to:

- To review all the related contributions.