| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0014-v3 |
| --- | --- |

| Title: | JVET AHG report: NNVC software development AhG14 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | F. Galpin, Y. Li, Y. Li, J. N. Shingala L. Wang, Z. Xie | Tel: Email: | franck.galpin@interdigital.com yue.li@bytedance.com yli30@qti.qualcomm.com jay.shingala@ittiam.com liqiangwang@tencent.com xiezhihuang@oppo.com |
| Source: | AHG14 on NNVC software development |  |  |

# Abstract

This report summarizes the activities of the AHG14 on NNVC software development that have taken place between the 33rd and 34th JVET meetings.

# Introduction

The mandates for the AhG are as follows:

- Coordinate development of the NNVC software and associated configuration files.

- Prepare and deliver NNVC-8.0 software version (and potential updates), and provide reference configuration encodings according to the NNVC common test conditions as described in JVET-AF2016.

- Investigate combinations of tools included in the NNVC software, prepare and release anchor data for all configurations of the software, including anchors for High and Low Operation Point (HOP/LOP) configurations.

- Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed.

- Coordinate with NNVC algorithm and software description (JVET-AG2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate.

- Coordinate with AHG11 on items related to NNVC activities.

# Software development

## Location

NNVC repository is located at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM

NNVC software is based on VTM-11.0 with enabled MCTF including the update from JVET-V0056, GOP32, and enabling deblocking in the RDO.

NNVC-8.0 anchor at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/blob/master/Anchor%20performance/NNVC-current_Reporting_Template-with-anchor-data.xlsm?ref_type=heads is used for NNVC performance evaluation.

## Software changes

## NNVC-8.0

Several commits were merged in the NNVC repository.

The following changes were integrated:

- JVET-AG0109 JVET-AG0219 sadl v8

- JVET-AF0172: scaling cleanup

- JVET-AG0174: HOP3

- JVET- AG0130: unified SR filter

The following fixes and clean-up were perfomed:

- MR185: simd build option clean-up

- MR187: HOP2 float version

- MR186: fix opset version of ONNX to 10

## NNVC-8.1

- MR197: support for Y4M

- MR198: unified model names

- MR200: fix build for ARM

- Other fixes (remaining debug code, json files correction etc.)

- MRxxx: fix memory padding issue

## Software version

## NNVC-8.1 was tagged xxx

## NNVC-8.0 was tagged February 20th, 2024

NNVC-8.0rc3 was tagged February 20th, 2024.

NNVC-7.1 was tagged November 29th, 2023.

NNVC-7.0 was tagged November 3rd, 2023.

NNVC-6.1 was tagged September 25th, 2023 (fix).

NNVC-6.0 was tagged September 6th, 2023.

NNVC-5.1 was tagged July 19th, 2023.

NNVC-5.0 was tagged May 11th, 2023.

NNVC-3.0 (a.k.a VTM-11.0_nnvc3.0) was tagged December 1st, 2022.

NCS-1.0 (a.k.a NNVC-3.0wip2) was tagged September 4th, 2022 (first release containing the FilterSets, using NNVC 2.0 as a base).

VTM-11.0_nnvc-2.0 was tagged August 4th, 2022 (add deblocking in RDO).

VTM-11.0_nnvc-1.0 was tagged May 6th, 2021 (VTM-11.0 base with MCTF enabled).

# CTC performance

See configurations section for naming convention.

## Comparison to VTM

### NNVC-7.1 VTM vs NNVC-8.0 VTM

NNVC-8.0 in VTM mode performance are the same as the ones in NNVC-5.0/NNVC-6.0/NNVC-7.1.

### NNVC-8.0 VTM vs NNVC-8.0 anchor

The NNVC-8.0 anchor includes LOP.2 filter and Intra Prediction tools activated.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.91% | -10.13% | -10.58% | -8.57% | -11.31% | -10.56% | 122% | 7629% |
| Class A2 | -6.80% | -12.67% | -9.74% | -6.92% | -11.64% | -7.52% | 114% | 7025% |
| Class B | -6.58% | -13.91% | -12.69% | -6.81% | -13.14% | -12.07% | 121% | 7467% |
| Class C | -6.59% | -14.87% | -14.44% | -7.10% | -13.23% | -13.00% | 121% | 7162% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -6.89% | -13.16% | -12.14% | -7.26% | -12.50% | -11.11% | 120% | 7326% |
| Class D | -7.16% | -14.79% | -14.97% | -6.54% | -13.04% | -13.02% | 117% | 7268% |
| Class F | -3.74% | -9.48% | -9.43% | -4.46% | -10.49% | -10.21% | 129% | 3275% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -5.31% | -13.68% | -13.36% | -6.23% | -15.19% | -17.25% | 113% | 7591% |
| Class C | -5.53% | -13.89% | -15.13% | -7.37% | -15.85% | -17.20% | 110% | 7143% |
| Class E | -6.17% | -8.64% | -11.92% | -7.97% | -10.13% | -13.32% | 119% | 7827% |
| Overall | -5.60% | -12.49% | -13.59% | -7.04% | -14.15% | -16.25% | 113% | 7496% |
| Class D | -6.49% | -13.14% | -15.10% | -7.37% | -14.18% | -18.02% | 110% | 7026% |
| Class F | -3.09% | -8.64% | -9.08% | -4.53% | -10.17% | -13.90% | 121% | 3871% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.62% | -12.69% | -12.29% | -9.00% | -14.91% | -12.55% | 168% | 5248% |
| Class A2 | -7.15% | -12.70% | -11.26% | -7.43% | -12.12% | -9.31% | 160% | 4409% |
| Class B | -7.35% | -13.02% | -13.55% | -7.46% | -13.33% | -13.40% | 167% | 4381% |
| Class C | -7.53% | -13.81% | -14.47% | -7.95% | -14.33% | -14.57% | 165% | 3524% |
| Class E | -10.71% | -14.15% | -15.12% | -10.89% | -13.52% | -14.18% | 168% | 5078% |
| Overall | -8.13% | -13.28% | -13.42% | -8.39% | -13.65% | -12.97% | 166% | 4414% |
| Class D | -7.65% | -12.72% | -15.05% | -7.45% | -13.57% | -15.41% | 157% | 3460% |
| Class F | -5.27% | -10.79% | -10.39% | -5.15% | -11.24% | -11.54% | 133% | 3832% |

Note: Results from Interdigital, crosschecked by Oppo.

### NNVC-8.0 VTM mode vs NNVC-8.0 HOP.3

The NNVC-8.0 where LOP.2 filter is replaced by HOP.3 and Intra Prediction tools activated.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -15.25% | -10.18% | -18.82% | -16.35% | -14.10% | -20.86% | 268% | 110495% |
| Class A2 | -14.77% | -13.58% | -9.89% | -14.32% | -12.33% | -6.47% | 243% | 109442% |
| Class B | -12.48% | -13.53% | -12.81% | -11.77% | -12.85% | -12.47% | 260% | 115443% |
| Class C | -13.41% | -17.35% | -16.77% | -12.53% | -13.32% | -13.55% | 219% | 100974% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -13.74% | -13.89% | -14.49% | -13.40% | -13.12% | -13.23% | 247% | 109249% |
| Class D | -14.56% | -16.75% | -17.13% | -11.42% | -12.52% | -12.35% | 214% | 96985% |
| Class F | -8.59% | -12.88% | -10.90% | -9.08% | -12.97% | -11.81% | 351% | 56200% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -10.97% | -8.91% | -5.85% | -11.11% | -10.68% | -10.37% | 251% | 113144% |
| Class C | -12.22% | -14.10% | -12.34% | -12.29% | -12.03% | -11.74% | 208% | 107881% |
| Class E | -12.68% | -10.72% | -7.64% | -13.14% | -9.85% | -13.53% | 432% | 144336% |
| Overall | -11.82% | -11.09% | -8.46% | -12.01% | -10.93% | -11.62% | 270% | 118351% |
| Class D | -13.68% | -15.24% | -10.44% | -12.43% | -11.39% | -10.84% | 193% | 93836% |
| Class F | -8.87% | -11.99% | -9.35% | -10.14% | -11.83% | -16.40% | 349% | 65056% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -12.87% | -9.06% | -16.12% | -14.41% | -13.68% | -18.00% | 315% | 75281% |
| Class A2 | -12.52% | -10.73% | -8.40% | -13.38% | -10.32% | -6.11% | 233% | 59516% |
| Class B | -11.16% | -10.20% | -10.84% | -11.13% | -11.27% | -11.73% | 236% | 56660% |
| Class C | -12.29% | -13.10% | -15.34% | -11.95% | -12.65% | -14.52% | 195% | 37894% |
| Class E | -15.82% | -15.48% | -15.22% | -15.99% | -14.06% | -15.46% | 254% | 63142% |
| Overall | -12.70% | -11.62% | -13.05% | -13.05% | -12.29% | -13.08% | 240% | 55773% |
| Class D | -11.83% | -11.97% | -15.59% | -10.93% | -11.96% | -13.53% | 189% | 38033% |
| Class F | -8.87% | -11.73% | -10.21% | -8.55% | -12.04% | -11.72% | 161% | 47748% |

Note: Results from InterDigital, crosschecked by Oppo.

### NNVC-8.0 VTM mode vs NNVC-8.0 VTM mode + LOP.2

The results reflect LOP only performance.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -5.92% | -8.83% | -9.08% | -6.68% | -10.46% | -9.36% | 114% | 7917% |
| Class A2 | -5.58% | -12.14% | -8.92% | -5.75% | -11.47% | -6.97% | 110% | 7549% |
| Class B | -4.99% | -13.20% | -12.06% | -5.26% | -12.71% | -12.06% | 111% | 7525% |
| Class C | -5.12% | -13.75% | -13.25% | -5.62% | -12.69% | -12.47% | 107% | 7272% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -5.33% | -12.26% | -11.15% | -5.74% | -12.01% | -10.61% | 110% | 7538% |
| Class D | -6.02% | -14.05% | -13.96% | -5.34% | -12.54% | -12.29% | 105% | 6861% |
| Class F | -2.77% | -8.94% | -8.89% | -3.37% | -10.21% | -10.25% | 115% | 3210% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -4.89% | -13.49% | -12.77% | -5.82% | -15.07% | -16.97% | 114% | 7987% |
| Class C | -5.26% | -13.41% | -14.60% | -7.03% | -15.43% | -17.27% | 105% | 7276% |
| Class E | -6.05% | -8.54% | -12.83% | -7.82% | -10.33% | -13.69% | 118% | 8011% |
| Overall | -5.30% | -12.23% | -13.40% | -6.72% | -14.00% | -16.25% | 112% | 7749% |
| Class D | -6.36% | -13.10% | -14.24% | -7.39% | -14.87% | -17.08% | 105% | 6949% |
| Class F | -2.94% | -8.15% | -8.23% | -4.54% | -9.55% | -13.42% | 121% | 3936% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -4.64% | -9.01% | -8.52% | -4.78% | -11.21% | -8.98% | 115% | 5097% |
| Class A2 | -4.48% | -10.12% | -8.62% | -4.75% | -9.96% | -6.96% | 106% | 4156% |
| Class B | -4.39% | -10.32% | -10.64% | -4.56% | -10.93% | -11.08% | 107% | 4077% |
| Class C | -4.64% | -11.11% | -11.70% | -4.93% | -12.02% | -12.27% | 102% | 2808% |
| Class E | -6.42% | -9.58% | -11.35% | -6.53% | -9.32% | -11.20% | 105% | 4571% |
| Overall | -4.84% | -10.12% | -10.31% | -5.04% | -10.79% | -10.33% | 107% | 3983% |
| Class D | -5.00% | -10.10% | -12.34% | -4.77% | -11.51% | -13.09% | 100% | 2707% |
| Class F | -3.21% | -9.07% | -8.61% | -3.02% | -9.83% | -9.96% | 101% | 3387% |

Note: Results from InterDigital, crosschecked by XX.

### NNVC-8.0 VTM mode vs NNVC-8.0 VTM mode + HOP.3

The results reflect HOP only performance.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -13.55% | -9.39% | -17.87% | -14.76% | -13.62% | -20.30% | 257% | 116262% |
| Class A2 | -13.68% | -12.97% | -9.03% | -13.22% | -12.02% | -6.00% | 238% | 115050% |
| Class B | -11.10% | -12.81% | -11.89% | -10.49% | -12.51% | -12.31% | 248% | 119588% |
| Class C | -12.15% | -16.49% | -15.36% | -11.25% | -12.81% | -12.54% | 201% | 104150% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -12.39% | -13.14% | -13.44% | -12.09% | -12.71% | -12.71% | 234% | 113729% |
| Class D | -13.43% | -16.07% | -16.36% | -10.26% | -12.25% | -12.39% | 200% | 100674% |
| Class F | -7.74% | -11.87% | -10.25% | -7.98% | -11.89% | -11.60% | 325% | 54035% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -10.60% | -8.57% | -5.76% | -10.80% | -10.94% | -10.38% | 247% | 119531% |
| Class C | -12.01% | -14.17% | -12.04% | -12.14% | -12.38% | -12.00% | 211% | 109479% |
| Class E | -12.51% | -11.12% | -6.05% | -12.85% | -10.42% | -13.01% | 436% | 150792% |
| Overall | -11.55% | -11.07% | -7.93% | -11.76% | -11.29% | -11.58% | 271% | 123024% |
| Class D | -13.32% | -14.87% | -9.83% | -11.84% | -11.32% | -9.83% | 205% | 101113% |
| Class F | -8.79% | -11.21% | -9.45% | -10.22% | -10.76% | -15.92% | 345% | 66146% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.39% | -5.38% | -12.55% | -10.81% | -10.02% | -14.67% | 245% | 72718% |
| Class A2 | -10.14% | -8.22% | -6.33% | -11.12% | -8.10% | -4.24% | 174% | 60217% |
| Class B | -8.53% | -6.91% | -7.58% | -8.58% | -8.64% | -9.15% | 169% | 55114% |
| Class C | -9.75% | -9.89% | -12.10% | -9.35% | -9.93% | -11.40% | 133% | 37919% |
| Class E | -12.07% | -11.04% | -11.12% | -12.41% | -10.06% | -12.30% | 179% | 65344% |
| Overall | -9.80% | -8.22% | -9.79% | -10.18% | -9.30% | -10.28% | 173% | 55459% |
| Class D | -9.42% | -9.42% | -12.58% | -8.43% | -10.42% | -11.51% | 126% | 36624% |
| Class F | -7.03% | -9.86% | -8.30% | -6.67% | -10.37% | -9.78% | 129% | 48234% |

Note: Results from InterDigital, crosschecked by xxxx.

### NNVC-8.0 VTM vs NNVC-8.0 NNSR

The results show the performance of NNVC with all tools on (LOP) vs VTM.

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 8 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM | V-MSIM |  | EncT |  | DecT CPU |  |
| Class A1 | -11.13% | -9.37% |  | -8.88% |  |  |  |  |  |  |  |  |  |  |
| Class A2 | -7.92% |  | -6.25% |  |  | -2.78% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -6.58% | -13.91% |  | -12.69% |  |  |  |  |  |  |  |  |  |  |
| Class C | -6.59% | -14.87% |  | -14.44% |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -7.76% | -11.73% |  | -10.41% |  |  |  |  |  |  |  |  |  |  |
| Class D | -7.16% | -14.79% |  | -14.97% |  |  |  |  |  |  |  |  |  |  |
| Class F | -3.74% | -9.48% |  | -9.43% |  |  |  |  |  |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 8 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM | V-MSIM |  | EncT |  | DecT CPU |  |
| Class A1 | -9.60% | -8.49% |  | -7.53% |  |  |  |  |  |  |  |  |  |  |
| Class A2 | -9.07% | -7.80% |  | -3.33% |  |  |  |  |  |  |  |  |  |  |
| Class B | -7.35% | -13.02% |  | -13.55% |  |  |  |  |  |  |  |  |  |  |
| Class C | -7.53% | -13.81% |  | -14.47% |  |  |  |  |  |  |  |  |  |  |
| Class E | -10.71% | -14.15% |  | -15.12% |  |  |  |  |  |  |  |  |  |  |
| Overall | -8.61% | -11.76% |  | -11.31% |  |  |  |  |  |  |  |  |  |  |
| Class D | -7.65% | -12.72% |  | -15.05% |  |  |  |  |  |  |  |  |  |  |
| Class F | -5.27% | -10.79% |  | -10.39% |  |  |  |  |  |  |  |  |  |  |

Note: Results from Oppo crosscheck by InterDigital,

## Comparison to previous NNVC anchor

### NNVC-7.1 anchor vs NNVC-8.0 anchor

The results reflect LOP/nnvc progress between NNVC 7.1 (as anchor) and NNVC 8.0 (as test).

The scaling clean-up affects only RA/LDB configuration. Update to SADL v8 speed-up both encoding and decoding.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.00% | -0.05% | 0.03% | 0.00% | 0.01% | 0.04% | 75% | 65% |
| Class A2 | -0.01% | 0.01% | 0.00% | -0.01% | 0.01% | -0.01% | 75% | 64% |
| Class B | -0.01% | 0.03% | 0.01% | 0.00% | 0.03% | -0.02% | 74% | 60% |
| Class C | -0.01% | 0.00% | -0.01% | 0.00% | 0.00% | -0.04% | 79% | 64% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.01% | 0.00% | 0.01% | 0.00% | 0.01% | -0.01% | 76% | 63% |
| Class D | 0.00% | 0.05% | 0.03% | 0.00% | 0.06% | 0.09% | 80% | 67% |
| Class F | -0.03% | 0.01% | -0.02% | -0.03% | 0.02% | -0.03% | 79% | 62% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -0.02% | -0.35% | -0.15% | -0.06% | -0.50% | 0.24% | 79% | 66% |
| Class C | 0.00% | -0.19% | -0.28% | 0.01% | -0.41% | -0.24% | 79% | 66% |
| Class E | -0.04% | 0.25% | 0.19% | -0.08% | 0.39% | 0.33% | 74% | 61% |
| Overall | -0.02% | -0.15% | -0.11% | -0.04% | -0.24% | 0.11% | 77% | 65% |
| Class D | 0.05% | -0.05% | 0.38% | 0.00% | 0.12% | -0.09% | 82% | 66% |
| Class F | 0.06% | 0.08% | -0.45% | 0.05% | -0.16% | -0.68% | 79% | 66% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 66% | 61% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 64% | 58% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 66% | 58% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 71% | 58% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 68% | 59% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 67% | 59% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 71% | 56% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 79% | 60% |

Note: Results from InterDigital, crosschecked by xxxx.

## Comparison to NNVC-8.0 anchor

### NNVC-8.0 anchor vs NNVC-8.0 HOP.3

### Official results for HOP.

The results below reflect LOP/HOP performance differences (LOP set as anchor, HOP as test).

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.96% | -0.24% | -9.22% | -8.51% | -3.49% | -11.65% | 219% | 1448% |
| Class A2 | -8.53% | -1.20% | -0.31% | -7.90% | -0.99% | 1.03% | 213% | 1558% |
| Class B | -6.33% | 0.50% | -0.15% | -5.30% | 0.39% | -0.49% | 215% | 1546% |
| Class C | -7.32% | -2.94% | -2.77% | -5.82% | -0.19% | -0.77% | 181% | 1410% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -7.36% | -0.91% | -2.69% | -6.60% | -0.82% | -2.49% | 206% | 1491% |
| Class D | -7.98% | -2.35% | -2.76% | -5.20% | 0.60% | 0.35% | 183% | 1334% |
| Class F | -5.13% | -3.97% | -1.72% | -4.90% | -3.09% | -1.88% | 272% | 1716% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -6.00% | 5.88% | 8.70% | -5.22% | 5.79% | 8.60% | 222% | 1490% |
| Class C | -7.09% | -0.03% | 3.22% | -5.30% | 4.96% | 6.66% | 190% | 1510% |
| Class E | -6.92% | -2.08% | 4.80% | -5.54% | 0.45% | -0.18% | 362% | 1844% |
| Overall | -6.59% | 1.92% | 5.90% | -5.33% | 4.18% | 5.76% | 238% | 1579% |
| Class D | -7.69% | -2.26% | 5.41% | -5.46% | 3.92% | 9.46% | 176% | 1335% |
| Class F | -6.03% | -3.70% | -0.20% | -5.94% | -1.71% | -3.03% | 289% | 1681% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -4.64% | 4.09% | -4.26% | -5.92% | 1.33% | -6.29% | 188% | 1434% |
| Class A2 | -5.80% | 2.38% | 3.18% | -6.39% | 2.06% | 3.54% | 145% | 1350% |
| Class B | -4.14% | 3.60% | 3.24% | -3.94% | 2.58% | 2.00% | 141% | 1293% |
| Class C | -5.20% | 0.88% | -1.01% | -4.33% | 1.85% | -0.06% | 118% | 1075% |
| Class E | -5.74% | -1.61% | -0.16% | -5.62% | -0.68% | -1.57% | 151% | 1243% |
| Overall | -5.00% | 2.01% | 0.47% | -5.04% | 1.58% | -0.18% | 145% | 1264% |
| Class D | -4.55% | 0.94% | -0.81% | -3.77% | 2.02% | 1.86% | 120% | 1099% |
| Class F | -3.88% | -1.04% | 0.22% | -3.58% | -0.93% | -0.15% | 121% | 1246% |

Note: Results from InterDigital, crosschecked by xxxx.

### NNVC-8.0 vs NNVC RPR

From JVET-AH0099

### For scaling x2 (crosschecked by Interdigital):

|  | BD-rate Over NNVC-8.0 anchor |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.06% | 1.25% | 2.77% |  |  |  |  |  |
| Class A2 | 0.09% | 10.28% | 9.82% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.59% | 2.31% | 2.52% |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class F | 0.00% | 0.00% | 0.00% |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 anchor |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.48% | 3.38% | 4.33% |  |  |  |  |  |
| Class A2 | -0.85% | 9.15% | 11.64% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class E | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Overall | -0.39% | 2.09% | 2.66% |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class F | 0.00% | 0.00% | 0.00% |  |  |  |  |  |

For scaling x1.5

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 (NN-Intra LOP2) |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -2.72% | 0.02% | 0.74% |  |  |  |  |  |
| Class A2 | -1.19% | 6.95% | 6.65% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.78% | 1.39% | 1.48% |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class F | 0.00% | 0.00% | 0.00% |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 (NN-Intra LOP2) |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.27% | 0.19% | 1.28% |  |  |  |  |  |
| Class A2 | -2.65% | 7.00% | 7.92% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class E | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Overall | -0.65% | 1.20% | 1.53% |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class F | 0.00% | 0.00% | 0.00% |  |  |  |  |  |

### NNVC-8.0 anchor vs NNVC-8.0 NRSR unified

### Official results for NNSR.

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR |  | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  | DecT CPU |  |
| Class A1 | -3.20% |  | 0.60% |  | 1.79% |  | -6.14% |  | -0.69% |  | -0.98% |  | 76% |  | 56% |  |
| Class A2 | -1.00% |  | 6.51% |  | 7.00% |  | -5.64% |  | 7.96% |  | 7.02% |  | 78% |  | 65% |  |
| Class B | 0.00% |  | 0.00% |  | 0.00% |  | 0.00% |  | 0.00% |  | 0.00% |  | 96% |  | 95% |  |
| Class C | 0.00% |  | 0.00% |  | 0.00% |  | 0.00% |  | 0.00% |  | 0.00% |  | 94% |  | 95% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.84% |  | 1.42% |  | 1.76% |  | -2.36% |  | 1.45% |  | 1.21% |  | 88% |  | 79% |  |
| Class D | 0.00% |  | 0.00% |  | 0.00% |  | 0.00% |  | 0.00% |  | 0.00% |  | 97% |  | 102% |  |
| Class F | 0.00% |  | 0.00% |  | 0.00% |  | 0.00% |  | 0.00% |  | 0.00% |  | 93% |  | 92% |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.08% | 4.37% | 4.92% | -3.29% | 4.55% | 5.81% | 111% | 116% |
| Class A2 | -2.07% | 5.11% | 8.48% | -8.26% | 4.31% | 8.41% | 84% | 120% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 92% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 90% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 94% | 100% |
| Overall | -0.53% | 1.58% | 2.23% | -1.93% | 1.48% | 2.37% | 93% | 105% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 89% | 91% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 93% | 100% |

Note: Results from Oppo checked by Interdigital.

## Other tools

Other results remain the same as tools were not changed.

# Contributions

We have 3 contributions for AhG14.

| JVET-AH0064 | AHG14: Port VTM Temporal Filter Fixes to NNVC software and Enable Temporal Filter in NNVC LD CTC | X. Li (Google) |
| --- | --- | --- |
| JVET-AH0088 | AhG14: SADL update | F. Galpin, T. Dumas, P. Bordes, E. Francois (InterDigital) |
| JVET-AH0106 | AHG14: The extension of SADL library | N. Fu, W. Bao, Z. Chen (Wuhan Univ.) |

# Configurations

The following configurations is used to generate the different NNVC results.

The column “tested” is read as follow:

- Y: the configuration has been tested using the new NNVC-7.1 software

- P: the results are the ones from previous NNVC software basis

- N: not tested.

The column “xcheck” is read as follow:

- Y: the test has been cross-checked

- P: no cross-checked performed but results are consistent with previous version on NNVC

- N: no cross-check available

| Name | Tools | Configuration | Tested | Xcheck |
| --- | --- | --- | --- | --- |
| NNVC VTM mode | none | encoder_xxx_vtm.cfg | P |  |
| NNVC Anchor/EE1 | Intra Pred + LOP.2 filterset | encoder_xxx_nnvc.cfg | Y | Y |
| NNVC. HOP | Intra Pred + HOP.3 | encoder_xxx_nnvc.cfg + nn-based/HOP.cfg | Y | Y |
| HOP only | HOP | encoder_xxx_vtm.cfg + nn-based/HOP.cfg | Y | Y |
| Intra lower complexity | NN Intra lower complexity | nn-based/intra2.cfg | Y | Y |
| Sr | Super-resolution | nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg | P |  |
| Pf | Adaptive post-filters | nn-based/nnpf/nnpf_xxx.cfg | P |  |

Deprecated options:

| set0 | Loop filter set #0 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_1.cfg | P |  |
| --- | --- | --- | --- | --- |
| set1 | Loop filter set #1 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_2.cfg | P |  |
| set0+rdo | Loop filter set #0 + Rdo | nn-based/NnlfOption_1.cfg + --EncNnlfOpt=1 | P |  |
| set1+rdo+intra+temporal filter | Loop filter set #1 + Temporal filter | nn-based/NnlfOption_2.cfg + --NnlfSet1Multiframe=1 | P |  |

# Recommendations

The AHG recommends to:

- Continue to develop NNVC software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/issues

Encourage people to submit merge requests fixing identified bugs.