| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 14–24 April 2024 | Document: JVET-AH0023-v1 |
| --- | --- |

| Title: | EE1: Summary report of exploration experiment on neural network-based video coding |  |  |
| --- | --- | --- | --- |
| Status: | input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, R. Chang, F. Galpin, Y.Li, D. Rusanovskyy, M. Santamaria, J. Ström, and Z. Xie | Email: | elena.alshina@huawei.com renjiechang@tencent.com franck.galpin@interdigital.com yue.li@bytedance.com dmytror@qti.qualcomm.com maria.santamaria_gomez@nokia.com jacob.strom@ericsson.com xiezhihuang@oppo.com |
| Source: | EE Coordinators |  |  |

_____________________________

# Abstract

This report summarizes the activities of the Exploration Experiment 1 (EE1) performed between the 33rd and 34th JVET meetings to evaluate Neural Network-based Video Coding (NNVC) technologies, analyze their performance, evaluate their complexity aspects, and clarify training procedure. Two teleconferences have been conducted during this meeting circle. With combination of two neural network-based tools 14-15% gain over VTM in random access configuration can be achieved, with roughly 470 kMAC/pxl computational complexity,  2.5 increase of encoding run time and 100 increase of decoding run time.  Up to 1% compression gain on top of NNVC-8.0 was demonstrated by adaptive resolution coding with neural network-based super-resolution filter. In this round of EE1 NN-Inter tool provided 2.6% compression gain on top of NNVC-8.0. Very low complexity (5 kMAC/pxl) version on NNLF was tested and provided. This very low complexity NNLF provide 3.7% compression gain with roughly one third of LOP NNLF complexity.

# Introduction

It was decided 1  that the code base for the test should be NNVC-8.0. The anchor for all tests is default configuration of NNVC-8.0 (NN-Intra and LOP-2 filter enabled). Anchor performance results provided by AhG 14. Results reporting template of AhG11 2  shall be used by all proponents.

For tests competing with technologies in NNVC-8 it was agreed to configure the proposed solution targeting close to existing NNVC tool,  but not exceeding:

- kMAC/pxl of EE1 test  kMAC/pxl NNVC (must),

- Number of Parameters EE1 test   Number of Parameters NNVC (if possible).

NN architecture provided in this test description should not be changed, beside minor adjustment for parameters (such as channels number) in order to meet recommendation above.

Exact parameters settings were announced by proponents by 2nd AhG11/14 teleconference on Feb. 28 3.

EE1 took efforts to align both training and testing across tests in each category. This was one of the goals of the 1st teleconference on Feb. 13 3. Namely training procedure for LOP filter (stage III) was improved in the test E1-1.0 and recommended to be used in all LOP filter tests (if applicable). Adaptive resolution coding has two sources of performance improvement: encoder choice for resolution and re-sampling filter. Parameters for adaptive resolution selection were studied in EE1-4.1 (using RPR re-sampling filter), shared during 1st teleconference and recommended to be used in all tests for NN-based super-resolution.

# List of tests

This round of EE1 tests includes:

- EE1-1: LOP in-loop filter

  - EE1-1.0: LOP re-training training strategy JVET-AG0156 JVET-AH0042

  - EE1-1.1: training of EE1-0 with architecture change JVET-AG0163 (merge convolutions) & JVET-AG0155 (separable convolutions). JVET-AH0050

  - EE1-1.2: training of EE1-0 for  JVET-AG0163 (merge convolutions) & JVET-AG0155 (separable convolutions) & JVET-AG0069 (transformed input). JVET-AH0080

  - EE1-1.3: training from the scratch  for  JVET-AG0163 (merge convolutions) & JVET-AG0155 (separable convolutions) & JVET-AG0069 (transformed input).JVET-AH0081

  - EE1-1.4: LOP2 architecture with content adaptivity  JVET-AG0111  JVET-AH0096

- EE1-2: HOP in-loop filter

  - EE1-2.1: Separate HOP model JVET-AG0175  --> JVET-AH0188

  - EE1-2.2: Wide activation HOP model JVET-AG0179  JVET-AH0189

  - EE1-2.3: HOP with transformers JVET-AG0162  JVET-AH0205

- EE1-3: NN-inter prediction

  - EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement JVET-AG0122  JVET-AH0107

- EE1-4: NN-super-resolution

  - EE1-4.0: adaptive resolution (s=2) coding for NNVC-8 (NNIntra and LOP2 filter) with NNVC SR filter

  - EE1-4.1: adaptive resolution (s=1.5 2) coding for NNVC-8 (NNIntra and LOP2 filter) with RPR  JVET-AH0099

  - EE1-4.2: adaptive resolution (s=1.5 2) coding for NNVC-8 (NNIntra and LOP2 filter) with JVET-AG0129  JVET-AH0104

  - EE1-4.3: adaptive resolution (s= 2) coding for NNVC-8 (NNIntra and LOP2 filter) with JVET-AG0114 (LOP version)  JVET-AH0133

- EE1-5. Very Low Operational Point

  - EE1-5.1 Study of the NN architecture at Very Low Operational Point (JVET-AG0155, JVET-AG0057, JVET-AF0206)  JVET-AH0051

All neural network tools combinations tested are summarized in Table 1. Results in this table reported relatively to NNVC configures as VTM. Run time is not always reliable, so it is shown in grey font. In each test category (except EE1-5) table shows performance of tools combination already available in NNVC, those tests are marked by prefix ‘NNVC8’.

Tabulated results are visualized ion Figure 1 and Figure 2.  Blue dots show combination of neural network-based filters with neural network based Intra. On Figure 1 we see that compression gain (BD-rate) for filters increases linearly with log of computational complexity.

Orange dots show tests in which the neural network-based Inter tool is added as the third tool into the combination of LOP filter and nnIntra.

Orange dots show tests in which the neural network-based Inter tool is added as the third tool into the combination of LOP filter and nnIntra.

Table 1 Summary of all test results vs NNVC (configured as VTM)

|  | Test | Random Access cfg BD-rate vs VTM |  |  |  |  | All Intra cfg BD-rate vs VTM |  |  |  |  | # tools combi-ned | Total kMAC /pxl | Total Num Param |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |  |  |  |
| LOP | NNVC8 | -6.9% | -13.2% | -12.1% | 1.2 | 73 | -8.1% | -13.3% | -13.4% | 1.7 | 44 | 2 | 24.9 | 1.6 |
|  | EE1-1.0 | -7.0% | -13.0% | -12.8% | 1.2 | 83 | -8.1% | -13.3% | -14.2% | 1.7 | 56 | 2 | 24.8 | 1.6 |
|  | EE1-1.1 | -7.2% | -11.8% | -10.9% | 1.1 | 93 | -8.2% | -12.1% | -12.3% | 1.1 | 51 | 2 | 24.8 | 1.5 |
|  | EE-1.2 | -7.3% | -13.1% | -11.2% | 1.2 | 57 | -8.4% | -14.2% | -13.9% | 1.7 | 40 | 2 | 24.7 | 1.7 |
|  | EE-1.3 | -7.6% | -14.0% | -12.2% | 1.1 | 58 | -8.5% | -14.7% | -14.4% | 1.7 | 42 | 2 | 24.7 | 1.7 |
|  | EE-1.4 | -8.0% | -15.0% | -14.4% | 2.1 | 63 |  |  |  |  |  | 2 | 24.9 | 1.6 |
| HOP | NNVC8HOP | -13.7% | -13.9% | -14.5% | 2.5 | 1092 | -12.7% | -11.6% | -13.0% | 2.4 | 558 | 2 | 473.8 | 2.9 |
|  | EE1-2.1 | -13.7% | -15.7% | -17.0% |  |  | -12.7% | -14.2% | -15.6% |  |  | 2 | 466.8 | 2.9 |
|  | EE1-2.2 | -13.9% | -14.4% | -14.8% |  |  | -12.7% | -12.0% | -13.0% |  |  | 2 | 468.8 | 2.9 |
|  | EE1-2.3.1 | -14.0% | -19.0% | -18.8% |  |  | -13.4% | -15.7% | -16.8% |  |  | 2 | 472.8 | 2.9 |
|  | EE1-2.3.2 | -14.6% | -23.5% | -23.8% |  |  | -13.6% | -19.3% | -20.4% |  |  | 2 | 472.8 | 2.9 |
| Inter | EE1-3.1 | -8.4% | -14.5% | -13.3% | 1.4 | 292 |  |  |  |  |  | 3 | 93.9 | 45 |
|  | EE1-3.2 | -9.3% | -14.7% | -13.5% | 1.5 | 601 |  |  |  |  |  | 3 | 207 | 4.5 |
| SuperRes | NNVC8RPR | -7.5% | -10.9% | -9.7% |  |  | -8.5% | -11.3% | -11.0% |  |  | 2 | 24.9 | 1.6 |
|  | EE1-4.0 | -7.8% | -11.8% | -10.5% |  |  | -8.6% | -11.8% | -11.3% |  |  | 3 | 45.4 | 1.7 |
|  | EE1-4.1x1.5 | -7.6% | -11.8% | -10.7% |  |  | -8.7% | -12.1% | -12.0% |  |  | 2 | 24.9 | 1.6 |
|  | EE1-4.1x .0 | -7.5% | -10.9% | -9.7% |  |  | -8.5% | -11.3% | -11.0% |  |  | 2 | 24.9 | 1.6 |
|  | EE1-4.2 1.5 | -7.8% | -12.3% | -11.3% |  |  | -8.8% | -12.5% | -12.4% |  |  | 3 | 37.9 | 1.7 |
|  | EE1-4.2x2.0 | -7.9% | -11.8% | -10.8% |  |  | -8.8% | -12.1% | -11.9% |  |  | 3 | 37.9 | 1.7 |
| VLOP | EE1-5.2 | -5.3% | -5.5% | -5.0% | 1.2 | 38 | -7.1% | -7.8% | -7.7% | 1.7 | 26 | 2 | 7.8 | 1.5 |

Figure 1 BD-rate (Y) relatively to VTM anchor in random access (RA) configuration vs kMAC/pxl for different tools combinations.

Figure 2 BD-rate (Y) relatively to VTM anchor in random access (RA) configuration total number of parameters for different tools combinations.

More details about tested tools can be found in presentation attached to this report.

# SW location

https://vcgit.hhi.fraunhofer.de/jvet-ag-ee1/VVCSoftware_VTM/-/tree/EE1-X.X

# References

[1]  E. Alshina, R.-L. Liao, S. Liu, A. Segall  Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AF2016.

[2]  E. Alshina, F. Galpin, R. Chang, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, Z. Xie  Exploration Experiments on Neural Network-based Video Coding (EE1)  JVET-AG2023.

[3]  E. Alshina, F. Galpin  [AHG11] [AHG14] AHG meeting report on NNVC  JVET-AH0043