| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0043-v2 |
| --- | --- |

| Title: | AhG11/AhG14 teleconference |  |  |
| --- | --- | --- | --- |
| Status: | input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, F. Galpin | Email: | elena.alshina@huawei.com franck.galpin@interdigital.com |
| Source: | AhG11/14 chairs |  |  |

_____________________________

# Abstract

This document summarizes discussion during AhG11/AhG14 teleconferences. First one was conducted on February 13, 2024. The second one conducted on February 28, 2024.  In total 39 experts attended those teleconferences. NNVC software development and EE1 tests have been discussed during those telcos.

# Agenda

First teleconference on 13.02.24

- up-dates on NNVC SW

  - Potential viewing preparation

- LOP NNLF filter training strategy (EE1-1.0)

- Adaptive resolution coding with RPR filter (EE1-4.1)

Second teleconference on 28.02.24

- Final settings for EE1 tests

# Up-dates on NNVC SW

Reported by AhG14.

EE1 SW branches to be established as

https://vcgit.hhi.fraunhofer.de/jvet-ag-ee1/VVCSoftware_VTM/-/tree/EE1-X.X

(after NNVC-8.0 official release)

Anchor results (NNVC-8.0) are available at jvet-ahg-nnvc / nnvc-ctc · GitLab (fraunhofer.de)

Following tests can be performed with NNVC-8.0 SW to be used for visual tests vs VVC:

- HOP + NNIntra

- HOP + NNIntra + NNSR adaptive resolution (NNVC 8.0)

# LOP NNLF filter training strategy (EE1-1.0)

In this test, original LOP2 architecture is used, see Figure 1. Participants have jointly trained LOP2 Stage 3 with batch size 64 and learning rate 0.0008, as proposed in JVET-AG0156 and JVET-AG0163 (EE1-2.3). The goal of the test is to verify stability, replicability of the proposed training strategy. No changes to LOP2 architecture and training dataset are introduced. Changes to number of training epochs, L1/L2 and learning rate points are anticipated.

Inference: the NNVC 8.0 software is used for inference, following LOP anchor settings.

Figure 1.

Training was conducted by Qualcomm. Dolby, Ittiam. Experiments with maximum number of epoch and scheduler.

- Test 0: ‘milestones’ 55, 57, 58, mse-epoch 58, max epochs 60 –  Luma BD-rate results 0.09% drop in all-intra, 0.09% gain in random access.

- Test 1: ‘milestones’ 37, 39, 40, mse-epoch 40, max epochs 60 –  Luma BD-rate results 0.11% drop in all-intra, 0.09% gain in random access.

- Test 2: ‘milestones’ 41, 51, 55, mse-epoch 58, max epochs 60 –  Luma BD-rate results 0.04% drop in all-intra, 0.17% gain in random access.

‘milestones’ – epoch learning rate reduced by factor 10, ‘mse epoch’ – epoch L1 loss changed to L2 loss.

With new training strategy training time (10 day reduced to 8 day) and memory requirements are lower.

Small drop in Luma performance in ‘all intra test’ compensated by Chroma gain.

More details can be found in JVET-AH0042.

Recommendation to use Test 2 setting for training LOP and EE1-1 tests (if applicable).

To-do: 1) upload code and test results to EE1-1.0 SW branch

# Adaptive resolution coding with RPR filter (EE1-4.1)

In this test, a study on encoder strategy for RPR with NN LF and NN-intra tools enabled is conducted. No training is expected because RPR filter is used. Two sub-tests are planed:

EE1-4.1.1 scaling factor s=2

EE1-4.1.2 scaling factor s=1.5

Test results reported by Bytedance (s=2) and Vivo (s=1.5).

Anchor: NN-Intra and LOP2 NNLF are enabled.

Test: Adaptive resolution coding with RPR filter, NN-Intra and LOP2 NNLF are enabled.

Encoder strategy for scaling factor and QP selection same as in NNVC-7.1, except only s=1 is allowed for QP 32.

SW modification is needed to reproduce results. Merge request to be submitted by Bytedance will be used in NNVC-8.0.

Memory buffer overflow happens due to NNVC SW bug. Bug fix was fixed by Vivo. Merge request to eb submitted asap, will be used in NNVC-8.0.

Test results for random access configuration, average among all mandator classes:

Scaling factors s=1 or s=2 :  0.6% Luma gain and 2.2% Chroma drop.

Scaling factors s=1 or s=1.5:  0.8% Luma gain and 1.4% Chroma drop.

To-do: 1) upload code and test results to EE1-4.1 SW branch, 2) submit two merge requests to fix NNVC-2.0

# Final settings for EE1 tests

For tests competing with technologies in NNVC-8 it is recommended to configure the proposed solution to have a complexity close to related NNVC(LOP/HOP), but not exceeding:

- kMAC/pxl of EE1 test  kMAC/pxl NNVC (must),

- Number of Parameters EE1 test   Number of Parameters NNVC (if possible).

NN architecture provided in this test description should not be changed, beside minor adjustment for parameters (such as channels number) in order to meet recommendation above. Exact parameters settings will be specified during 2nd AhG11/14 teleconference Feb. 28.