| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, Online, 17-26 January2024 | Document: JVET-Ax00xx-v1 |
| --- | --- |

| Title: | EE1-4.1.2: Adaptive resolution coding for NNVC-7.1 with RPR using scaling factor s=1.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhuoyi. Lyu, Chuan. Zhou | Tel: Email: | zhuoyi.lv@vivo.com chuan.zhou@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

This document reports the results of EE1-4.1.2: Adaptive resolution coding for NNVC-7.1 with RPR using scaling factor s=1.5.

In addition, one modification is recommended to be adopted to NNVC-8.0.

Comparison to NNVC-7.1 anchor

The NNVC-7.1 anchor with NN-Intra and LOP-2 filter enabled.

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM |  | V-MSIM |  | EncT |  | DecT CPU |  |
| Class A1 | -2.71% | -0.03% | 0.77% |  |  |  |  |  |  |  |  |
| Class A2 | -1.20% | 6.98% | 6.66% |  |  |  |  |  |  |  |  |
| Overall | -1.96% | 3.48% | 3.71% |  |  |  |  |  |  |  |  |  |
|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-7.1 |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT | DecT CPU |
| Class A1 | -1.27% | 0.19% | 1.28% |  |  |  |  |  |  |  |  |
| Class A2 | -2.65% | 7.00% | 7.92% |  |  |  |  |  |  |  |  |
| Overall | -1.96% | 3.59% | 4.60% |  |  |  |  |  |  |  |  |

Note: Results from vivo, crosschecked by Bytedance for gains.

QP offset for scaling factor s=1.5

nnsr_classA1.cfg

NnsrQpOffset                   : 4

NnsrQpOffsetOverrideByQp       : 22 3 27 3 32 3 37 3 42 3

nnsr_classA2.cfg

NnsrQpOffset                   : 4

NnsrQpOffsetOverrideByQp       : 22 4 27 4 32 4 37 4 42 4

Recommended modifications of NNVC-8.0

- NNLF-related buffer overflow may occur in the decoder side when picture is down-sampled.

m_nnfilterUnified.init(m_nnlfModelName,

pps->getPicWidthInLumaSamples(), pps->getPicHeightInLumaSamples(), sps->getChromaFormatIdc(), sps->getNnlfUnifiedMaxNumPrms());

It is recommended to be changed as:

m_nnfilterUnified.init(m_nnlfModelName,

sps->getMaxPicWidthInLumaSamples(), sps->getMaxPicHeightInLumaSamples(), sps->getChromaFormatIdc(), sps->getNnlfUnifiedMaxNumPrms());

Conclusion

In this document, the results of EE1-4.1.2: Adaptive resolution coding for NNVC-7.1 with RPR using scaling factor s=1.5 are reported.

References

[1] E. Alshina, R.-L. Liao, S. Liu, A. Segall  Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AF2016.