| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF1006-v2 |
| --- | --- |

| Title: | New profiles, colour descriptors, and SEI messages for HEVC (draft 2) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Y.-K. Wang B. Bross T. Ikai G. J. Sullivan A. Tourapis | Email: | yekui.wang@bytedance.com benjamin.bross@hhi.fraunhofer.de ikai.tomohiro@sharp.co.jp gary.sullivan@dolby.com atourapis@apple.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains the draft text for changes on new profiles, colour descriptors, and SEI messages for the 5th edition of the High Efficiency Video Coding (HEVC) standard (Rec. ITU-T H.265 | ISO/IEC 23008-2). The changes include: 1) the support of seven new profiles, namely the Extended Multiview Main, Multiview Main 10, Extended Multiview Main 10, Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, and Multiview Monochrome 16 profiles; 2) the support of three additional colour type identifiers; 3) the HEVC-specific supports for some supplemental enhancement information (SEI) messages that may be included in HEVC bitstreams but are not to be specified in the HEVC specification, and 4) some technical corrections and editorial improvements to the 5th edition text of HEVC. The SEI messages are the neural network post-filter characteristics (NNPFC) SEI message, the neural-network post-filter activation (NNPFA) SEI message, and the phase indication SEI message, that are to be specified in the 3rd edition of the Versatile Supplemental Enhancement Information messages for coded video bitstreams (VSEI) standard (ITU-T H.274 | ISO/IEC 23002-7).

Draft 2 incorporated items:

- In 7.2: Added a NOTE on the b(8) syntax descriptor. [JVET-AF0064]

- In 8.5.3.2.2, bullet item 6: Replaced "variable" with "variables". [Email from C. Reader]

- In D.2.24: Changed the coding of nesting_zero_bit from u(1) to f(1). [JVET-AF0064]

- In D.2.44: Changed the coding of mcts_nesting_zero_bit from u(1) to f(1). [JVET-AF0064]

- In D.3.13, Eqn. D-15: Added a closing curly bracket after "G[ c ][ x + 1 ][ y − 1 ]". [FR008 in WG 5 N 243]

- In D.3.45: Improved the phrasing of an SEI message being indicated by the encoder (i.e., the content producer) as being "necessary". [Email from G. J. Sullivan]

- In D.3.45, D.3.46, and I.14.3.3: Appended "in decoding order" after "the first access unit of the CVS". [JVET-AF0189]

- In F.3.29: Improved the phrasing of the NOTE. [Email dicussion between A. Tourapis and Y.-K. Wang]

- In F.7.4.2.2: Improved the phrasing of NOTE 1. [Email dicussion between A. Tourapis and Y.-K. Wang]

- In F.14.2.5: Changed the coding of bsp_nesting_zero_bit from u(1) to f(1). [JVET-AF0064]

- In G.11.1.1, first sentence: Changed "the Multiview Main and Multiview Main 10 profile" to "the Multiview Main or Multiview Main 10 profile". [JVET-AF0063]

- In G.11.1.1 and I.11.1.1: Removed "to" from "greater than to". [JVET-AF0063]

- In G.11.1.1: Applied the following constraint to the Multiview Main 10 profile: "All active PPSs for layers in subBitstream shall have colour_mapping_enabled_flag equal to 0 only." [JVET-AF0063]

- In G.11.1.2: Changed the constraint for the multiview monochrome profiles requiring chroma_format_idc to be equal to 1 to requiring chroma_format_idc to be equal to 0. [JVET-AF0063]

- In G.11.1.2: Replaced "exensions" with "extensions". [JVET-AF0063]

Draft 1 incorporated items:

- Changes for NNPF and phase indication SEI messages (based on JVET-AE0101 as well as the latest changes to the interface text for the NNPF SEI messages agreed at the JVET-AE meeting

- JVET-AD1008 Additional colour type identifiers

- Fixes and changes for the film grain characteristics SEI message

- New multiview profiles from JVET-AE0296, plus an additional Multiview Monochrome 10 profile

- Some errata items to subclauses A.4.1, A.4.2, G.11.2.3, and H.11.2.2 reported by Cliff Reader (thanks!)

- Changes to D.1 related to referring to VSEI in HEVC

# Changes to the specification text of HEVC

# Subclause 2.3

Add the following reference:

Rec. ITU-T H.274 | ISO/IEC 23002-7 (in force) Versatile supplemental enhancement information messages for coded video bitstreams

# Subclause 7.2

Replace the following:

—	b(8): byte having any pattern of bit string (8 bits). The parsing process for this descriptor is specified by the return value of the function read_bits( 8 ).

with the following:

—	b(8): byte having any pattern of bit string (8 bits). The parsing process for this descriptor is specified by the return value of the function read_bits( 8 ).

NOTE 1	The b(8) syntax descriptor is functionally the same as u(8) but is used in contexts where there is a desire to emphasize that the interpretation of the syntax element involves the processing of bytes and that each byte is not necessarily interpreted as an 8-bit unsigned integer Also, b(8) is only used at byte-aligned positions in the syntax.

Append a note number to the existing NOTE, to become NOTE 2.

# Subclause 8.5.3.2.2

In bullet item 6, replace "The variable numCurrMergeCand and numOrigMergeCand are ..." with "The variables numCurrMergeCand and numOrigMergeCand are ...".

# Subclause A.4.1

In Table A.8, make the following changes:

- Replace "Max slice segments per picture MaxSliceSegmentsPerPicture" with "Max # of slice segments per picture MaxSliceSegmentsPerPicture".

- Insert a line break in the table heading before the global variable MaxTileRows, and similarly for the other global variables defined in this table, particularly MaxSliceSegmentsPerPicture, which itself in the ISO text is split into two lines.

# Subclause A.4.2

In Table A.9, make the following changes:

- Insert a line break in the table heading before MinCrBase in Table A.9, and similarly for the other global variables defined in this table.

Make the following change:

Min( (Max( 1, MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxSliceSegmentsPerPicture )

# Subclause D.1

Replace the content of subclause D.1 with the following:

This annex specifies 1) syntax and semantics for the SEI message payload, which is the container of SEI messages, 2) the syntax and semantics for SEI message payloads some SEI messages, and 3) the use of the SEI messages for which the payload type values are specified in this document and the syntax and semantics are specified in Rec. ITU-T H.274 | ISO/IEC 23002-7.

SEI messages assist in processes related to decoding, display or other purposes. However, SEI messages are not required for constructing the luma or chroma samples by the decoding process. Conforming decoders are not required to process this information for output order conformance to this document (see Annex C and F.13 for the specification of conformance). Some SEI message information is required to check bitstream conformance and for output timing decoder conformance.

In C.5.2 and in F.13 including its subclauses, specifications for the presence of SEI messages are also satisfied when those messages (or some subset of them) are conveyed to decoders (or to the HRD) by other means not specified in this document. When present in the bitstream, SEI messages for which the syntax and semantics are specified in this document shall obey the syntax and semantics specified in 7.3.5 and this annex, and SEI messages for which the payload type values are specified in this document and the syntax and semantics are specified in Rec. ITU-T H.274 | ISO/IEC 23002-7 shall obey the syntax and semantics specified in ITU-T H.274 | ISO/IEC 23002-7. When the content of an SEI message is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the SEI message is not required to use the same syntax specified in this annex or ITU-T H.274 | ISO/IEC 23002-7. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

# Subclause D.2.1

Replace the general SEI message syntax with the following:

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 2 ) |  |
| pan_scan_rect( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 6 ) |  |
| recovery_point( payloadSize ) |  |
| else if( payloadType = = 9 ) |  |
| scene_info( payloadSize ) |  |
| else if( payloadType = = 15 ) |  |
| picture_snapshot( payloadSize ) |  |
| else if( payloadType = = 16 ) |  |
| progressive_refinement_segment_start( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 19 ) |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 23 ) |  |
| tone_mapping_info( payloadSize ) |  |
| else if( payloadType = = 45 ) |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 47 ) |  |
| display_orientation( payloadSize ) |  |
| else if( payloadType = = 56 ) |  |
| green_metadata( payloadsize ) /* specified in ISO/IEC 23001-11 */ |  |
| else if( payloadType = = 128 ) |  |
| structure_of_pictures_info( payloadSize ) |  |
| else if( payloadType = = 129 ) |  |
| active_parameter_sets( payloadSize ) |  |
| else if( payloadType = = 130 ) |  |
| decoding_unit_info( payloadSize ) |  |
| else if( payloadType = = 131 ) |  |
| temporal_sub_layer_zero_idx( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 134 ) |  |
| region_refresh_info( payloadSize ) |  |
| else if( payloadType = = 135 ) |  |
| no_display( payloadSize ) |  |
| else if( payloadType = = 136 ) |  |
| time_code( payloadSize ) |  |
| else if( payloadType = = 137 ) |  |
| mastering_display_colour_volume( payloadSize ) |  |
| else if( payloadType = = 138 ) |  |
| segmented_rect_frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 139 ) |  |
| temporal_motion_constrained_tile_sets( payloadSize ) |  |
| else if( payloadType = = 140 ) |  |
| chroma_resampling_filter_hint( payloadSize ) |  |
| else if( payloadType = = 141 ) |  |
| knee_function_info( payloadSize ) |  |
| else if( payloadType = = 142 ) |  |
| colour_remapping_info( payloadSize ) |  |
| else if( payloadType = = 143 ) |  |
| deinterlaced_field_identification( payloadSize ) |  |
| else if( payloadType = = 144 ) |  |
| content_light_level_info( payloadSize ) |  |
| else if( payloadType = = 145 ) |  |
| dependent_rap_indication( payloadSize ) |  |
| else if( payloadType = = 146 ) |  |
| coded_region_completion( payloadSize ) |  |
| else if( payloadType = = 147 ) |  |
| alternative_transfer_characteristics( payloadSize ) |  |
| else if( payloadType = = 148 ) |  |
| ambient_viewing_environment( payloadSize ) |  |
| else if( payloadType = = 149 ) |  |
| content_colour_volume( payloadSize ) |  |
| else if( payloadType = = 150 ) |  |
| equirectangular_projection( payloadSize ) |  |
| else if( payloadType = = 151 ) |  |
| cubemap_projection( payloadSize ) |  |
| else if( payloadType = = 152 ) |  |
| fisheye_video_info( payloadSize ) |  |
| else if( payloadType = = 154 ) |  |
| sphere_rotation( payloadSize ) |  |
| else if( payloadType = = 155 ) |  |
| regionwise_packing( payloadSize ) |  |
| else if( payloadType = = 156 ) |  |
| omni_viewport( payloadSize ) |  |
| else if( payloadType = = 157 ) |  |
| regional_nesting( payloadSize ) |  |
| else if( payloadType = = 158 ) |  |
| mcts_extraction_info_sets( payloadSize ) |  |
| else if( payloadType = = 159 ) |  |
| mcts_extraction_info_nesting( payloadSize ) |  |
| else if( payloadType = = 160 ) |  |
| layers_not_present( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 161 ) |  |
| inter_layer_constrained_tile_sets( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 162 ) |  |
| bsp_nesting( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 163 ) |  |
| bsp_initial_arrival_time( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 164 ) |  |
| sub_bitstream_property( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 165 ) |  |
| alpha_channel_info( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 166 ) |  |
| overlay_info( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 167 ) |  |
| temporal_mv_prediction_constraints( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 168 ) |  |
| frame_field_info( payloadSize ) /* specified in Annex F */ |  |
| else if( payloadType = = 176 ) |  |
| three_dimensional_reference_displays_info( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 177 ) |  |
| depth_representation_info( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 178 ) |  |
| multiview_scene_info( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 179 ) |  |
| multiview_acquisition_info( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 180 ) |  |
| multiview_view_position( payloadSize ) /* specified in Annex G */ |  |
| else if( payloadType = = 181 ) |  |
| alternative_depth_info( payloadSize ) /* specified in Annex I */ |  |
| else if( payloadType = = 200 ) |  |
| sei_manifest( payloadSize ) |  |
| else if( payloadType = = 201 ) |  |
| sei_prefix_indication( payloadSize ) |  |
| else if( payloadType = = 202 ) |  |
| annotated_regions( payloadSize ) |  |
| else if( payloadType = = 205 ) |  |
| shutter_interval_info( payloadSize ) |  |
| else if( payloadType = = 210 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_characteristics( payloadSize ) |  |
| else if( payloadType = = 211 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_activation( payloadSize ) |  |
| else if( payloadType = = 212 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| phase_indication( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 132 ) |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 146 ) |  |
| coded_region_completion( payloadSize ) |  |
| else if( payloadType = = 210 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_characteristics( payloadSize ) |  |
| else if( payloadType = = 211 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_activation( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

# Subclause D.2.24

In the table row for nesting_zero_bit, replace “u(1)” with “f(1)”.

# Subclause D.2.44

In the table row for mcts_nesting_zero_bit, replace “u(1)” with “f(1)”.

# Subclause D.3.1

Make the following changes:

…

reserved_payload_extension_data shall not be present in bitstreams conforming to this version of this document. However, decoders conforming to this version of this document shall ignore the presence and value of reserved_payload_extension_data. When present, the length, in bits, of reserved_payload_extension_data is equal to 8 * payloadSize − nEarlierBits − nPayloadZeroBits − 1, where nEarlierBits is the number of bits in the sei_payload( ) syntax structure that precede the reserved_payload_extension_data syntax element and nPayloadZeroBits is the number of payload_bit_equal_to_zero syntax elements at the end of the sei_payload( ) syntax structure.

If more_data_in_payload( ) is TRUE after the parsing of the SEI message syntax structure (e.g., the buffering_period( ) syntax structure) and nPayloadZeroBits is not equal to 7, PayloadBits is set equal to 8 * payloadSize − nPayloadZeroBits − 1; otherwise, PayloadBits is set equal to 8 * payloadSize.

payload_bit_equal_to_one shall be equal to 1.

payload_bit_equal_to_zero shall be equal to 0.

NOTE 1	SEI messages with the same value of payloadType are conceptually the same SEI message regardless of whether they are contained in prefix or suffix SEI NAL units.

NOTE 2	For SEI messages with payloadType in the range of 0 to 47, inclusive, that are specified in this document, the payloadType values are aligned with similar SEI messages specified in Rec. ITU-T H.264 | ISO/IEC 14496-10.

The list SingleLayerSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 200 to 202, inclusive, and 205, and 210 to 212, inclusive.

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 200 to 202, inclusive, and 205, and 210 to 212, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 159, inclusive, 200 to 202, inclusive, and 205, and 210 to 212, inclusive.

NOTE 3	SingleLayerSeiList consists of the payloadType values of the SEI messages specified in Annex D excluding 0 (buffering period), 1 (picture timing), 4 (user data registered by Recommendation ITU-T T.35), 5 (user data unregistered), 130 (decoding unit information) and 133 (scalable nesting). VclAssociatedSeiList consists of the payloadType values of the SEI messages that, when non-scalable-nested and contained in an SEI NAL unit, infer constraints on the NAL unit header of the SEI NAL unit on the basis of the NAL unit header of the associated VCL NAL unit. PicUnitRepConSeiList consists of the payloadType values of the SEI messages that are subject to the restriction on 8 repetitions per picture unit.

…

# Subclause D.3.13

Replace the first sentence with the following:

This SEI message provides the decoder with a parameterized model for a film grain synthesis process. The film grain synthesis process should be applied to the decoded pictures prior to their display.

Replace formula D-15 with the following:

G[ c ][ x ][ y ] = ( comp_model_value[ c ][ sj ][ 0 ] * n[ c ][ x ][ y ] +
		comp_model_value[ c ][ sj ][ 1 ] * ( G[ c ][ x − 1 ][ y ] +
		( ( comp_model_value[ c ][ s j ][ 4 ] * G[ c ][ x ][ y − 1 ] ) >>	(D-15)
		log2_scale_factor ) ) + 	comp_model_value[ c ][ s j ][ 3 ] *
		( ( ( comp_model_value[ c ][ s j ][ 4 ] * ( G[ c ][ x − 1 ][ y − 1 ] + G[ c ][ x + 1 ][ y − 1 ] ) ) >>
		log2_scale_factor ) + G[ c ][ x + 1 ][ y − 1 ] )+
		comp_model_value[ c ][ s j ][ 5 ] * ( G[[ c ][ x − 2 ][ y ] +
		( ( comp_model_value[ c ][ s j ][ 4 ] * comp_model_value[ c ][ s j ][ 4 ] * G[ c ][ x ][ y − 2 ] ) >>
		( 2 * log2_scale_factor ) ) ) +
		comp_model_value[ c ][ s j ][ 2 ] * G[ c − 1 ][ x ][ y ] ) >> log2_scale_factor

# Subclause D.3.45

Replace the following:

An SEI message is indicated by the encoder (i.e., the content producer) as being "necessary" when the information conveyed by the SEI message is considered as necessary for interpretation by the decoder or receiving system in order to properly process the content and enable an adequate user experience; it does not mean that the bitstream is required to contain the SEI message in order to be a conforming bitstream. It is at the discretion of the encoder to determine which SEI messages are to be considered as necessary in a particular CVS. However, it is suggested that some SEI messages, such as the frame packing arrangement, segmented rectangular frame packing arrangement, and omnidirectional projection indication SEI messages, should typically be considered as necessary.

with the following:

An SEI message is indicated by the encoder (i.e., the content producer) as being "necessary" when the information conveyed by the SEI message is considered as necessary for interpretation by the decoder or receiving system in order to properly process the content and enable an adequate user experience; it does not mean that the bitstream is required to contain the SEI message in order to be a conforming bitstream and it does not mean that any particular decoder behaviour in response to such an SEI message is required for conformance to this version of this document. It is at the discretion of the encoder to determine which SEI messages are to be considered as necessary in a particular CVS. However, it is suggested that some SEI messages, such as the frame packing arrangement, segmented rectangular frame packing arrangement, and omnidirectional projection indication SEI messages, should typically be considered as necessary.

Replace the following:

When an SEI manifest SEI message is present in any access unit of a CVS, an SEI manifest SEI message shall be present in the first access unit of the CVS.

with the following:

When an SEI manifest SEI message is present in any access unit of a CVS, an SEI manifest SEI message shall be present in the first access unit of the CVS in decoding order.

# Subclause D.3.46

Replace the following:

When an SEI prefix indication SEI message is present in any access unit of a CVS, an SEI prefix indication SEI message shall be present in the first access unit of the CVS.

with the following:

When an SEI prefix indication SEI message is present in any access unit of a CVS, an SEI prefix indication SEI message shall be present in the first access unit of the CVS in decoding order.

# New subclause D.4

Add subclause D.4 as follows:

D.4 Use of SEI messages specified in other specifications

### D.4.1 General

The SEI messages having syntax structures identified in subclause ‎D.2.1 that are specified in other specifications, including Rec. ITU-T H.274 | ISO/IEC 23002-7, may be used together with bitstreams specified by this document.

When any particular SEI message specified in Rec. ITU-T H.274 | ISO/IEC 23002-7 is included in a bitstream specified by this document, the SEI payload syntax shall be as specified in Rec. ITU-T H.274 | ISO/IEC 23002-7, that syntax shall be included into the sei_payload( ) syntax structure as specified in subclause ‎D.2.1 and shall use the payloadType value specified in clause ‎D.2.1, the corresponding semantics specified in Rec. ITU-T H.274 | ISO/IEC 23002-7 shall apply, and, additionally, any SEI-message-specific constraints, variables, and semantics specified in this annex for that particular SEI message shall apply.

The value of PayloadBits, as specified in subclause ‎D.2.2, is passed to the parser of the SEI message syntax structures specified in Rec. ITU-T H.274 | ISO/IEC 23002-7.

### D.4.2 Use of the neural network post-filter characteristics SEI message and the neural-network post-filter activation SEI message

Let currPic be the cropped decoded output picture for which the neural-network post-processing filter (NNPF) defined by the neural-network post-filter characteristics (NNPFC) SEI message is activated by a neural-network post-filter activation (NNPFA) SEI message and currLayerId be the nuh_layer_id value of currPic.

It is a requirement of bitstream conformance that when a picture unit contains an NNPFA SEI message, the value of pic_output_flag in the slice headers of that picture unit shall be equal to 1.

NOTE	Since only cropped decoded output pictures are used as input pictures of the NNPF, the value of pic_output_flag in the slice headers of the coded picture corresponding to each input picture of the NNPF is equal to 1.

The variable pictureRateUpsamplingFlag is set equal to ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0.

The variable numInputPics is set equal to nnpfc_num_input_pics_minus1 + 1.

The variable numInferences is derived as follows:

—	If all of the following conditions are true, the variable numPostRoll is set equal to the value of i such that nnpfc_interpolated_pics[ i ] is greater than 0 and the variable numInferences is set equal to 1 + numPostRoll:

—	nnpfc_purpose is equal to 8 (i.e., the only purpose for the NNPF is picture rate upsampling).

—	The NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1.

—	nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i that is greater than 0.

—	Either of the following conditions is true:

—	currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId.

—	currPic is the last picture in the CLVS in output order and nnpfa_no_foll_clvs_flag is equal to 1.

—	Otherwise, if all of the following conditions are true, the variable numPostRoll is set equal to InpIdx[ I ] for the value of i such that nnpfa_output_flag[ i ] is equal to 1, and the variable numInferences is set equal to 1 + numPostRoll:

—	pictureRateUpsamplingFlag is equal to 0.

—	numInputPics is greater than 1.

—	The NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1, nnpfa_output_flag[ idx ] is equal to 1 for a single value of idx in the range of 0 to NumInpPicsInOutputTensor – 1, inclusive, and for that single value of idx, InpIdx[ idx ] is greater than 0.

—	Either of the following conditions is true:

—	currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId.

—	currPic is the last picture in the CLVS in output order and nnpfa_no_foll_clvs_flag is equal to 1.

—	Otherwise, the variable numInferences is set equal to 1.

For each value of j in the range of 0 to numInferences − 1, inclusive, the following applies:

—	The arrays inputPic[ i ] and inputPresentFlag[ i ] for i in the range of 0 to numInputPics − 1, inclusive, representing all the input pictures and the presence of input pictures, respectively, are specified as follows:

—	When j is greater than 0, for each value of k in the range of 0 to j − 1, inclusive, inputPic[ k ] is set to be currPic and inputPresentFlag[ k ] is set equal to 0.

—	The j-th input picture, inputPic[ j ], is set to be currPic and inputPresentFlag[ j ] is set equal to 1.

—	When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − 1, inclusive, in increasing order of i:

—	If both of the following conditions are true, and, inputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1.

—	Either of the following conditions is true:

—	pictureRateUpsamplingFlag is equal to 1 and currPic is associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and a particular value of fp_current_frame_is_frame0_flag, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId, precede inputPic[ i − 1 ] in output order, and are associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

—	pictureRateUpsamplingFlag is equal to 0 or currPic is not associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId and precede inputPic[ i − 1 ] in output order.

—	nnpfa_no_prev_clvs_flag is equal to 0 or the coded picture corresponding to prevPic and the current picture are present in the same CLVS.

—	Otherwise, the following applies:

—	inputPic[ i ] is set to be the same picture as inputPic[ i − 1 ] and inputPresentFlag[ i ] is set equal to 0.

—	It is a requirement of bitstream conformance that, when pictureRateUpsamplingFlag is equal to 1, nnpfc_interpolated_pics[ i − 1 ] shall be equal to 0.

—	It is a requirement of bitstream conformance that when inputPresentFlag[ i ] is equal to 0 and nnpfc_input_pic_output_flag[ i ] is equal to 1, the value of nnpfa_output_flag[ idx ] shall be equal to 0 for the value of idx such that InpIdx[ idx ] is equal to i.

—	For purposes of interpretation of the NNPFC SEI message, the following variables are specified:

—	If numInputPics is greater than 1 and there is a second NNPF that is defined by at least one NNPFC SEI message, is activated by an NNPFA SEI message for currPic, and has nnpfc_purpose equal to 4, the following applies:

—	CroppedWidth is set equal to nnpfcOutputPicWidth defined for the second NNPF.

—	CroppedHeight is set equal to nnpfcOutputPicHeight defined for the second NNPF.

—	Otherwise, the following applies:

—	CroppedWidth is set equal to the value of pic_width_in_luma_samples − ‌SubWidthC * ‌ ( conf_win_left_offset + conf_win_right_offset ) for currPic.

—	CroppedHeight is set equal to the value of pic_height_in_luma_samples − ‌SubHeightC * ‌ ( conf_win_top_offset + conf_win_bottom_offset ) for currPic.

—	The luma sample arrays CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are derived as follows for each value of i in the range of 0 to numInputPics − 1, inclusive:

—	The variable sourcePic is derived as follows:

—	If inputPresentFlag[ i ] is equal to 1 or nnpfc_absent_input_pic_zero_flag is equal to 0, sourcePic is set to be inputPic[ i ].

—	Otherwise(inputPresentFlag[ i ] is equal to 0 and nnpfc_absent_input_pic_zero_flag is equal to 1), sourcePic is set to be a picture with a luma sample array of CroppedWidth × CroppedHeight samples equal to 0 and Cb and Cr sample arrays of ( CroppedWidth / SubWidthC ) × ( CroppedHeight / SubHeightC ) samples equal to 0.

—	If numInputPics is equal to 1, the following applies:

—	The luma sample array CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are set to be the 2-dimensional arrays of decoded sample values of the Y, Cb and Cr components, respectively, of sourcePic.

—	Otherwise (numInputPics is greater than 1), the following applies:

—	The variable sourceWidth is set equal to the value of pps_pic_width_in_luma_samples − ‌SubWidthC * ( pps_conf_win_left_offset + pps_conf_win_right_offset ) for sourcePic.

—	The variable sourceHeight is set equal to the value of pps_pic_height_in_luma_samples − ‌SubHeightC * ( pps_conf_win_top_offset + pps_conf_win_bottom_offset ) for sourcePic.

—	If sourceWidth is equal to CroppedWidth and sourceHeight is equal to CroppedHeight, resampledPic is set to be the same as sourcePic.

—	Otherwise (sourceWidth is not equal to CroppedWidth or sourceHeight is not equal to CroppedHeight), the following applies:

—	There shall be an NNPF, hereafter referred to as the super resolution NNPF, that is defined by at least one NNPFC SEI message, is activated by an NNPFA SEI message for sourcePic, and has nnpfc_purpose equal to 4, nnpfcOutputPicWidth equal to CroppedWidth and nnpfcOutputPicHeight equal to CroppedHeight.

—	resampledPic is set to be the output of the neural-network inference of the super resolution NNPF with sourcePic being an input.

—	The luma sample array CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are set to be the 2-dimensional arrays of decoded sample values of the Y, Cb and Cr components, respectively, of resampledPic.

—	BitDepthY and BitDepthC are set equal to BitDepthY and BitDepthC, respectively.

—	ChromaFormatIdc is set equal to chroma_format_idc.

—	The array StrengthControlVal[ i ] for all values of i in the range of 0 to numInputPics − 1, inclusive, specifying the filtering strength control value for the input pictures for the NNPF, is derived as follows:

—	StrengthControlVal[ i ] is set equal to the value of ( firstSliceQpY + QpBdOffsetY ) ÷ ( 51 + QpBdOffsetY ), where firstSliceQpY is equal to SliceQpY of the first slice of inputPic[ i ].

There shall not be more than two NNPFC SEI messages present in a picture unit with the same value of nnpfc_id. When there are two NNPFC SEI messages present in a picture unit with the same value of nnpfc_id, these SEI messages shall have different content. When two NNPFC SEI messages with the same nnpfc_id and different content are present in the same picture unit, both of these NNPFC SEI messages shall be in the same SEI NAL unit.

### D.4.3. Use of the phase indication SEI message

For purposes of interpretation of the phase indication SEI message, the following variables are specified:

–	CroppedWidth is set equal to pic_width_in_luma_samples − SubWidthC * ( conf_win_left_offset + conf_win_right_offset ).

–	CroppedHeight is set equal to pic_height_in_luma_samples − SubHeightC * ( conf_win_top_offset + conf_win_bottom_offset ).

# Subclause E.3.1

Starting with the sentence that begins with “The application of the transfer characteristics function is denoted by ( x )′ for an argument x.”, replace the remainder of the semantics of the matrix_coeffs syntax element with the following:

The application of the transfer characteristics function is denoted by ( x )′ for an argument x.

—	If matrix_coeffs is not equal to 14 or 15, the signals E′R, E′G, and E′B are determined by application of the transfer characteristics function as follows:

E′R = ( ER )′	(E-1)

E′G = ( EG )′	(E-2)

E′B = ( EB )′	(E-3)

In this case, the range of E′R, E′G, and E′B is specified as follows:

—	If transfer_characteristics is not equal to 11 or 12, E′R, E′G, and E′B are real numbers with values in the range of 0 to 1, inclusive.

—	Otherwise, (transfer_characteristics is equal to 11 or 12), E′R, E′G, and E′B are real numbers with a larger range not specified in this document.

—	Otherwise (matrix_coeffs is equal to 14 or 15), the signals E′L, E′M, and E′S are determined by the following ordered steps:

a)	The "linear-domain" real-valued signals EL, EM, and ES are determined as follows:

—	If matrix_coeffs is equal to 14, the signals EL, EM, and ES are determined as follows:

EL = ( 1 688 * ER + 2 146 * EG + 262 * EB ) ÷ 4 096	(E-4)

EM = ( 683 * ER + 2 951 * EG + 462 * EB ) ÷ 4 096	(E-5)

ES = ( 99 * ER + 309 * EG + 3 688 * EB ) ÷ 4 096	(E-6)

—	Otherwise (matrix_coeffs is equal to 15), the signals EL, EM, and ES are determined as follows:

EL = ( 1 747 * ER + 2 169 * EG + 180 * EB ) ÷ 4 096	(E-7)

EM = ( 673 * ER + 3 029 * EG + 394 * EB ) ÷ 4 096	(E-8)

ES = ( 50 * ER + 207 * EG + 3 839 * EB ) ÷ 4 096	(E-9)

b)	The signals E′L, E′M, and E′S are determined by application of the transfer characteristics function as follows:

E′L = ( EL )′		(E-10)

E′M = ( EM )′		(E-11)

E′S = ( ES )′		(E-12)

When matrix_coeffs is equal to 0, 8, 16, or 17, the variables bitDepthRGB and maxValRGB are derived using the following ordered steps:

a)	The variable bitDepthRGB is derived as follows:

—	If matrix_coeffs is equal to 0 or 8, the following applies:

bitDepthRGB = BitDepthY	(E-13)

—	Otherwise, if matrix_coeffs is equal to 16, the following applies:

bitDepthRGB = BitDepthY – 2	(E-14)

—	Otherwise (matrix_coeffs is equal to 17), the following applies:

bitDepthRGB = BitDepthY – 1	(E-15)

b)	The variable maxValRGB is derived as follows:

maxValRGB = ( 1 << bitDepthRGB ) − 1	(E-16)

The interpretation of matrix_coeffs is specified as follows:

—	If video_full_range_flag is equal to 0, the following applies:

—	If matrix_coeffs is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, 13, 14, or 15, the following formulae apply:

Y = Clip1Y( Round( ( 1 << ( BitDepthY − 8 ) ) * ( 219 * E′Y + 16 ) ) )	(E-17)

Cb = Clip1C( Round( ( 1 << ( BitDepthC − 8 ) ) * ( 224 * E′PB + 128 ) ) )	(E-18)

Cr = Clip1C( Round( ( 1 << ( BitDepthC − 8 ) ) * ( 224 * E′PR + 128 ) ) )	(E-19)

—	Otherwise, if matrix_coeffs is equal to 0, 8, 16, or 17, the following formulae apply:

R = Clip3( 0, maxValRGB, ( 1 << ( bitDepthRGB − 8 ) ) * ( 219 * E′R + 16 ) )	(E-20)

G = Clip3( 0, maxValRGB, ( 1 << ( bitDepthRGB − 8 ) ) * ( 219 * E′G + 16 ) )	(E-21)

B = Clip1Y( 0, maxValRGB, ( 1 << ( bitDepthRGB − 8 ) ) * ( 219 * E′B + 16 ) )	(E-22)

—	Otherwise, if matrix_coeffs is equal to 2, the interpretation of the matrix_coeffs syntax element is unknown or is determined by the application.

—	Otherwise (matrix_coeffs is not equal to 0, 1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, or 17), the interpretation of the matrix_coeffs syntax element is reserved for future definition by ITU-T | ISO/IEC.

—	Otherwise (video_full_range_flag is equal to 1), the following applies:

—	If matrix_coeffs is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, 13, 14, or 15, the following applies:

Y = Clip1Y( Round( ( ( 1 << BitDepthY ) − 1 ) * E′Y ) )	(E-23)

Cb = Clip1C( Round( ( ( 1 << BitDepthC ) − 1 ) * E′PB + ( 1 << ( BitDepthC − 1 ) ) ) )	(E-24)

Cr = Clip1C( Round( ( ( 1 << BitDepthC ) − 1 ) * E′PR + ( 1 << ( BitDepthC − 1 ) ) ) )	(E-25)

—	Otherwise, if matrix_coeffs is equal to 0 8, 16, or 17, the following applies:

R = Clip3( 0, maxValRGB, maxValRGB * E′R )	(E-26)

G = Clip3( 0, maxValRGB, maxValRGB * E′G )	(E-27)

B = Clip3( 0, maxValRGB, maxValRGB * E′B )	(E-28)

—	Otherwise, if matrix_coeffs is equal to 2, the interpretation of the matrix_coeffs syntax element is unknown or is determined by the application.

—	Otherwise (matrix_coeffs is not equal to 0, 1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12 13, 14, 15, 16, or 17), the interpretation of the matrix_coeffs syntax element is reserved for future definition by ITU-T | ISO/IEC. Reserved values for matrix_coeffs shall not be present in bitstreams conforming to this version of this document. Decoders shall interpret reserved values of matrix_coeffs as equivalent to the value 2.

It is a requirement of bitstream conformance to this version of this document that when colour_primaries is not equal to 1, 4, 5, 6, 7, 8, 9, 10, 11, 12, or 22, matrix_coeffs shall not be equal to 12 or 13.

When matrix_coeffs is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, or 13, the constants KB and KR are specified as follows:

—	If matrix_coeffs is not equal to 12 or 13, the constants KB and KR are specified in Table E.5.

—	Otherwise (matrix_coeffs is equal to 12 or 13), the constants KR and KB are computed as follows, using the chromaticity coordinates (xR, yR), (xG, yG), (xB, yB), and (xW, yW) specified by Table E.3 for the colour_primaries syntax element for the red, green, blue, and white colour primaries, respectively.

KR = 	(E-29)

KB = 	(E-30)

where the values of zR, zG, zB, and zW, are given by:

zR = 1 − ( xR + yR )	(E-31)

zG = 1 − ( xG + yG )	(E-32)

zB = 1 − ( xB + yB )	(E-33)

zW = 1 − ( xW + yW )	(E-34)

The variables E′Y, E′PB, and E′PR (for matrix_coeffs not equal to 0, 8, 16, or 17) or Y, Cb, and Cr (for matrix_coeffs equal to 0, 8, 16, or 17) are specified as follows:

—	If matrix_coeffs is not equal to 0, 8, 10, 11, 13, 14, 15, 16, or 17, the following formulae apply:

E′Y = KR * E′R + ( 1 − KR − KB ) * E′G + KB * E′B	(E-35)

E′PB = 0.5 * ( E′B − E′Y ) ÷ ( 1 − KB )	(E-36)

E′PR = 0.5 * ( E′R − E′Y ) ÷ ( 1 − KR )	(E-37)

NOTE 8	E′Y is a real number with the value 0 associated with nominal black and the value 1 associated with nominal white. E′PB and E′PR are real numbers with the value 0 associated with both nominal black and nominal white. When transfer_characteristics is not equal to 11 or 12, E′Y is a real number with values in the range of 0 to 1, inclusive. When transfer_characteristics is not equal to 11 or 12, E′PB and E′PR are real numbers with values in the range of −0.5 to 0.5, inclusive. When transfer_characteristics is equal to 11 or 12, E′Y, E′PB, and E′PR are real numbers with a larger range not specified in this document.

—	Otherwise, if matrix_coeffs is equal to 0, the following formulae apply:

Y = Round( G )	(E-38)

Cb = Round( B )	(E-39)

Cr = Round( R )	(E-40)

—	Otherwise, if matrix_coeffs is equal to 8 and BitDepthC is equal to BitDepthY, the following formulae apply:

Y = Round( 0.5 * G + 0.25 * ( R + B ) )	(E-41)

Cb = Round( 0.5 * G − 0.25 * ( R + B ) ) + ( 1 << ( BitDepthC − 1 ) )	(E-42)

Cr = Round( 0.5 * (R − B ) ) + ( 1 << ( BitDepthC − 1 ) )	(E-43)

NOTE 9	In this case, for purposes of the YCgCo nomenclature used in Table E.5, Cb and Cr of Formulae (E-42) and (E-43) may be referred to as Cg and Co, respectively. An appropriate inverse conversion for Formulae (E-41) to (E-43) is as follows:

t = Y − ( Cb − ( 1 << ( BitDepthC − 1 ) ) )	(E-44)

G = Clip1Y( Y + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-45)

B = Clip1Y( t − ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-46)

R = Clip1Y( t + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-47)

—	Otherwise, if matrix_coeffs is equal to 8, 16, or 17, the following formulae apply:

Cr = Round( R ) − Round( B ) + ( 1 << ( BitDepthC − 1 ) )	(E-48)

t = Round( B ) + ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(E-49)

Cb = Round( G ) − t + ( 1 << ( BitDepthC − 1 ) )	(E-50)

Y = t + ( ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(E-51)

NOTE 10	In this case, for purposes of the YCgCo nomenclature used in Table E.5, Cb and Cr of Formulae (E-50) and (E-48) may be referred to as Cg and Co, respectively. An appropriate inverse conversion for Formulae (E-48) to (E-51) is as follows:

t = Y − ( ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(E-52)

G = Clip3( 0, maxValRGB, t + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-53)

B = Clip3( 0, maxValRGB, t − ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )	(E-54)

R = Clip3( 0, maxValRGB, B + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-55)

—	Otherwise, if matrix_coeffs is equal to 10 or 13, the signal E′Y is determined by application of the transfer characteristics function as follows, and Formulae (E-58) to (E-61) apply for specification of the signals E′PB and E′PR:

EY = KR * ER + ( 1 − KR − KB ) * EG + KB * EB	(E-56)

E′Y = ( EY )′	(E-57)

NOTE 11	In this case, EY is defined from the “linear-domain” signals for ER, EG, and EB, prior to application of the transfer characteristics function, which is then applied to produce the signal E′Y. EY and E′Y are real values with the value 0 associated with nominal black and the value 1 associated with nominal white.

while the signals E′PB and E′PR are determined as follows:

E′PB = ( E′B − E′Y ) ÷ ( 2 * NB )  for −NB <= E′B − E′Y <= 0	(E-58)

E′PB = ( E′B − E′Y ) ÷ ( 2 * PB )  for 0 < E′B − E′Y <= PB	(E-59)

E′PR = ( E′R − E′Y ) ÷ ( 2 * NR )  for −NR <= E′R − E′Y <= 0	(E-60)

E′PR = ( E′R − E′Y ) ÷ ( 2 * PR )  for 0 < E′R − E′Y <= PR	(E-61)

where the constants NB, PB, NR, and PR are determined by application of the transfer characteristics function to expressions involving the constants KB and KR as follows:

NB = ( 1 − KB )′	(E-62)

PB = 1 − ( KB )′ 	(E-63)

NR = ( 1 − KR )′	(E-64)

PR = 1 − ( KR )′	(E-65)

—	Otherwise if matrix_coeffs is equal to 11, the following formulae apply:

E′Y = E′G	(E-66)

E′PB = 0.5 * ( 0.986 566 * E′B − E′Y )	(E-67)

E′PR = 0.5 * ( E′R − 0.991 902 * E′Y )	(E-68)

NOTE 12	In this case, for purposes of the Y′D′ZD′X nomenclature used in Table E.5, E′PB may be referred to as D′Z and E′PR may be referred to as D′X.

—	Otherwise, if matrix_coeffs is equal to 14, the following formulae apply:

—	If transfer_characteristics is not equal to 18, the following formulae apply:

E′Y = 0.5 * ( E′L + E′M )	(E-69)

E′PB = ( 6 610 * E′L − 13 613 * E′M + 7 003 * E′S ) ÷ 4 096	(E-70)

E′PR = ( 17 933 * E′L − 17 390 * E′M − 543 * E′S ) ÷ 4 096	(E-71)

—	Otherwise, the following formulae apply:

E′Y = 0.5 * ( E′L + E′M ) 	(E-72)

E′PB = ( 3 625 * E′L − 7 465 * E′M + 3 840 * E′S ) ÷ 4 096	(E-73)

E′PR = ( 9 500 * E′L − 9 212 * E′M − 288 * E′S ) ÷ 4 096	(E-74)

NOTE 13	In this case, for purposes of the ICTCP nomenclature used in Table E.5, E′Y, E′PB, and E′PR of Formulae (E-69), (E-70), and (E-71) or Formulae (E-72), (E-73), and (E-74) may be referred to as I, CT, and CP, respectively. Formulae (E-69) to (E-71) were designed specifically for use with transfer_characteristics equal to 16 (PQ), and Formulae (E-72) to (E-74) were designed specifically for use with transfer_characteristics equal to 18 (HLG).

—	Otherwise (matrix_coeffs is equal to 15), the following formulae apply:

E′Y = ( 1 638 * E′L + 1 638 * E′M + 820 * E′S ) ÷ 4 096	(E-75)

E′PB = ( 18 248 * E′L − 19 870 * E′M + 1 622 * E′S ) ÷ 4 096	(E-76)

E′PR = ( 3 300 * E′L + 1 463 * E′M − 4 763 * E′S ) ÷ 4 096	(E-77)

NOTE 14	In this case, for purposes of the IPT-C2 nomenclature used in Table E.5, E′Y, E′PB, and E′PR of Formulae (E-75), (E-76), and (E-77) may be referred to as I, P, and T, respectively.

Table E.5 — Matrix coefficients interpretation using the matrix_coeffs syntax element

| Value | Matrix | Informative remark |
| --- | --- | --- |
| 0 | Identity | The identity matrix. Typically used for GBR (often referred to as RGB); however, may also be used for YZX (often referred to as XYZ) IEC 61966-2-1 sRGB SMPTE ST 428-1 (2006) See Formulae (E-38) to (E-40) |
| 1 | KR = 0.212 6; KB = 0.072 2 | ITU-R Rec. BT.709-6 ITU-R Rec. BT.1361-0 conventional colour gamut system and extended colour gamut system (historical) IEC 61966-2-4 xvYCC709 SMPTE RP 177 (1993) Annex B See Formulae (E-35) to (E-37) |
| 2 | Unspecified | Image characteristics are unknown or are determined by the application. |
| 3 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 4 | KR = 0.30; KB = 0.11 | FCC Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) See Formulae (E-35) to (E-37) |
| 5 | KR = 0.299; KB = 0.114 | ITU-R Rec. BT.470-6 System B, G (historical) ITU-R Rec. BT.601-7 625 ITU-R Rec. BT.1358-0 625 (historical) ITU-R Rec. BT.1700-0 625 PAL and 625 SECAM IEC 61966-2-1 sYCC IEC 61966-2-4 xvYCC601 (functionally the same as the value 6) See Formulae (E-35) to (E-37) |
| 6 | KR = 0.299; KB = 0.114 | ITU-R Rec. BT.601-7 525 ITU-R Rec. BT.1358-1 525 or 625 (historical) ITU-R Rec. BT.1700-0 NTSC SMPTE ST 170 (2004) (functionally the same as the value 5) See Formulae (E-35) to (E-37) |
| 7 | KR = 0.212; KB = 0.087 | SMPTE ST 240 (1999, historical) See Formulae (E-35) to (E-37) |
| 8 | YCgCo or YCgCo-R | See Formulae (E-41) to (E-47) for YCgCo (when BitDepthC is equal to BitDepthY) See Formulae (E-48) to (E-55) for YCgCo-R (when BitDepthC is equal to BitDepthY + 1) |
| 9 | KR = 0.262 7; KB = 0.059 3 | Rec. ITU-R BT.2020-2 non-constant luminance system Rec. ITU-R BT.2100-2 Y′CbCr See Formulae (E-35) to (E-37) |
| 10 | KR = 0.262 7; KB = 0.059 3 | Rec. ITU-R BT.2020-2 constant luminance system See Formulae (E-56) to (E-65) |
| 11 | Y′D′ZD′X | SMPTE ST 2085 (2015) See Formulae (E-66) to (E-68) |
| 12 | See Formulae (E-29) to (E-34) | Chromaticity-derived non-constant luminance system See Formulae (E-35) to (E-37) |
| 13 | See Formulae (E-29) to (E-34) | Chromaticity-derived constant luminance system See Formulae (E-56) to (E-65) |
| 14 | ICTCP | Rec. ITU-R BT.2100-2 ICTCP See Formulae (E-69) to (E-71) for transfer_characteristics value 16 (PQ) See Formulae (E-72) to (E-74) for transfer_characteristics value 18 (HLG) |
| 15 | IPT-C2 | Society of Motion Picture and Television Engineers ST 2128 See Formulae (E-75) to (E-77) |
| 16 | YCgCo-Re | See Formulae (E-41) to (E-55) |
| 17 | YCgCo-Ro | See Formulae (E-41) to (E-55) |
| 18..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

# Subclause F.3.29

Replace the following:

NOTE	An IRAP picture belonging to an independent layer contains only I slices. An IRAP picture belonging to a predicted layer with nuh_layer_id value currLayerId may contain P, B, and I slices, cannot use inter prediction from other pictures with nuh_layer_id equal to currLayerId, and may use inter-layer prediction from its direct reference layers. The IRAP picture belonging to a predicted layer with nuh_layer_id value currLayerId and all subsequent non-RASL pictures with nuh_layer_id equal to currLayerId in decoding order can be correctly decoded without performing the decoding process of any pictures with nuh_layer_id equal to currLayerId that precede the IRAP picture in decoding order, when the necessary parameter sets are available when they need to be activated and LayerInitializedFlag[ refLayerId ] is equal to 1 for refLayerId equal to all nuh_layer_id values of the direct reference layers of the layer with nuh_layer_id equal to currLayerId.

with the following:

NOTE	An IRAP picture belonging to an independent layer does not refer to any pictures other than itself for inter prediction in its decoding process. An IRAP picture belonging to a predicted layer with nuh_layer_id value currLayerId does not use inter prediction from other pictures than itself with nuh_layer_id equal to currLayerId, and can use inter-layer prediction from its direct reference layers. The IRAP picture belonging to a predicted layer with nuh_layer_id value currLayerId and all subsequent non-RASL pictures with nuh_layer_id equal to currLayerId in decoding order can be correctly decoded without performing the decoding process of any pictures with nuh_layer_id equal to currLayerId that precede the IRAP picture in decoding order, when the necessary parameter sets are available when they need to be activated and LayerInitializedFlag[ refLayerId ] is equal to 1 for refLayerId equal to all nuh_layer_id values of the direct reference layers of the layer with nuh_layer_id equal to currLayerId.

# Subclause F.7.4.2.2

Replace the following:

NOTE 1	When a picture picA that is a CRA picture and belongs to a layer with nuh_layer_id equal to layerId is present in a bitstream and pictures belonging to the layer with nuh_layer_id equal to layerId and preceding, in decoding order, the picture picA are dropped due to layer down-switching followed by layer up-switching, the RASL pictures associated with the picture picA, if any, may have some reference pictures that may not be available for reference unless one of the following conditions is true:

with the following:

NOTE 1	When a CRA picture craPicA in a particular layer is present in the bitstream and pictures in the same layer as craPicA and preceding, in decoding order, craPicA are dropped due to layer down-switching followed by layer up-switching, the RASL pictures associated with craPicA, if any, can have some reference pictures that are unavailable for reference unless one of the following conditions is true:

# Subclause F.7.4.3.1.1

Replace "ScalabiltyId" in "Table F.1 – Mapping of ScalabiltyId to scalability dimensions" with "ScalabilityId".

# Subclause F.11.2

Replace Table F.3 with the following:

Table F.3 — Specification of CompatibleProfileList

| Profile to which the decoder conforms | Profiles that the decoder shall support CompatibleProfileList |  |
| --- | --- | --- |
| Scalable Main | Scalable Main, Main, Main Still Picture |  |
| Scalable Main 10 | Scalable Main 10, Main, Main Still Picture, Main 10, Scalable Main |  |
| Scalable Monochrome | The compatible format range extensions profiles of the Monochrome profile, and the compatible scalable format range extensions profiles of the Scalable Monochrome profile |  |
| Scalable Monochrome 12 | The compatible format range extensions profiles of the Monochrome 12 profile, and the compatible scalable format range extensions profiles of the Scalable Monochrome 12 profile |  |
| Scalable Monochrome 16 | The compatible format range extensions profiles of the Monochrome 16 profile, and the compatible scalable format range extensions profiles of the Scalable Monochrome 16 profile |  |
| Scalable Main 4:4:4 | Scalable Main, Main, Main Still Picture, the compatible format range extensions profiles of the Main 4:4:4 profile, and the compatible scalable format range extensions profiles of the Scalable Main 4:4:4 profile |  |
| Multiview Main | Multiview Main, Main, Main Still Picture |  |
| Extended Multiview Main | Extended Multiview Main, Multiview Main, Main, Main Still Picture |  |
| Multiview Main 10 | Multiview Main 10, Main, Main Still Picture, Main 10, Multiview Main |  |
| Extended Multiview Main 10 | Extended Multiview Main 10, Multiview Main 10, Main, Main Still Picture, Main 10, Extended Multiview Main, Multiview Main |  |
| Multiview Monochrome | Multiview Monochrome, Monochrome |  |
| Multiview Monochrome 10 | Multiview Monochrome 10, Multiview Monochrome, Monochrome 10, Monochrome |  |
| Multiview Monochrome 12 | Multiview Monochrome 12, Multiview Monochrome 10, Multiview Monochrome, Monochrome 12, Monochrome 10, Monochrome |  |
| Multiview Monochrome 16 | Multiview Monochrome 16, Multiview Monochrome 12, Multiview Monochrome 10, Multiview Monochrome, Monochrome 16, Monochrome 12, Monochrome 10, Monochrome |
| 3D Main | 3D Main, Multiview Main, Main, Main Still Picture |  |

# Subclause F.14.2.5

Change the coding of bsp_nesting_zero_bit from u(1) to f(1).

# Subclause F.14.3.1

Make the following changes:

The general SEI payload semantics specified in subclause D.3.1 apply with the following modifications and additions:

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 161, 165, 167, 168, 200 to 202, inclusive, and 205, and 210 to 212, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 168, inclusive, 200 to 202, inclusive, and 205, and 210 to 212, inclusive.

…

# Subclause G.3

Add the following definitions:

### auxiliary layer: A layer with a nuh_layer_id value equal to i, such that DepthLayerFlag[ i ] and DependencyId[ i ] are both equal to 0 and AuxId[ i ] is greater than 0.

### primary layer: A layer with a nuh_layer_id value equal to i, such that DepthLayerFlag[ i ], DependencyId[ i ], and AuxId[ i ] are all equal to 0.

# Subclause G.11.1.1

Make the following changes:

# G.11.1.1 Multiview Main, Extended Multiview Main, Multiview Main 10, and Extended Multiview Main 10 profiles

For a layer in an output operation point associated with an OLS in a bitstream, the layer conforming to the Multiview Main, Extended Multiview Main, Multiview Main 10, or Extended Multiview Main 10 profile, the following applies:

—	Let olsIdx be the OLS index of the OLS, the sub-bitstream subBitstream and the base layer sub-bitstream baseBitstream are derived as specified in F.11.3.

When vps_base_layer_internal_flag is equal to 1, the base layer sub-bitstream baseBitstream shall obey the following constraints:

—	When the layer conforms to the Multiview Main or Extended Multiview Main profile, the base layer sub-bitstream baseBitstream shall be indicated to conform to the Main profile.

—	When the layer conforms to the Multiview Main 10 or Extended Multiview Main 10 profile, the base layer sub-bitstream baseBitstream shall be indicated to conform to the Main 10 or the Main profile.

The sub-bitstream subBitstream shall obey the following constraints:

—	All active VPSs shall have vps_num_rep_formats_minus1 in the range of 0 to 15, inclusive.

—	When the layer conforms to the Multiview Main or Multiview Main 10 profile, all active SPSs for layers in the sub-bitstream subBitstream shall have chroma_format_idc equal to 1 only.

—	When the layer conforms to the Extended Multiview Main or Extended Multiview Main 10 profile, all active SPSs for a primary layer in the sub-bitstream subBitstream shall have chroma_format_idc equal to 1 only, and all active SPSs for an auxiliary layer in the sub-bitstream subBitstream shall have chroma_format_idc equal to 0 or 1.

—	When the layer conforms to the Extended Multiview Main, Multiview Main 10, or Extended Multiview Main 10 profile, for a layer layerA with nuh_layer_id equal to i in the sub-bitstream subBitstream and another layer layerB with nuh_layer_id equal to j, when layerB is a reference layer of layerA, the value of chroma_format_idc for layerA and the value of chroma_format_idc for layerB shall be identical, and AuxId[ i ] shall be equal to AuxId[ j ].

—	All active SPSs for layers in the sub-bitstream subBitstream shall have transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, implicit_rdpcm_enabled_flag, explicit_rdpcm_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, high_precision_offsets_enabled_flag, persistent_rice_adaptation_enabled_flag, and cabac_bypass_alignment_enabled_flag, when present, equal to 0 only.

—	CtbLog2SizeY derived from all active SPSs for layers in the sub-bitstream subBitstream shall be in the range of 4 to 6, inclusive.

—	All active PPSs for layers in the sub-bitstream subBitstream shall have log2_max_transform_skip_block_size_minus2 and chroma_qp_offset_list_enabled_flag, when present, equal to 0 only.

—	ScalabilityId[ j ][ smIdx ] derived according to any active VPS shall be equal to 0 for any smIdx value not equal to 1 or 3 and for any value of j such that layer_id_in_nuh[ j ] is among layerIdListTarget that was used to derive the sub-bitstream subBitstream.

—	When NumLayersInIdList[ OlsIdxToLsIdx[ olsIdx ] ] is equal to 2, output_layer_flag[ olsIdx ][ j ] derived according to any active VPS shall be equal to 1 for j in the range of 0 to 1, inclusive, for the sub-bitstream subBitstream.

—	All active VPSs shall have alt_output_layer_flag[ olsIdx ] equal to 0 only.

—	When ViewOrderIdx[ i ] derived according to any active VPS is equal to 1 for the layer with nuh_layer_id equal to i in the sub-bitstream subBitstream, inter_view_mv_vert_constraint_flag shall be equal to 1 in the sps_multilayer_extension( ) syntax structure in each active SPS for that layer.

—	When ViewOrderIdx[ i ] derived according to any active VPS is greater than 0 for the layer with nuh_layer_id equal to i in the sub-bitstream subBitstream, num_ref_loc_offsets shall be equal to 0 in each active PPS for that layer.

—	When ViewOrderIdx[ i ] derived according to any active VPS is greater than 0 for the layer with nuh_layer_id equal to i in the sub-bitstream subBitstream, the values of pic_width_in_luma_samples and pic_height_in_luma_samples in each active SPS for that layer shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, in each active SPS for all reference layers of that layer.

—	For a layer with nuh_layer_id iNuhLId equal to any value included in layerIdListTarget that was used to derive the sub-bitstream subBitstream, the value of NumRefLayers[ iNuhLId ], which specifies the total number of direct and indirect reference layers and is derived as specified in F.7.4.3.1, shall be less than or equal to 4.

—	All active SPSs for layers in the sub-bitstream subBitstream shall have sps_range_extension_flag and sps_scc_extension_flag equal to 0 only.

—	All active PPSs for layers in the sub-bitstream subBitstream shall have pps_range_extension_flag and pps_scc_extension_flag equal to 0 only.

—	When an active PPS for any layer in the sub-bitstream subBitstream has tiles_enabled_flag equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

—	When an active PPS for any layer in the sub-bitstream subBitstream has tiles_enabled_flag equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

—	The number of times read_bits( 1 ) is called in 9.3.4.3.3 and 9.3.4.3.4 when parsing coding_tree_unit( ) data for any CTU shall be less than or equal to 5 * RawCtuBits / 3.

—	For any active VPS, ViewOrderIdx[ i ] shall be greater than ViewOrderIdx[ j ] for any values of i and j among layerIdListTarget that was used to derive the sub-bitstream subBitstream such that AuxId[ i ] is equal to AuxId[ j ] and i is greater than j.

—	All active PPSs for layers in the sub-bitstream subBitstream shall have colour_mapping_enabled_flag equal to 0 only.

When the layer conforms to the Multiview Main or Extended Multiview Main profile, the sub-bitstream subBitstream shall obey the following constraints:

—	All active SPSs for layers in the sub-bitstream subBitstream shall have bit_depth_luma_minus8 equal to 0 only.

—	All active SPSs for layers in the sub-bitstream subBitstream shall have bit_depth_chroma_minus8 equal to 0 only.

—	When the layer conforms to the Multiview Main profile, the tier and level constraints specified for the Multiview Main profile in G.11.2 shall be fulfilled.

—	When the layer conforms to the Extended Multiview Main profile, the tier and level constraints specified for the Extended Multiview Main profile in G.11.2 shall be fulfilled.

When the layer conforms to the Multiview Main 10 or Extended Multiview Main profile, the sub-bitstream subBitstream shall obey the following constraints:

—	All active SPSs for a primary layer in the sub-bitstream subBitstream shall have bit_depth_luma_minus8 in the range of 0 to 2, inclusive.

—	All active SPSs for a primary layer in the sub-bitstream subBitstream shall have bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

—	For a layer layerA with nuh_layer_id equal to i in the sub-bitstream subBitstream and another layer layerB with nuh_layer_id equal to j, when layerB is a reference layer of layerA, the value of bit_depth_luma_minus8 for layerA and the value of bit_depth_luma_minus8 for layerB shall be identical, and the value of bit_depth_chroma_minus8 for layerA and the value of bit_depth_chroma_minus8 for layerB shall be identical.

—	When the layer conforms to the Multiview Main profile 10, the tier and level constraints specified for the Multiview Main 10 profile in G.11.2 shall be fulfilled.

—	When the layer conforms to the Extended Multiview Main profile 10, the tier and level constraints specified for the Extended Multiview Main 10 profile in G.11.2 shall be fulfilled.

In the remainder of this subclause and G.11.2.1, all syntax elements in the profile_tier_level( ) syntax structure refer to those in the profile_tier_level( ) syntax structure associated with the layer.

Conformance of a layer in an output operation point associated with an OLS in a bitstream to the Multiview Main profile is indicated as follows:

—	If OpTid of the output operation point is equal to vps_max_sub_layer_minus1, the conformance is indicated by having both of the following conditions satisfied:

—	general_profile_idc being equal to 6 or general_profile_compatibility_flag[ 6 ] being equal to 1,

—	general_max_12bit_constraint_flag being equal to 1, general_max_10bit_constraint_flag being equal to 1, general_max_8bit_constraint_flag being equal to 1, general_max_422chroma_constraint_flag being equal to 1, general_max_420chroma_constraint_flag being equal to 1, general_max_monochrome_constraint_flag being equal to 0, general_intra_constraint_flag being equal to 0, general_one_picture_only_constraint_flag being equal to 0, and general_lower_bit_rate_constraint_flag being equal to 1

—	Otherwise (OpTid of the output operation point is less than vps_max_sub_layer_minus1), the conformance is indicated by having both of the following conditions satisfied:

—	sub_layer_profile_idc[ OpTid ] being equal to 6 or sub_layer_profile_compatibility_flag[ OpTid ][ 6 ] being equal to 1

—	sub_layer_max_12bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_10bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_8bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_422chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_420chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_monochrome_constraint_flag[ OpTid ] being equal to 0, sub_layer_intra_constraint_flag[ OpTid ] being equal to 0, sub_layer_one_picture_only_constraint_flag[ OpTid ] being equal to 0, and sub_layer_lower_bit_rate_constraint_flag[ OpTid ] being equal to 1

Conformance of a bitstream to the Extended Multiview Main profile is indicated by general_profile_idc being equal to 12 or general_profile_compatibility_flag[ 12 ] being equal to 1. Conformance of a sub-layer representation with TemporalId equal to i to the Extended Multiview Main profile is indicated by sub_layer_profile_idc[ i ] being equal to 12 or sub_layer_profile_compatibility_flag[ i ][ 12 ] being equal to 1.

Conformance of a layer in an output operation point associated with an OLS in a bitstream to the Multiview Main 10 profile is indicated as follows:

—	If OpTid of the output operation point is equal to vps_max_sub_layer_minus1, the conformance is indicated by having both of the following conditions satisfied:

—	general_profile_idc being equal to 6 or general_profile_compatibility_flag[ 6 ] being equal to 1,

—	general_max_12bit_constraint_flag being equal to 1, general_max_10bit_constraint_flag being equal to 1, general_max_8bit_constraint_flag being equal to 0, general_max_422chroma_constraint_flag being equal to 1, general_max_420chroma_constraint_flag being equal to 1, general_max_monochrome_constraint_flag being equal to 0, general_intra_constraint_flag being equal to 0, general_one_picture_only_constraint_flag being equal to 0, and general_lower_bit_rate_constraint_flag being equal to 1.

—	Otherwise (OpTid of the output operation point is less than vps_max_sub_layer_minus1), the conformance is indicated by having both of the following conditions satisfied:

—	sub_layer_profile_idc[ OpTid ] being equal to 6 or sub_layer_profile_compatibility_flag[ OpTid ][ 6 ] being equal to 1,

—	sub_layer_max_12bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_10bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_8bit_constraint_flag[ OpTid ] being equal to 0, sub_layer_max_422chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_420chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_monochrome_constraint_flag[ OpTid ] being equal to 0, sub_layer_intra_constraint_flag[ OpTid ] being equal to 0, sub_layer_one_picture_only_constraint_flag[ OpTid ] being equal to 0, and sub_layer_lower_bit_rate_constraint_flag[ OpTid ] being equal to 1.

Conformance of a bitstream to the Extended Multiview Main 10 profile is indicated by general_profile_idc being equal to 13 or general_profile_compatibility_flag[ 13 ] being equal to 1. Conformance of a sub-layer representation with TemporalId equal to i to the Extended Multiview Main 10 profile is indicated by sub_layer_profile_idc[ i ] being equal to 13 or sub_layer_profile_compatibility_flag[ i ][ 13 ] being equal to 1.

# New subclause G.11.1.2

Add a new subclause G.11.1.2 as follows:

# G.11.1.2 Multiview format range extensions profiles

The following profiles, collectively referred to as the multiview format range extensions profiles, are specified in this subclause:

—	the Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, and Multiview Monochrome 16 profiles.

For a layer in an output operation point associated with an OLS in a bitstream, the layer conforming to the Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, or Multiview Monochrome 16 profile, the following applies:

—	Let olsIdx be the OLS index of the OLS, the sub-bitstream subBitstream is derived as specified in F.11.3.

The sub-bitstream subBitstream shall obey the following constraints:

—	All active VPSs shall have vps_num_rep_formats_minus1 in the range of 0 to 15, inclusive.

—	All active SPSs for a layer conforming to the Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12 or Multiview Monochrome 16 profile in the sub-bitstream subBitstream shall have chroma_format_idc equal to 0 only.

—	All active SPSs for layers in the sub-bitstream subBitstream shall have separate_colour_plane_flag, cabac_bypass_alignment_enabled_flag, when present, equal to 0 only.

—	CtbLog2SizeY derived from all active SPSs for layers in the sub-bitstream subBitstream shall be in the range of 4 to 6, inclusive.

—	ScalabilityId[ j ][ smIdx ] derived according to any active VPS shall be equal to 0 for any smIdx value not equal to 1 or 3 and for any value of j such that layer_id_in_nuh[ j ] is among layerIdListTarget that was used to derive the sub-bitstream subBitstream.

—	The constraints specified in Table H.3, in which entries marked with “–“ indicate that the table entry does not impose a profile-specific constraint on the corresponding syntax element, shall apply for all active SPSs and PPSs for layers in the sub-bitstream subBitstream.

NOTE	For some syntax elements with table entries marked with “–“, a constraint may be imposed indirectly – e.g. by semantics constraints that are imposed elsewhere in this document when other specified constraints are fulfilled.

—	For a layer layerA with nuh_layer_id equal to i in the sub-bitstream subBitstream and another layer layerB with nuh_layer_id equal to j, when layerB is a reference layer of layerA, the value of chroma_format_idc for layerA and the value of chroma_format_idc for layerB shall be identical, the value of bit_depth_luma_minus8 for layerA and the value of bit_depth_luma_minus8 for layerB shall be identical, the value of bit_depth_chroma_minus8 for layerA and the value of bit_depth_chroma_minus8 for layerB shall be identical, and AuxId[ i ] shall be equal to AuxId[ j ].

—	For any layer in the sub-bitstream subBitstream, the values of pic_width_in_luma_samples and pic_height_in_luma_samples in each active SPS for that layer shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, in each active SPS for all reference layers of that layer.

—	When there are two different layers, a primary layer and an auxiliary layer, with nuh_layer_id equal to i and j in the sub-bitstream subBitstream, and ViewOrderIdx[ i ] is equal to ViewOrderIdx[ j ], the value of pic_width_in_luma_samples and pic_height_in_luma_samples in each active SPS for the auxiliary layer shall be less than or equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, in each active SPS for the primary layer.

—	All active SPSs for layers in the sub-bitstream subBitstream shall have the same value of chroma_format_idc.

—	All active SPSs for layers in the sub-bitstream subBitstream shall have sps_scc_extension_flag equal to 0 only.

—	All active PPSs for layers in the sub-bitstream subBitstream shall have pps_scc_extension_flag equal to 0 only.

—	When an active PPS for any layer in the sub-bitstream subBitstream has tiles_enabled_flag equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

—	When an active PPS for any layer in subBitstream has tiles_enabled_flag equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

—	The number of times read_bits( 1 ) is called in 9.3.4.3.3 and 9.3.4.3.4 when parsing coding_tree_unit( ) data for any CTU shall be less than or equal to 5 * RawCtuBits / 3.

—	For any active VPS, DependencyId[ i ] shall be greater than DependencyId[ j ] for any values of i and j among layerIdListTarget that was used to derive the sub-bitstream subBitstream such that AuxId[ i ] is equal to AuxId[ j ] and i is greater than j.

—	The tier and level constraints specified for the Multiview Monochrome, Multiview Monochrome 10, and Multiview Monochrome 12 profiles in G.11.2, as applicable, shall be fulfilled.

Table G.3 — Allowed values for syntax elements in the multiview format range extensions profiles

| Profile for which constraint is specified | chroma_format_idc | bit_depth_luma_minus8 and bit_depth_chroma_minus8 | transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, implicit_rdpcm_enabled_flag, explicit_rdpcm_enabled_flag, intra_smoothing_disabled_flag, persistent_rice_adaptation_enabled_flag, and log2_max_transform_skip_block_size_minus2 | extended_precision_processing_flag | chroma_qp_offset_list_enabled_flag |
| --- | --- | --- | --- | --- | --- |
| Multiview Monochrome | 0 | 0 | 0 | 0 | 0 |
| Multiview Monochrome 10 | 0 | 0..2 | 0 | 0 | 0 |
| Multiview Monochrome 12 | 0 | 0..4 | 0 | 0 | 0 |
| Multiview Monochrome 16 | 0 | 0..8 | 0 | 0 | 0 |

In the remainder of this subclause and H.11.2.1, all syntax elements in the profile_tier_level( ) syntax structure refer to those in the profile_tier_level( ) syntax structure associated with the layer.

Conformance of a layer in an output operation point associated with an OLS in a bitstream for the multiview format range extensions profiles is indicated as follows:

—	If OpTid of the output operation point is equal to vps_max_sub_layer_minus1, the conformance is indicated by general_profile_idc being equal to 10 or general_profile_compatibility_flag[ 10 ] being equal to 1, with the additional indications specified in Table H.4 for the general constraint flags.

—	Otherwise (OpTid of the output operation point is less than vps_max_sub_layer_minus1), the conformance is indicated by sub_layer_profile_idc[ OpTid ] being equal to 10 or sub_layer_profile_compatibility_flag[ OpTid ][ 10 ] being equal to 1, with the additional indications specified in Table H.4 for the flags associated with the index OpTid.

All other combinations of general_max_14bit_constraint_flag, general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_8bit_constraint_flag, general_max_422chroma_constraint_flag, general_max_420chroma_constraint_flag, general_max_monochrome_constraint_flag, general_intra_constraint_flag, general_one_picture_only_constraint_flag, and general_lower_bit_rate_constraint_flag with general_profile_idc equal to 10 or general_profile_compatibility_flag[ 10 ] equal to 1 are reserved for future use by ITU-T | ISO/IEC. All other combinations of sub_layer_max_14bit_constraint_flag[ OpTid ], sub_layer_max_12bit_constraint_flag[ OpTid ], sub_layer_max_10bit_constraint_flag[ OpTid ], sub_layer_max_8bit_constraint_flag[ OpTid ], sub_layer_max_422chroma_constraint_flag[ OpTid ], sub_layer_max_420chroma_constraint_flag[ OpTid ], sub_layer_max_monochrome_constraint_flag[ OpTid ], sub_layer_intra_constraint_flag[ OpTid ], sub_layer_one_picture_only_constraint_flag[ OpTid ], and sub_layer_lower_bit_rate_constraint_flag[ OpTid ] with sub_layer_profile_idc[ OpTid ] equal to 10 or sub_layer_profile_compatibility_flag[ OpTid ][ 10 ] equal to 1 are reserved for future use by ITU-T | ISO/IEC. Such combinations shall not be present in bitstreams conforming to this document. However, decoders conforming to the multiview format range extensions profiles shall allow other combinations as specified below in this clause to occur in the bitstream.

Table G.4 — Bitstream indications for conformance to multiview range extensions profiles

| Profile for which the bitstream indicates conformance | general_max_14bit_constraint_flag or sub_layer_max_14bit_constraint_‌flag[ OpTid ] | general_max_12bit_constraint_flag or sub_layer_max_12bit_constraint_‌flag[ OpTid ] | general_max_10bit_constraint_flag or sub_layer_max_10bit_constraint_‌flag[ OpTid ] | general_max_8bit_constraint_flag or sub_layer_max_8bit_constraint_‌flag[ OpTid ] | general_max_422chroma_constraint_flag or sub_layer_max_422chroma_constraint_‌flag[ OpTid ] | general_max_420chroma_constraint_flag or sub_layer_max_420chroma_constraint_‌flag[ OpTid ] | general_max_monochrome_constraint_flag or sub_layer_max_monochrome_constraint_‌flag[ OpTid ] | general_intra_constraint_flag or sub_layer_intra_constraint_‌flag[ OpTid ] | general_one_picture_only_constraint_flag or sub_layer_one_picture_only_constraint_‌flag[ OpTid ] | general_lower_bit_rate_constraint_flag or sub_layer_lower_bit_rate_constraint_‌flag[ OpTid ] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Multiview Monochrome | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| Multiview Monochrome 10 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| Multiview Monochrome 12 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| Multiview Monochrome 16 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |

# Subclause G.11.2.1

Make the following changes:

# G.11.2.1 General tier and level limits

For purposes of comparison of tier capabilities, the tier with general_tier_flag or sub_layer_tier_flag[ i ] equal to 0 is considered to be a lower tier than the tier with general_tier_flag or sub_layer_tier_flag[ i ] equal to 1.

For purposes of comparison of level capabilities, a particular level of a specific tier is considered to be a lower level than some other level of the same tier when the value of the general_level_idc or sub_layer_level_idc[ i ] of the particular level is less than that of the other level.

The following is specified for expressing the constraints in this subclause and H.11.2.2:

—	For the Multiview Main and Extended Multiview Main profiles, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Main profile.

—	For the Multiview Main 10 and Extended Multiview Main 10 profiles, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Main 10 profile.

—	For the Multiview Monochrome profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Monochrome profile.

—	For the Multiview Monochrome 10 profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Monochrome 10 profile.

—	For the Multiview Monochrome 12 profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Monochrome 12 profile.

—	For the Multiview Monochrome 16 profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Monochrome 16 profile.

—	Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0 (i.e. the 0-th access unit).

—	Let the variable fR be set as follows:

—	If the bitstream or sub-layer representation is indicated to conform to the Main tier or is indicated to conform to a level that is lower than level 7.0, fR is set equal to 1 ÷ 300.

—	Otherwise, fR is set equal to 1  960.

—	Let the variable olsIdx be the index of the OLS.

—	For each layer with nuh_layer_id equal to currLayerId, let the variable layerSizeInSamplesY be derived as follows:

layerSizeInSamplesY = pic_width_vps_in_luma_samples * pic_height_vps_in_luma_samples	(G-2)

where pic_width_vps_in_luma_samples and pic_height_vps_in_luma_samples are found in the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS.

When the specified level is not level 8.5, each layer with nuh_layer_id equal to currLayerId conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in F.13, where “access unit” is used to denote the picture unit in the layer and the CPB is understood to be the BPB:

a)	The value of layerSizeInSamplesY shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.8 for the tier and level of the layer.

b)	The value of pic_width_vps_in_luma_samples of the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

c)	The value of pic_height_vps_in_luma_samples of the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

d)	The value of max_vps_dec_pic_buffering_minus1[ olsIdx ][ LayerIdxInVps[ currLayerId ] ][ HighestTid ] shall be less than or equal to MaxDpbSize as derived by Formula (A-2), with PicSizeInSamplesY being replaced with layerSizeInSamplesY, for the tier and level of the layer.

e)	For level 5 and higher levels, the value of CtbSizeY for the layer shall be equal to 32 or 64.

f)	The value of NumPicTotalCurr for each picture in the layer shall be less than or equal to 8.

g)	When decoding each coded picture in the layer, the value of num_tile_columns_minus1 shall be less than MaxTileCols and num_tile_rows_minus1 shall be less than MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.8 for the tier and level of the layer.

h)	For the VCL HRD parameters of the layer, CpbSize[ i ] shall be less than or equal to CpbVclFactor * MaxCPB for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where CpbSize[ i ] is specified in F.13.1 and MaxCPB is specified in Table A.8 for the tier and level of the layer in units of CpbVclFactor bits.

i)	For the NAL HRD parameters of the layer, CpbSize[ i ] shall be less than or equal to CpbNalFactor * MaxCPB for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where CpbSize[ i ] is specified in F.13.1 and MaxCPB is specified in Table A.8 for the tier and level of the layer in units of CpbNalFactor bits.

j)	For each coded picture, the value of BinCountsInNalUnits shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY ) ÷ 32.

Table A.8 specifies the limits for each level of each tier for levels other than level 8.5.

NOTE	Since there are no limits specified by Table A.8 for level 8.5, it is not possible in general for a practical decoder to be assured of being able to decode all bitstreams that conform to this level. The purpose of the definition of level 8.5 is to provide a suitable label for bitstreams that can exceed the limits of all other specified levels. When the bitstream is indicated to conform to level 8.5, a decoder is expected to examine the characteristics of the bitstream during its operation in order to determine whether it is capable of decoding the bitstream.

A tier and level to which a layer in an output operation point associated with an OLS in a bitstream conforms are indicated by the syntax elements general_tier_flag and general_level_idc if OpTid of the output layer set is equal to vps_max_sub_layer_minus1, and by the syntax elements sub_layer_tier_flag[ OpTid ] and sub_layer_level_idc[ OpTid ] otherwise, as follows:

—	If the specified level is not level 8.5, general_tier_flag or sub_layer_tier_flag[ OpTid ] equal to 0 indicates conformance to the Main tier, and general_tier_flag or sub_layer_tier_flag[ OpTid ] equal to 1 indicates conformance to the High tier, according to the tier constraints specified in Table A.8, and general_tier_flag and sub_layer_tier_flag[ OpTid ] shall be equal to 0 for levels below level 4 (corresponding to the entries in Table A.8 marked with "-"). Otherwise (the specified level is level 8.5), it is a requirement of bitstream conformance that general_tier_flag and sub_layer_tier_flag[ OpTid ] shall be equal to 1 and the value 0 for general_tier_flag and sub_layer_tier_flag[ OpTid ] is reserved for future use by ITU-T | ISO/IEC, and decoders shall ignore the value of general_tier_flag and sub_layer_tier_flag[ OpTid ].

—	general_level_idc and sub_layer_level_idc[ OpTid ] shall be set equal to a value of 30 times the level number specified in Table A.8.

# Subclause G.11.2.2

Make the following changes:

G.11.2.2 Profile-specific tier and level limits for the Multiview Main, Extended Multiview Main, Multiview Main 10, Extended Multiview Main 10, and multiview format range extensions profiles

The following is specified for expressing the constraints in this subclause:

—	The variable HbrFactor is set equal to 1.

—	The variable BrVclFactor is set equal to CpbVclFactor * HbrFactor.

—	The variable BrNalFactor is set equal to CpbNalFactor * HbrFactor.

—	The variable MinCr is set equal to MinCrBase * MinCrScaleFactor ÷ HbrFactor, where MinCrBase is specified in Table A.9.

When the specified level is not level 8.5, each layer conforming to the Multiview Main, Extended Multiview Main, Multiview Main 10, or Extended Multiview Main 10 profile, or a multiview format range extensions profile at a specified tier and level shall obey the following constraints for each conformance test as specified in F.13, where "access unit" is used to denote the picture unit in the layer, and the CPB is understood to be the BPB:

a)	The nominal removal time of access unit n (with n greater than 0) from the CPB, as specified in F.13.2.3, shall satisfy the constraint that AuNominalRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] is greater than or equal to Max( layerSizeInSamplesY ÷ MaxLumaSr, fR ), where layerSizeInSamplesY is the value of layerSizeInSamplesY for access unit n − 1 and MaxLumaSr is the value specified in Table A.9 that applies to access unit n − 1 for the tier and level of the layer.

b)	The difference between consecutive output times of pictures in different access units, as specified in F.13.3.3, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( layerSizeInSamplesY ÷ MaxLumaSr, fR ), where layerSizeInSamplesY is the value of layerSizeInSamplesY of access unit n and MaxLumaSr is the value specified in Table A.9 for access unit n for the tier and level of the layer, provided that access unit n is an access unit that has a picture that is output and is not the last of such access units.

c)	The removal time of access unit 0 shall satisfy the constraint that the number of coded slice segments in access unit 0 is less than or equal to Min( Max( 1, MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxSliceSegmentsPerPicture * layerSizeInSamplesY / MaxLumaPs ), MaxSliceSegmentsPerPicture ), for the value of layerSizeInSamplesY of access unit 0, where MaxSliceSegmentsPerPicture, MaxLumaPs, and MaxLumaSr are the values specified in Table A.8 and Table A.9 for the tier and level of the layer.

d)	The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of slice segments in access unit n is less than or equal to Min( ( Max( 1, MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxSliceSegmentsPerPicture ), where MaxSliceSegmentsPerPicture, MaxLumaPs, and MaxLumaSr are the values specified in Table A.8 and Table A.9 that apply to access unit n for the tier and level of the layer.

e)	For the VCL HRD parameters for the layer, BitRate[ i ] shall be less than or equal to BrVclFactor * MaxBR for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where BitRate[ i ] is specified in F.13.1 and MaxBR is specified in Table A.9 in units of BrVclFactor bits/s for the tier and level of the layer.

f)	For the NAL HRD parameters for the layer, BitRate[ i ] shall be less than or equal to BrNalFactor * MaxBR for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where BitRate[ i ] is specified in F.13.1 and MaxBR is specified in Table A.9 in units of BrNalFactor bits/s for the tier and level of the layer.

g)	The sum of the NumBytesInNalUnit variables for access unit 0 shall be less than or equal to FormatCapabilityFactor * ( Max( layerSizeInSamplesY, fR * MaxLumaSr ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) ) ÷ MinCr for the value of layerSizeInSamplesY of access unit 0, where MaxLumaSr is specified in Table A.9, and both MaxLumaSr and FormatCapabilityFactor are the values that apply to access unit 0 for the tier and level of the layer.

h)	The sum of the NumBytesInNalUnit variables for access unit n (with n greater than 0) shall be less than or equal to FormatCapabilityFactor * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ÷ MinCr, where MaxLumaSr is specified in Table A.9, and both MaxLumaSr and FormatCapabilityFactor are the values that apply to access unit n for the tier and level of the layer.

i)	The removal time of access unit 0 shall satisfy the constraint that the number of tiles in coded pictures in access unit 0 is less than or equal to Min( Max( 1, MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxTileCols * MaxTileRows * PicSizeInSamplesY / MaxLumaPs ), MaxTileCols * MaxTileRows ), for the value of layerSizeInSamplesY of access unit 0, where MaxTileCols and MaxTileRows are the values specified in Table A.8 that apply to access unit 0 for the tier and level of the layer.

j)	The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of tiles in coded pictures in access unit n is less than or equal to Min( Max( 1, MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxTileCols * MaxTileRows ), where MaxTileCols and MaxTileRows are the values specified in Table A.8 that apply to access unit n for the tier and level of the layer.

# Subclause G.14.3.1

Make the following changes:

The specifications in clause F.14.3.1 apply with the following modifications and additions:

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 161, 165, 167, 168, 177, 178, 179, 200 to 202, inclusive, and 205, and 210 to 212, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 168, inclusive, 176 to 180, inclusive, 200 to 202, inclusive, and 205, and 210 to 212, inclusive.

…

# Subclause H.11.1.1

Replace the following:

For a layer in an output operation point associated with an OLS in a bitstream, the layer being conforming to the Scalable Main or Scalable Main 10 profile, the following applies:

with the following:

For a layer in an output operation point associated with an OLS in a bitstream, the layer conforming to the Scalable Main or Scalable Main 10 profile, the following applies:

# Subclause H.11.1.2

Replace the following:

For a layer in an output operation point associated with an OLS in a bitstream, the layer being conforming to the Scalable Monochrome, Scalable Monochrome 12, Scalable Monochrome 16 or Scalable Main 4:4:4 profile, the following applies:

with the following:

For a layer in an output operation point associated with an OLS in a bitstream, the layer conforming to the Scalable Monochrome, Scalable Monochrome 12, Scalable Monochrome 16 or Scalable Main 4:4:4 profile, the following applies:

# Subclause H.11.2.2

Make the following change:

Min( ( Max( 1, MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxSliceSegmentsPerPicture )

# Subclause I.11.1.1

Replace the following:

For a layer in an output operation point associated with an OLS in a bitstream, the layer being conforming to the 3D Main profile, the following applies:

with the following:

For a layer in an output operation point associated with an OLS in a bitstream, the layer conforming to the 3D Main profile, the following applies:

# Subclause I.11.1.1

Replace the following:

—	When ViewOrderIdx[ i ] or DepthLayerFlag[ i ] derived according to any active VPS is greater than to 0 for the layer with nuh_layer_id equal to i in subBitstream, num_ref_loc_offsets shall be equal to 0 in each active PPS for that layer.

with the following:

—	When ViewOrderIdx[ i ] or DepthLayerFlag[ i ] derived according to any active VPS is greater than 0 for the layer with nuh_layer_id equal to i in subBitstream, num_ref_loc_offsets shall be equal to 0 in each active PPS for that layer.

# Subclause I.14.3.1

Make the following changes:

The specifications in clause G.14.3.1 apply with the following modifications and additions:

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 161, 165, 167, 168, 177, 178, 179, 200 to 202, inclusive, and 205, and 210 to 212, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 168, inclusive, 176 to 181, inclusive, 200 to 202, inclusive, and 205, and 210 to 212, inclusive.

…

# Subclause I.14.3.3

Replace the following:

When the CVS contains an alternative depth information SEI message with depth_type equal to 0, and the first access unit in the CVS is an IRAP access unit, there shall be an alternative depth information SEI message with depth_type equal to 0 in the first access unit of the CVS.

with the following:

When the CVS contains an alternative depth information SEI message with depth_type equal to 0, and the first access unit in the CVS is an IRAP access unit, there shall be an alternative depth information SEI message with depth_type equal to 0 in the first access unit of the CVS in decoding order.

# Bibliography

Add the following after entry [41] and renumber the remaining entries:

[42] SMPTE ST 2128, Colorimetry of IPT-PQ-C2 Color Space for High Dynamic Range and Wide Color Gamut Images