| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0046-v2 |
| --- | --- |

| Title: | On MV-HEVC profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang, Hongbin Liu, Li Zhang, Shaohui Jiao, Chao Hu, Jianyong Cui Bytedance Inc. Alexis M. Tourapis, Dimitri Podborski, Seethal Paluri Apple Inc. | Email: | {yekui.wang, liuhongbin.01, lizhang.idm, jiaoshaohui, huchao.jason, cuijianyong} @bytedance.com {atourapis, dpodborski, s_paluri} @apple.com |
| Source: | Bytedance Inc., Apple Inc. |  |  |

_____________________________

# Abstract

It is asserted that the existing Multiview Main profile and the being-developed Multiview Main 10 profile do not allow the combination of auxiliary (alpha or depth) layers with chroma_format_idc equal to 0 and texture layers in an output layer set (OLS). Consequently, for use of a combination of texture layers and monochrome auxiliary (alpha or depth) layers, an encoder conforming to these profiles has to encode the auxiliary (alpha or depth) layers as non-monochome, using chroma_format_idc equal to 1. This may not be as efficient or desirable for some applications. In particular, the alpha or depth layers may have already been captured and encoded as monochrome signals, and this would require converting and reencoding them for placement within a Multiview Main or Main 10 bitstream.

To resolve this and a few other observed issues, this contribution proposes the following changes to the latest draft text for new profiles, colour descriptors, and SEI messages for HEVC in JVET-AF1006-v2 and in WG5 N0244:

- Add two new profiles, an Extended Multiview Main profile and an Extended Multiview Main 10 profile, that are less restrictive and allow the combination of auxiliary (alpha or depth) layers with chroma_format_idc equal to 0 and primary layers with chroma_format_idc equal to 1 in an OLS.

- It is asserted that it is desirable that the bit depth of a depth component can be up to 16 bits in an MV-HEVC bitstream. Therefore, it is proposed to add a Multiview Monochrome 16 profile.

- Within an OLS containing a layer conforming to one of the new Multiview profiles (i.e., the Multiview Main 10 profile already in JVET-AF1006-v2 and in WG5 N0244 and the Extended Multiview Main and Extended Multiview Main 10 profiles proposed in this contribution), it is required that the reference layers of a current layer shall have the same chroma format, bit depth, and auxiliary type (in case of auxiliary layers) as the current layer. Note that there already exists a constraint that requires the same picture width and height for a reference layer and a current layer.

- It is asserted that it is desirable to be able to use a different bit depth and/or picture size for primary (e.g. texture), depth, and alpha maps that do not depend on each other for decoding. Therefore, it is proposed, within an OLS containing a layer conforming to one of the new Multiview profiles, to allow different layers that do not depend on each other for decoding to have a different bit depth and/or picture size.

- For the new Multiview monochrome profiles (i.e. the Multiview Monochrome, Multiview Monochrome 10, and Multiview Monochrome 12 profiles already in JVET-AF1006-v2 and in WG5 N0244 and the Multiview Monochrome 16 profile proposed in this contribution), remove or do not include the constraint that the base layer shall conform to a monochrome profile, as the constraint disallows a combination of the base layer being an existing texture video coded in a 4:2:0 format and auxiliary data that may be coded using a monochrome representation.

- Replace "ScalabiltyId" in "Table F.1 – Mapping of ScalabiltyId to scalability dimensions" with "ScalabilityId".

- Consistently use "the sub-stream subBitstream", instead of sometimes use "the sub-stream subBitstream" and sometimes just use "subBitstream".

The exact proposed text changes are included in the attachment of this contribution, with changes marked relative to JVET-AF1006-v2.

Furthermore, it is suggested that the following aspects are discussed:

- On terminology for "a layer with a nuh_layer_id value equal to i, such that DepthLayerFlag[ i ], DependencyId[ i ], and AuxId[ i ] are all equal to 0", to determine which one is better, "primary layer" or "texture layer". It should be noted that in Annex I the term “texture layer” is defined for the same thing, but on the other hand, "primary layer" is the antonym of "auxiliary layer".

- Should spatial scalability, bit-depth scalability, and colour format scalability between different views/layers with inter-layer prediction be allowed for the new profiles? This would mean that some tools that currently do not exist in Annex G (MV-HEVC) but specified in Annex H (SHVC) would be needed.

- Should the Extended Multiview Main profile be excluded and to only add the Extended Multiview Main 10 profile, as the latter is a superset of the former?

In v2 of this contribution, a couple of aspects of the specification for the Extended Multiview Main profile and an Extended Multiview Main 10 profile have been updated in the spec text attachment.

# Patent rights declaration(s)

Bytedance Inc. and Apple Inc,. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).