| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AF0047-v1 |
| --- | --- |

| Title: | AHG9: On grouping of post-processing filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang, Wei Jia, Jizheng Xu, Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Email: | {yekui.wang, wei.jia, xujizheng, lizhang.idm} @bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

At the 30th JVET meeting in April 2023, the BoG on NNPF and SEI processing order SEI messages agreed and the JVET endorsed that it is desirable to support all the following NNPF related capabilities in v4 of VSEI and VVC:

- Multiple activated NNPFs for a picture in a cascading manner.

- Multiple activated NNPFs (with the same or different purposes) for a picture in an alternative manner (e.g. choose to apply none, filter a only, or filter b only).

- Multiple activated NNPFs (with the same or different purposes) for a picture in a cascading plus alternative manner (e.g. choose to apply none, filter a only, filter b only, or both filter a and b in a cascading manner in the indicated order).

- Multiple activated NNPFs for a picture while the receiver may process the bitstream multiple times to generate multiple different results (e.g. for each time, to choose to apply none, filter a only, or filter b only).

- Multiple activated NNPF cascades in an alternative manner (e.g. choose to apply none, both filter a and b in a cascading manner in the indicated order, or both filter c and d in a cascading manner in the indicated order).

JVET-AF0061 proposed a method based on the SEI processing order (SPO) SEI message for addressing the above agreed/endorsed NNPF related capabilities and extend the scope to also cover non-NN-based post-processing filters (PPFs) in addition to NNPFs. JVET-AF0061 also included a description of how the agreed/endorsed capabilities are supported by the proposed method.

The proposed changes in JVET-AF0061 on the general PPF filtering process, the updates to the semantics of the NNPFC and NNPFA SEI messages, and the update to the VVC specification on use of NNPFC and NNPFA SEI messages have been included in the VSEI TuC.

Given that the proposed SPO based method can support the agreed/endorsed NNPF related capabilities in v4 of VSEI and VVC without syntax changes, it is proposed to move the changes from the VSEI TuC on the general PPF filtering process and the updates to the semantics of the NNPFC and NNPFA SEI messages to the next version of the VSEI preliminary WD, and the change on use of NNPFC and NNPFA SEI messages to the next version of the VVC preliminary WD.

The text changes based on JVET-AG2034-v1 and JVET-AG2027 are provided in attachments of this contribution.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).