| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0048-v1 |
| --- | --- |

| Title: | AHG4: ECM test results on some 8K HDR sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yule Sun, Yin Zhao, Elena Alshina (Huawei) Yan Li (CMG) Qian Zhang (ABP, NRTA) | Email: | elena.alshina@huawei.com liyan@cctv.com zhangqian@abp2003.cn |
| Source: | Huawei Technologies, China Media Group, Academy of Broadcasting Planning, NRTA |  |  |

_____________________________

- Abstract

Current CTC does not include 8K HDR sequences. In this contribution, three 8K HDR sequences recorded for the CCTV New Year’s Gala provided by the China Media Group (CMG) and Academy of Broadcasting Planning, NRTA, are tested with ECM, VTM and HM. The sequences have complex object motion, camera motion or background change. As targeting at broadcasting application, Random Access configuration is used. Test results show that the coding efficiency improvement of ECM-11.0 over VTM-11.0ecm11 is around 15.6% averaged across the three sequences, which is roughly half of the coding efficiency improvement of VTM-11.0ecm11 over HM18.0 under JVET Common Test Conditions (CTC).

Introduction to the test sequences

The test sequences are segments of the CCTV New Year’s Gala which is one of the most important events broadcast annually. Samples of the sequences are provided in Figures 1 to 3.

The original sequences are in YUV422, 10-bit, 50-fps, 10-second, BT.2020, HLG format. In the coding experiment, we first convert YUV422 to YUV420 by using ffmpeg (filter is bicubic), and crop the center 5K (5120x2880) region to encode, due to the very long ECM simulation time of 8K sequences (e.g., the longest RA simulation time for a one-second segment of the 5K sequences is 34 days on our machine with intel Xeon Platinum 8378A 3.00GHz). PSNR is measured in YUV420 domain. The position of 5K cropping window is depicted as the yellow boxes in Figure 1 to 3. Content in the center 5K window generally contain the most important information of these sequences, and is relatively more difficult to encode compared with other regions in the same sequence.

Figure 1. Samples of Sequence SingDance (POC 0, 166, 333, 499 for the four images)

Figure 2. Samples of Sequence PekingOpera (POC 0, 166, 333, 499 for the four images)

Figure 3. Samples of Sequence Sing (POC 0, 166, 333, 499 for the four images)

Simulation results

- Codec: HM-18.0, VTM-11.0ecm11, ECM-11.0

- Configuration: RA (GOP size 16 for VTM vs. HM, and GOP size 32 for ECM vs. VTM)

- QP: 22, 27, 32, 37

Test conditions are as much as possible aligned with JVET CTC. VTM was tested both with GOP size 16 (for comparison with HEVC) and with GOP size 32 for comparison with ECM.

Test results are summarized in the tables below.

Table 1. ECM vs VTM (using GOP 32)

|  | Random Access Main 10, GOP32 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ECM-11.0 Over VTM-11.0ecm11 (GOP size both 32) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| SingDance_centercrop_5k | -16.09% | -22.63% | -25.13% | / | / |
| PekingOpera_centercrop_5k | -12.00% | -9.47% | -10.62% | / | / |
| Sing_centercrop_5k | -18.73% | -33.12% | -31.39% | / | / |
| Overall | -15.61% | -21.74% | -22.38% | 743.2% | 756.4% |

Table 1. VTM vs HM (using GOP 16)

|  | Random Access Main 10, GOP16 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM-11.0ecm11 Over HM18.0 (GOP size both 16) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| SingDance_centercrop_5k | -32.90% | -28.46% | -33.42% | / | / |
| PekingOpera_centercrop_5k | -23.23% | 16.62% | 14.30% | / | / |
| Sing_centercrop_5k | -38.25% | -42.07% | -35.19% | / | / |
| Overall | -31.46% | -17.97% | -18.11% | 448.6% | 132.2% |

Encoding and decoding time increment of ECM compared to VTM is very similar to one regularly reported by AhG6.

For the three sequences, RA coding gain of ECM-11.0 is 15.6%, and is around half of the coding efficiency improvement from HM to VTM. In particular, the PekingOpera sequence which has camera motion and foreground character motion, is challenging for both VTM and ECM. Coding gain on it is lower than that on the other two sequences, and the bitrate at QP 22 for this 5K cropped sequence is around 110Mbps. Sequence Sing requires the lowest bitrate among the three sequences (up to 32Mbps).

RD curves of the sequences are illustrated in Figure 4. Note that the coding efficiency between VTM GOP32 and VTM GOP16 is minor for these sequences, i.e., PSNR-BD-rate of GOP 32 vs. GOP 16 is -0.43% / -2.28% / -2.37% for Y / U / V.

SingDance

PekingOpera

Sing

Figure 4 PSNR-Y RD curves of the three sequences

Left: ECM (GOP 32) vs VTM (GOP 32); Right: VTM (GOP 16) vs HM (GOP 16)

Conclusion

In this contribution, the coding performance of ECM, VTM, and HM on three typical sequences of 8K HDR show content are tested. Average coding gain in RA configuration is around 15.6%. Test results show that video sequences with vivid colors, a lot of tiny details and fast motion remain challenging for ECM.

In order to address challenged industry meets with modern TV content coding it is suggested to include more challenging sequences (similar to tested in this contribution) into JVET CTC. In case of the interest from the group, donation of test sequences can be negotiated.

Patent rights declaration(s)

Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

China Media Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Academy of Broadcasting Planning, NRTA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).