| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 14–24 April 2024 | Document: JVET-AH0050-v1 |
| --- | --- |

| Title: | EE1-1.1: LOP candidate with Low Complexity TB and new BB structure |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Shao, P. Yin, S. McCarthy (Dolby) J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya (Ittiam) D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm) | Email: | Tong.Shao@dolby.com pyin@dolby.com jay.shingala@ittiam.com ajayshyam@ittiam.com ajat.suneja@ittiam.com siddarth.badya@ittiam.com dmytror@qti.qualcomm.com yli30@qti.qualcomm.com |
| Source: | Dolby, Ittiam, Qualcomm |  |  |

_____________________________

# Abstract

A Unified Filter Architecture for the LOP and its training was proposed in JVET-AF0043 and adopted to NNVC as LOP2. In this test combination of separable convolutions in fusion/transition block (TB) (JVET-AG0155) and modified Backbone (BB) block (JVET-AG0163) for LOP is jointly trained by proponents. The Stage 3 training followed procedure selected in EE1-1.0.

Inference was conducted in int16 implementation with NNVC-8.0. Compared to LOP2 anchor, complexity reduction of 0.3% (kMAC/pixel) and 19% of convolutional layers reduction are reported.

Comparing to EE1-1.0 anchor, EE1-1.1 design reportedly provides: { -0.2%, 1.4%, 2.0%} and { -0.1%, 1.5%, 2.3%} for RA and AI, respectively.

Comparing to NNVC-8.0 Anchor (LOP2, nnIntra ON), EE1-1.1 with NNIntra ON, reportedly provides: { -0.4%, 1.6%, 1.5%} and { -0.1%, 1.4%, 1.3%} for RA and AI, respectively.

Comparing to VTM anchor, EE1-1.1 with NNIntra ON, reportedly provides: { -7.2%, -11.8%, -10.9%} and { -8.2%, -12.1%, -12.3%} for RA and AI, respectively.

It is proposed to adopt EE1-1.1 test into to NNVC for LOP.

# Introduction

A Unified Filter Architecture for the LOP and its training was proposed in JVET-AF0043 and adopted to NNVC as LOP2, see Figure 1.

Figure 1 LOP2 network architecture.

In test EE1-1.0, the LOP2 training strategy was refined and selected as anchor to the LOP tests for this meeting cycle. The hyper parameters of LOP2 training selected in EE1-1.0 are shown below:

- batch size 64

- rate 0.0008

- ‘milestones’ 41, 51, 55.

Further complexity reduction of LOP2 architecture, such as separable convolutions in fusion/transition block (JVET-AG0155) and modified BB block (JVET-AG0163) for LOP, have been proposed and provided a base for current EE1 test.

# Proposed architecture

In this test a combination of separable convolutions in fusion/transition block (JVET-AG0155) and modified BB block (JVET-AG0163) for LOP is jointly trained by proponents. EE1-1.1 LOP architecture is shown in Figure 2.

Figure 2. EE1-1.1 architecture.

# Training process

Training was conducted following EE1-1.0 selected procedure independently by Dolby, Ittiam and Qualcomm. Summary of training information is shown in Table 1.

Luma and Chroma L1 validation loss for Qualcomm (QC Test A) and Dolby-Ittiam (DbItt Test A) trained models are shown in Figure 3 and Figure 4 respectively. From the figures it is observed that chroma validation loss is similar while the “QC Test A” showed slightly better luma validation loss. Further inference test is conducted to confirm the validation results.

Table 1 Training information.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Nvidia GeForce RTX4090 / A100 |
|  | Framework: | Pytorch v1.12.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | Stage3: 60 epochs |
|  | Batch size: | Stage3: 64 |
|  | Loss function: | L1, L2 |
|  | Training time (for 1 model): | ~7 to 10 days per stage |
|  | Training data information: | LOP2 Stage 3 dataset: DIV2K, BVI-DVC, TVD |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | Stage3: 8e-4 |
|  | Learning rate update strategy | Stage3: 41, 51, 55. mse-epoch:58 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Training data update strategy: |  |
|  | Other information: |  |

Figure 3 Joint training results: Luma L1 validation loss

Figure 4 Joint training results: Chroma L1 validation loss

# Inference process

Inference testing was conducted using NNVC-8.0 software by Dolby, Ittiam and Qualcomm independently. The inference information is shown in Table 2, summaries of simulations are presented in Table 3 to Table 7. Complete simulation results are provided along with this document.

Table 2 Inference information

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL float, SADL int16 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters | 46186 |
|  | Total Parameter Number |  |
|  | Parameter Precision (Bits) | Floating (32 bits) / integer (16 bits) |
|  | Memory Parameter (MB) | 0.05 |
|  | MAC/Pixel (Kilo) | 17.0 |
| Optional |  |  |
|  | Total Conv. Layers | 86 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 128x128 / 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Other information: |  |

Summary of results for proposed method (float, nnIntra OFF) trained by Qualcomm comparing to EE1-1.0 anchor is shown in Table 3.

Table 3 Inference testing results(float) Qualcomm trained, BD-rate change comparing to EE1-1.0, nnIntra OFF

|  | EE1-1.1 QC vs EE1-1.0, float (NNVC-7.1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  |  | AI |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0.42% | 0.18% | 0.88% | 0.09% | 1.35% | 3.28% |
| Class A2 | -0.17% | 0.94% | 1.85% | -0.09% | 0.90% | 1.53% |
| Class B | -0.26% | 1.56% | 2.03% | -0.18% | 1.51% | 1.91% |
| Class C | -0.33% | 1.61% | 2.12% | -0.14% | 2.43% | 2.52% |
| Class E |  |  |  | -0.24% | 1.42% | 2.12% |
| Overall | -0.13% | 1.17% | 1.79% | -0.12% | 1.57% | 2.25% |
| Class D | -0.30% | 2.26% | 1.86% | -0.17% | 2.43% | 2.21% |
| Class F | -0.09% | 1.12% | 1.88% | 0.02% | 1.49% | 0.81% |

Summary of results for proposed method (float, nnIntra OFF) trained by Dolby and Ittiam comparing to EE1-1.0 anchor is shown in Table 4.

Table 4 Inference testing results(float) Dolby Ittiam trained, BD-rate change comparing to EE1-1.0, nnIntra OFF

|  | EE1-1.1 DbItt vs EE1-1.0, float (NNVC-7.1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  |  | AI |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0.44% | -0.24% | 2.02% | 0.17% | 0.89% | 4.74% |
| Class A2 | -0.11% | 1.15% | 1.71% | -0.07% | 0.68% | 1.60% |
| Class B | -0.12% | 1.73% | 1.91% | -0.13% | 1.31% | 2.23% |
| Class C | -0.15% | 1.90% | 2.48% | -0.10% | 1.78% | 2.96% |
| Class E |  |  |  | -0.22% | 1.69% | 2.57% |
| Overall | -0.01% | 1.27% | 2.04% | -0.08% | 1.30% | 2.76% |
| Class D | -0.26% | 2.81% | 2.16% | -0.11% | 2.48% | 2.49% |
| Class F | -0.09% | 0.72% | 0.94% | -0.04% | 0.48% | 0.30% |

Model trained by Qualcomm has better BD Rate results, hence it was selected for integer quantization.

Summary of comparative results for proposed method (int16, nnIntra OFF) comparing to EE1-1.0 anchor is shown in Table 5.

Table 5 Inference testing results, BD-rate change comparing to EE1-1.0, nnIntra OFF for all.

|  | EE1-1.1 vs EE1-1.0, Int16 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  |  | AI |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0.41% | 0.93% | 1.04% | 0.11% | 0.83% | 1.70% |
| Class A2 | -0.13% | 1.15% | 2.08% | -0.02% | 0.79% | 1.63% |
| Class B | -0.40% | 1.62% | 2.53% | -0.22% | 1.45% | 2.11% |
| Class C | -0.42% | 1.53% | 2.37% | -0.20% | 2.44% | 2.73% |
| Class E |  |  |  | -0.30% | 1.51% | 2.26% |
| Overall | -0.19% | 1.36% | 2.10% | -0.14% | 1.47% | 2.12% |
| Class D | -0.49% | 2.27% | 2.30% | -0.26% | 2.58% | 2.43% |
| Class F | -0.27% | 0.95% | 1.69% | -0.02% | 1.42% | 0.77% |

Summary of comparative results for proposed method (int16, nnIntra ON) comparing to NNVC-8.0 anchor (LOP2, nnIntra ON) is shown in Table 6.

Table 6 Inference testing results with nnIntra ON, BD-rate change comparing to NNVC-8.0 anchor.

|  | EE1-1.1 vs NNVC8.0, Int16 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  |  | AI |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.05% | 1.18% | -0.49% | 0.10% | 1.54% | 0.18% |
| Class A2 | -0.47% | 1.91% | 1.59% | -0.03% | 1.09% | 1.26% |
| Class B | -0.41% | 1.91% | 2.10% | -0.14% | 1.22% | 1.14% |
| Class C | -0.41% | 1.39% | 2.19% | -0.11% | 1.99% | 1.94% |
| Class E |  |  |  | -0.16% | 1.32% | 2.02% |
| Overall | -0.35% | 1.63% | 1.50% | -0.08% | 1.44% | 1.32% |
| Class D | -0.40% | 1.30% | 2.09% | -0.13% | 1.85% | 2.15% |
| Class F | -0.23% | 1.18% | 1.96% | -0.05% | 1.76% | 0.79% |

Summary of comparative results for proposed method (int16, nnIntra ON) comparing to VTM anchor (nnTools OFF) is shown in Table 7.

Table 7 Inference testing results with nnIntra ON, BD-rate change comparing to VTM anchor.

|  | EE1-1.1 vs VTM Anchor, Int16 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  |  | AI |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -7.96% | -9.10% | -11.06% | -8.53% | -11.36% | -12.20% |
| Class A2 | -7.23% | -11.10% | -8.34% | -7.18% | -11.78% | -10.17% |
| Class B | -6.95% | -12.33% | -10.89% | -7.48% | -11.98% | -12.57% |
| Class C | -6.97% | -13.70% | -12.57% | -7.63% | -12.14% | -12.86% |
| Class E |  |  |  | -10.85% | -13.03% | -13.46% |
| Overall | -7.21% | -11.80% | -10.86% | -8.20% | -12.05% | -12.32% |
| Class D | -7.53% | -13.78% | -13.22% | -7.77% | -11.13% | -13.27% |
| Class F | -3.93% | -8.53% | -7.71% | -5.32% | -9.33% | -9.71% |

# Additional information (Test B)

EE1-1.1 introduces changes to the length and depth of luma and chroma branches, thus validity of the currently utilized in EE1-1.0 training weights "component_loss_weightings" = [12, 2] was questioned. Proponents jointly trained the EE1-1.1 architecture with modified luma/chroma weights, parameters "component_loss_weightings" was set equal to [10, 2].

Summary of comparative results for this Test B (int16, nnIntra OFF) comparing to EE1-1.0 anchor is shown in Table 8.

Table 8 Inference testing results (TestB), BD-rate change comparing to EE1-1.0, nnIntra OFF for all.

|  | EE1-1.1 vs EE1-1.0, Int16 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  |  | AI |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0.46% | 0.12% | -0.02% | 0.19% | 0.16% | -0.09% |
| Class A2 | 0.00% | 0.62% | 1.81% | -0.04% | -0.07% | 0.56% |
| Class B | -0.14% | 0.39% | 0.84% | -0.15% | 0.15% | 0.88% |
| Class C | -0.28% | 0.62% | 1.28% | -0.20% | 0.57% | 1.19% |
| Class E |  |  |  | -0.27% | -0.16% | 0.61% |
| Overall | -0.03% | 0.45% | 0.98% | -0.11% | 0.16% | 0.69% |
| Class D | -0.41% | 1.35% | 1.48% | -0.24% | 1.33% | 0.97% |
| Class F | -0.14% | 0.66% | 1.04% | 0.02% | 1.46% | 1.63% |

Based on the observed results, it may be concluded that luma/chroma weights utilized in EE1-1.0 remain beneficial for proposed architecture.

# Conclusion:

In this test combination of separable convolutions in fusion/transition block (JVET-AG0155) and modified BB block (JVET-AG0163) for LOP is jointly trained by proponents. The Stage 3 training was conducted, following the procedure selected in EE1-1.0.

Inference was conducted in int16 implementation with NNVC-8.0. Compared to LOP2 anchor, complexity reduction of 0.3% (kMAC/pixel) and 19% of convolutional layers reduction are reported.

Comparing to EE1-1.0 anchor, EE1-1.1 design reportedly provides: {-0.2%, 1.4%, 2.0%} and {-0.1%, 1.5%, 2.3%} of BD-rate changes in RA and AI configurations, respectively.

Comparing to NNVC8.0 Anchor (LOP2, nnIntra ON), EE1-1.1 with NNIntra ON, reportedly provides: { -0.4%, 1.6%, 1.5%} and { -0.1%, 1.4%, 1.3%}  of BD-rate changes for RA and AI, respectively.

Comparing to VTM anchor, EE1-1.1 with NNIntra ON, reportedly provides: { -7.2%, -11.8%, -10.9%} and { -8.2%, -12.1%, -12.3%} for RA and AI, respectively.

It is proposed to adopt EE1-1.1 test into to NNVC for LOP.

# References

- AHG11: Status of the joint EE1-0 (LOP.2) unified filter training, JVET-AF0043, Dmytro Rusanovskyy, Yun Li, Marta Karczewicz (Qualcomm), Jay N. Shingala, Ajay Shyam (Ittiam), Tong Shao, Peng Yin (Dolby).

- Dmytro Rusanovskyy, Yun Li, Marta Karczewicz (Qualcomm), “EE1-Related: On LOP2 training process”, JVET-AG0156, January 2024.

- T. Shao, P. Yin, S. McCarthy, J. N. Shingala, A. Shyam, A. Suneja, S. Badya, “EE1-2.3: Further complexity reduction on the joint LOP.2”, JVET-AG0163, January 2024.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Dolby may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).