| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, by Rennes, FR, 17–24 April 2024 | Document: JVET-AH0053-v1 |
| --- | --- |

| Title: | AHG9: Comments on Generative Face Video SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Technologies under consideration for future extensions of VSEI includes a Generative Face Video (GFV) SEI. Some modifications are proposed to GFV SEI. Following is proposed:

- Proposal 1: It is proposed to conditionally signal gfv_base_pic_flag only when gfv_cnt is equal to 0 and infer its value otherwise.

- Proposal 2: A bitstream conformance constraint is proposed to require that the value of the syntax element gfv_num_matrix_types_minus1 is the same for the base picture and corresponding non-base picture with gfv_matrix_pred_flag equal to 1. Also a bitstream conformance constraint is proposed to require that the value of derived variables numMatrices[ i ], matrixHeight[ i ], matrixWidth[ i ] is the same for the base picture and corresponding non-base picture with gfv_matrix_pred_flag equal to 1, for each i in the range of 0 to gfv_num_matrix_types_minus1, inclusive.

- Proposal 3: It is asserted that when gfv_coordinate_present_flag is equal to 0, gfv_num_matrices_equal_to_num_kps_flag[ i ] is not present and gfv_num_matrices_equal_to_num_kps_flag[ i ] does not have an inferred value when not present, but it is used in a conditional if statement (with an OR operator) to gate the syntax element gfv_num_matrices_info[ i ]. It is proposed to infer a value for gfv_num_matrices_equal_to_num_kps_flag[ i ] when not present to avoid undefined behavior. It is also asserted that this simplifies the if condition to signal gfv_num_matrices_info[ i ] syntax element.

- Proposal 4: It is asserted that when gfv_coordinate_z_present_flag is equal to 0, syntax elements gfv_coordinate_z_max_value_minus1, gfv_coordinate_z_abs[ i ], gfv_coordinate_z_sign_flag[ i ], gfv_coordinate_dz_abs[ i ], and gfv_coordinate_dz_sign_flag[ i ]  are not present. Thus it is asserted that a conditional check regarding if gfv_coordinate_z_present_flag is equal to 1 needs to be performed in the derivation of coordinateZ[ i ], coordinateDeltaZ[ i ]  and BaseKpCoordinateZ[ i ].

# Introduction

The Technologies under Consideration for future extension of VSEI [1] document includes a Generative Face Video SEI. The generative face video (GFV) SEI message indicates facial parameters and specifies a facial parameter translator network, denoted as TranslatorNN( ), that may be used to convert various formats of facial parameters signaled in the SEI message into a fixed format of parameters, and a face picture generator neural network, denoted as GenerativeNN( ), that may be used to generate output pictures using the fixed format of facial parameters and previously decoded output pictures.

Some modifications are proposed to GFV SEI. Following is proposed:

- Proposal 1: It is proposed to conditionally signal gfv_base_pic_flag only when gfv_cnt is equal to 0 and infer its value otherwise.

- Proposal 2: A bitstream conformance constraint is proposed to require that the value of the syntax element gfv_num_matrix_types_minus1 is the same for the base picture and corresponding non-base picture with gfv_matrix_pred_flag equal to 1. Also a bitstream conformance constraint is proposed to require that the value of derived variables numMatrices[ i ], matrixHeight[ i ], matrixWidth[ i ] is the same for the base picture and corresponding non-base picture with gfv_matrix_pred_flag equal to 1, for each i in the range of 0 to gfv_num_matrix_types_minus1, inclusive.

- Proposal 3: It is asserted that when gfv_coordinate_present_flag is equal to 0, gfv_num_matrices_equal_to_num_kps_flag[ i ] is not present and gfv_num_matrices_equal_to_num_kps_flag[ i ] does not have an inferred value when not present, but it is used in a conditional if statement (with an OR operator) to gate the syntax element gfv_num_matrices_info[ i ]. It is proposed to infer a value for gfv_num_matrices_equal_to_num_kps_flag[ i ] when not present to avoid undefined behavior. It is also proposed to modify the condition to signal gfv_num_matrices_info[ i ].

- Proposal 4: It is asserted that when gfv_coordinate_z_present_flag is equal to 0, syntax elements gfv_coordinate_z_max_value_minus1, gfv_coordinate_z_abs[ i ], gfv_coordinate_z_sign_flag[ i ], gfv_coordinate_dz_abs[ i ], and gfv_coordinate_dz_sign_flag[ i ]  are not present. Thus it is asserted that a conditional check regarding if gfv_coordinate_z_present_flag is equal to 1 needs to be performed in the derivation of coordinateZ[ i ], coordinateDeltaZ[ i ]  and BaseKpCoordinateZ[ i ].

# Proposals

## Proposal 1

In Technologies under consideration for future extensions of VSEI, the syntax element gfv_cnt specifies a GFV SEI message instance count value for the particular gfv_id value within a picture unit. Also, gfv_base_pic_flag specifies if the current decoded picture corresponds to a base picture or not.

It is asserted that currently when gfv_cnt is greater than 0, gfv_base_pic_flag is required to be equal to 0. As a result, it is asserted that signaling gfv_base_pic flag syntax element is not required when gfv_cnt is greater than 0 and it needs to be signalled only when gfv_cnt is equal to 0. It is thus proposed to conditionally signal gfv_base_pic_flag only when gfv_cnt is equal to 0 and infer its value otherwise. It is proposed that when not present the value of gfv_base_pic_flag is inferred to be equal to 0.

The proposed modifications are shown below.

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_cnt | ue(v) |
| if( gfv_cnt == 0 ) |  |
| gfv_base_pic_flag /*indicate if current decoded output picture is a base picture*/ | u(1) |
| if( gfv_base_pic_flag ) { /*specify TranslatorNN( )*/ |  |
| gfv_nn_present_flag | u(1) |
| if( gfv_nn_present_flag ) { |  |
| gfv_nn_base_flag | u(1) |
| gfv_nn_mode_idc | ue(v) |
| if( gfv_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_a | u(1) |
| gfv_nn_tag_uri | st(v) |
| gfv_nn_uri | st(v) |
| } |  |
| } |  |
| } else /* current decoded output picture is a driving picture*/ |  |
| gfv_drive_pic_fusion_flag /*indicate if DrivePicture is input to GenerativeNN( )*/ | ue(v) |
| gfv_coordinate_present_flag | u(1) |
| … |  |
| } |  |

gfv_cnt specifies a GFV SEI message instance count value for this gfv_id value within a picture unit.

The gfv_cnt of the first GFV SEI message, in decoding order, with a particular value of gfv_id within picture unit shall be equal to 0. When gfv_cnt assigned to currGfvCnt is greater than 0, a GFV SEI message with the same gfv_id value and gfv_cnt equal to currGfvCnt − 1 shall precede the current GFV SEI message in decoding order in the same picture unit.

The value of gfv_cnt shall be in the range of 0 to 65 535, inclusive.

gfv_base_pic_flag equal to 1 indicates the current decoded output picture corresponds to a base picture. gfv_base_pic_flag equal to 0 indicates the current decoded output picture does not correspond to a base picture or this SEI message does not specify syntax elements for a base picture. When gfv_cnt is greater than 0, gfv_base_pic_flag shall be equal to 0. When gfv_base_pic_flag is not present, it is inferred to be equal to 0.

The following constraints apply to the value of gfv_base_pic_flag:

–	When a GFV SEI message is the first GFV SEI message, in decoding order, that has a particular gfv_id value within the current CLVS, the value of gfv_base_pic_flag shall be equal to 1.

–	When a GFV SEI message that has a particular gfv_id value has gfv_base_pic_flag being equal to 0, this SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that follows the current decoded picture in output order within the current CLVS and is associated with a subsequent GFV SEI message, in decoding order, having gfv_base_pic_flag equal to 0 and that particular gfv_id value within the current CLVS, whichever is earlier.

## Proposal 2

It is asserted that when gfv_matrix_pred_flag is equal to 1, the derived variable matrixElementVal[ i ][ j ][ k ][ m ] requires the use of the derived variable BaseMatrixElementVal[ i ][ j ][ k ][ m ] which is set when gfv_base_pic flag is equal to 1, i.e. when the current decoded output picture corresponds to a base picture. Thus it is asserted that for the derivation of matrix prediction variables matrixElementVal[ i ][ j ][ k ][ m ] to work as intended, there must be a valid value for BaseMatrixElementVal[ i ][ j ][ k ][ m ] for each of i, j, k, m, when gfv_matrix_pred_flag is equal to 1 for the non-base picture.

To satisfy this, a bitstream conformance constraint is proposed to require that the value of the syntax element gfv_num_matrix_types_minus1 is the same for the base picture and corresponding non-base picture with gfv_matrix_pred_flag equal to 1. Also a bitstream conformance constraint is proposed to require that the value of derived variables numMatrices[ i ], matrixHeight[ i ], matrixWidth[ i ] is the same for the base picture and corresponding non-base picture with gfv_matrix_pred_flag equal to 1, for each i in the range of 0 to gfv_num_matrix_types_minus1, inclusive.

The proposed modifications are as shown below:

gfv_matrix_present_flag equal to 1 indicates that matrix parameters are present. gfv_matrix_present_flag equal to 0 indicates that matrix parameters are not present.

gfv_matrix_element_precision_factor_minus1 plus 1 indicates the length, in bits, of syntax elements gfv_matrix_element_dec[ i ][ j ][ k ][ m ] and gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ].

gfv_num_matrix_types_minus1 plus 1 indicates the number of matrix types signalled in the SEI message. The value of gfv_num_matrix_typesmatrix_type_num_minus1 shall be in the range of 0 to 26 – 1, inclusive.

It is a requirement of bitstream conformance that when gfv_matrix_pred_flag is equal to 1 and gfv_base_pic_flag is equal to 0, the value of gfv_num_matrix_types_minus1 shall be equal to the value of gfv_num_matrix_types_minus1 in each of the preceding GFV SEI message in decoding order in the current CLVS which has the same gfv_id value as the gfv_id value in the current SEI and has gfv_base_pic_flag equal to 1.

gfv_matrix_pred_flag equal to 1 indicates syntax elements gfv_matrix_element_int [ i ][ j ][ k ][ m ] , gfv_matrix_element_dec [ i ][ j ][ k ][ m ] are present and gfv_matrix_element_sign_flag [ i ][ j ][ k ][ m ] may be present. gfv_matrix_pred_flag equal to 0 indicates gfv_matrix_delta_element_int [ i ][ j ][ k ][ m ] , gfv_matrix_delta_element_dec [ i ][ j ][ k ][ m ] are present and syntax element gfv_matrix_delta_element_sign_flag [ i ][ j ][ k ][ m ] may be present. When gfv_matrix_pred_flag is not present, it is inferred to be 0.

…

gfv_num_matrices_equal_to_num_kps_flag[ i ] equal to 1 indicates that the number of matrices of the i-th matrix type is equal to gfv_num_kps_minus1 + 1. gfv_num_matrices_equal_to_num_kps_flag[ i ] equal to 0 indicates the number of matrices of the i-th matrix type is not equal to gfv_num_coordinates_minus1 + 1.

gfv_num_matrices_info[ i ] provides information to derive the number of the matrices of the i-th matrix type.

gfv_matrix_width_minus1[ i ] plus 1 indicates the width of the matrix of the i-th matrix type.

gfv_matrix_height_minus1[ i ] plus 1 indicates the height of the matrix of the i-th matrix type.

gfv_matrix_for_3D_space_flag[ i ] equal to 1 indicates the matrix of the i-th matrix type is a matrix defined in three-dimensional space. gfv_matrix_for_3D_space_flag[ i ] equal to 0 indicates the matrix of the i-th matrix type is a matrix defined in two-dimensional space.

When gfv_matrix_width_minus1[ i ] is not present, it is inferred as follows:

––	If gfv_matrix_type_idx[ i ] is equal to 0, 1 or 4, and one of coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is present and equal to 1, gfv_matrix_width_minus1[i] is inferred to be equal to 2

––	otherwise, if matrix_type_idx[ i ] is equal to 0, 1 or 4, and one of coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is present and equal to 0, gfv_matrix_width_minus1[ i ] is inferred to be equal to 1

––	otherwise (matrix_type_idx[ i ] is equal to 5 or 6), gfv_matrix_width_minus1[ i ] is inferred to be equal to 0

When gfv_matrix_height_minus1[ i ] is not present, it is inferred as follows:

––	If matrix_type_idx is equal to 0, 1, 4, 5 or 6, and one of gfv_coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is present and equal to 1, gfv_matrix_height_minus1[ i ] is inferred to be equal to 2.

––	otherwise (gfv_matrix_type_idx is equal to 0, 1, 4, 5 or 6, and one of gfv_coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is 0), gfv_matrix_height _minus1[ i ] is inferred to be equal to 1.

The variables matrixWidth[ i ] and matrixHeight[ i ] indicating the width and height of the matrix of the i-th matrix type are derived as follows

matrixWidth[ i ] = gfv_matrix_width_minus1[ i ] + 1

matrixHeight[ i ] = gfv_matrix_height_minus1[ i ] + 1

gfv_num_matrices_minus1[ i ] plus 1 indicates the number of matrices of the i-th matrix type.

The variable numMatrices[ i ] indicating the number of the matrices of the i-th matrix type is derived as follows:

if( gfv_matrix_type_idx[ i ] == 0 || gfv_matrix_type_idx[ i ] == 1 ) {

if( gfv_coordinate_present_flag )

numMatrices[ i ] = gfv_num_matrices_equal_to_num_kps_flag[ i ] ? gfv_num_kps_minus1 + 1 : ( gfv_num_matrices_info[ i ] < gfv_num_kp_minus1 ? gfv_num_matrices_info [ i ] + 1 : gfv_num_matrices_info [ i ] + 2 )

else

numMatrices[ i ] = gfv_num_matrices_info[ i ] + 1

}

else if( gfv_matrix_type_idx[ i ] >= 2 &&  gfv_matrix_type_idx[ i ] < 7)

numMatrices[ i ] = 1

else

numMatrices[ i ] = gfv_num_matrices_minus1[ i ] + 1

It is a requirement of bitstream conformance that when gfv_matrix_pred_flag is equal to 1 and gfv_base_pic_flag is equal to 0, the values of numMatrices[ i ] , matrixWidth[ i ], and matrixHeight[ i ] for i in the range of 0 to gfv_num_matrix_types_minus1, inclusive shall be respectively equal to the values of numMatrices[ i ], matrixWidth[ i ], and matrixHeight[ i ] for i in the range of 0 to gfv_num_matrix_types_minus1, inclusive in each of the preceding GFV SEI message in decoding order in the current CLVS which has the same gfv_id value as the gfv_id value in the current SEI and has gfv_base_pic_flag equal to 1.

## Proposal 3

It is asserted that when gfv_coordinate_present_flag is equal to 0, gfv_num_matrices_equal_to_num_kps_flag[ i ] is not present and gfv_num_matrices_equal_to_num_kps_flag[ i ] does not have an inferred value when not present, but it is used in a conditional if statement (with an OR operator) to gate the syntax element gfv_num_matrices_info[ i ]. It is proposed to infer a value for gfv_num_matrices_equal_to_num_kps_flag[ i ] when not present to avoid undefined behavior. It is also asserted that this simplifies the if condition to signal gfv_num_matrices_info[ i ] syntax element.

The proposed modifications are shown below.

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| … |  |
| gfv_coordinate_present_flag |  |
| if( gfv_coordinate_present_flag ) { |  |
| gfv_coordinate_precision_factor_minus1 |  |
| gfv_num_kp_minus1 |  |
| gfv_kp_pred_flag |  |
| … |  |
| gfv_matrix_present_flag | u(1) |
| if(gfv_matrix_present_flag ) { |  |
| gfv_matrix_element_precision_factor_minus1 | ue(v) |
| gfv_num_matrix_types_minus1 | ue(v) |
| if( !gfv_base_pic_flag ) |  |
| gfv_matrix_pred_flag | u(1) |
| for( i = 0; i <= num_matrix_types_minus1; i++ ) { |  |
| gfv_matrix_type_idx[ i ] | u(6) |
| if( gfv_matrix_type_idx[ i ] = = 0 ¦ ¦ gfv_matrix_type_idx[ i ] = = 1 ) { |  |
| if( gfv_coordinate_present_flag ) |  |
| gfv_num_matrices_equal_to_num_kps_flag[ i ] | u(1) |
| if( !gfv_coordinate_present_flag ¦ ¦ !gfv_num_matrixmatrices_equal_to_num_kps_flag[ i ] ) |  |
| gfv_num_matrices_info[ i ] | ue(v) |
| }else if( gfv_matrix_type_idx[ i ] = = 2 ¦ ¦ gfv_matrix_type_idx[ i ] = = 3 ¦ ¦ gfv_matrix_type_idx[ i ] >= 7 ){ |  |
| if( gfv_matrix_type_idx[ i ] >= 7 ) |  |
| gfv_num_matrices_minus1[ i ] | ue(v) |
| gfv_matrix_width_minus1[ i ] | ue(v) |
| gfv_matrix_height_minus1[ i ] | ue(v) |
| }else if( gfv_matrix_type_idx[ i ] >= 4 && gfv_matrix_type_idx[ i ] <= 6 && !gfv_coordinate_present_flag ){ |  |
| gfv_matrix_for_3D_space_flag[ i ] | u(1) |
| for( j = 0; j < numMatrices[ i ]; j++ ) |  |
| for( k = 0; k < matrixHeight[ i ]; k++ ) |  |
| for( m = 0; m <matrixWidth[ i ]; m++ ) { |  |
| if( !gfv_matrix_pred_flag ) { |  |
| gfv_matrix_element_int[ i ][ j ][ k ][ m ] | ue(v) |
| gfv_matrix_element_dec[ i ][ j ][ k ][ m ] | u (v) |
| if( gfv_matrix_element_int[ i][ j ][ k ][ m ] ¦ ¦ gfv_matrix_element_dec[ i ][ j ][ k ][ m ] ) |  |
| gfv_matrix_element_sign_flag[ i ][ j ][ k ][ m ] | u(1) |
| } |  |
| else { |  |
| gfv_matrix_delta_element_int[ i ][ j ][ k ][ m ] | ue(v) |
| gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ] | u (v) |
| if( gfv_matrix_delta_element_int[ i][ j ][ k ][ m ] ¦ ¦ gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ] ) |  |
| gfv_matrix_delta_element_sign_flag[ i ][ j ][ k ][ m ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

gfv_num_matrices_equal_to_num_kps_flag[ i ] equal to 1 indicates that the number of matrices of the i-th matrix type is equal to gfv_num_kps_minus1 + 1. gfv_num_matrices_equal_to_num_kps_flag[ i ] equal to 0 indicates the number of matrices of the i-th matrix type is not equal to gfv_num_coordinates_minus1 gfv_num_kps_minus1 + 1. When not present gfv_num_matrices_equal_to_num_kps_flag[ i ] is inferred to be equal to 0.

## Proposal 4

It is asserted that when gfv_coordinate_z_present_flag is equal to 0, syntax elements gfv_coordinate_z_max_value_minus1, gfv_coordinate_z_abs[ i ], gfv_coordinate_z_sign_flag[ i ], gfv_coordinate_dz_abs[ i ], and gfv_coordinate_dz_sign_flag[ i ]  are not present. Thus it is asserted that a conditional check regarding if gfv_coordinate_z_present_flag is equal to 1 needs to be performed in the derivation of coordinateZ[ i ], coordinateDeltaZ[ i ]  and BaseKpCoordinateZ[ i ].

The proposed modifications are shown below.

The variables coordinateDeltaX[ i ], coordinateDeltaY[ i ] and when gfv_coordinate_z_present_flag  is equal to 1 coordinateDeltaZ[ i ] indicating the delta x-axis coordinate, delta y-axis coordinate and delta z-axis coordinate of the i-th keypoint, respectively, are derived as follows:

coordinateDeltaX[ i ] = (1-2 * gfv_coordinate_dx_sign_flag[ i ]) * gfv_coordinate_dx_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateDeltaY[i] = (1-2 * gfv_coordinate_dy_sign_flag[ i ]) * gfv_coordinate_dy_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

if (gfv_coordinate_z_present_flag ) 
	    coordinateDeltaZ[i] = (1-2 * coordinate_dz_sign_flag[ i ]) * gfv_coordinate_dz_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

The variables coordinateX[ i ], coordinateY[ i ] and coordinateZ[ i ] indicating the x-axis coordinate, y-axis coordinate and z-axis coordinate of the i-th keypoint, respectively, are derived as follows:

When gfv_kp_pred_flag is equal to 0,

coordinateX[ i ] = (1-2 * gfv_coordinate_x_sign_flag[ i ]) * gfv_coordinate_x_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateY[i] = (1-2 * gfv_coordinate_y_sign_flag[ i ]) * gfv_coordinate_y_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

if (gfv_coordinate_z_present_flag ) 
		coordinateZ[i] = (1-2 * gfv_coordinate_z_sign_flag[ i ]) * gfv_coordinate_z_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

when gfv_kp_pred_flag is equal to 1,

if( gfv_base_pic_flag ) {
	coordinateX[ i ] = (( i > 0 ) ? coordinateX[ i - 1 ] : 0 ) + coordinateDeltaX[ i ]
	coordinateY[ i ] = (( i > 0 ) ? coordinateY[ i - 1 ] : 0 ) + coordinateDeltaY[ i ] 
	if (gfv_coordinate_z_present_flag )
			coordinateZ[ i ] = (( i > 0 ) ? coordinateZ[ i - 1 ] : 0 ) + coordinateDeltaZ[ i ]
}
else {
	coordinateX[ i ] = BaseKpCoordinateX[ i ] + coordinateDeltaX[ i ]
	coordinateY[ i ] = BaseKpCoordinateY[ i ] + coordinateDeltaY[ i 
	if (gfv_coordinate_z_present_flag )
		coordinateZ[ i ] = BaseKpCoordinateZ[ i ] + coordinateDeltaZ[ i ]
}

where BaseKpCoordinateX[ i ], BaseKpCoordinateY[ i ], BaseKpCoordinateZ[ i ] indicating the x-axis, y-axis and z-axis coordinates, respectively, of the i-th keypoint for the base picture are derived as follows:

if( gfv_base_pic_flag ) {
	BaseKpCoordinateX[ i ] = coordinateX[ i ]
	BaseKpCoordinateY[ i ] = coordinateY[ i ] 	
	if (gfv_coordinate_z_present_flag )
		BaseKpCoordinateZ[ i ] = coordinateZ[ i ]
}

# Conclusion

This document proposes modifications to the Generative Face Video SEI. It is proposed to adopt the SEI message into the modifications in the output document which includes GFV SEI message.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JVET-AG2032, M. M. Hannuksela, J. Chen, S. Deshpande, S. McCarthy, Technologies under consideration for future extensions of VSEI, March 2023

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.