| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0054-v1 |
| --- | --- |

| Title: | AHG16: On the generative face video SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Boon Teo Jing Yuan Thong Kiyofumi Abe Chong Soon Lim Karlekar Jayashree | Tel: Email: | +65-6550-5417 hanboon.teo@sg.panasonic.com jingyuan.thong@sg.panasonic.com abe.kiyo@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution proposes modifications to the generative face video (GFV) SEI message outlined in TuC for future extensions of VSEI, JVET-AG2032. In particular, the following are proposed to be included:

Aspect 1: A flag that indicates the facial parameters in current SEI message were detected with low confidence.

Aspect 2: A flag that indicates to copy facial parameters from previously decoded generative face video SEI message.

# Aspect 1: Confidence of facial parameters

## Problem Statement

Although facial parameters may be derived from the driving picture and transmitted in the bitstream, these may be created with low confidence. This may arise from, but not limited to, the following scenarios:

- Partial presence of face

- Momentary occlusions of a part of the face, like face mask, eye patch, face adornments, or any other objects that could potentially contribute to the partial or total concealment of the face

- Extreme head poses

- Motion blur, possibly due to face movement larger than camera capturing rate, or sudden camera movements

- Simple or low complexity face detector used at encoder

However, such information may lead to generation of distorted or erroneous frames. It is proposed to add a simple signal to inform the decoder of the low confidence. This allows the encoder to explicitly inform the decoder about it so that the decoder can consider performing additional processing on the facial parameters such as error concealment techniques.

## Proposal

It is proposed to incorporate a flag to indicate if the facial parameters present in the SEI message were derived at the encoder with low confidence. Since only the encoder can differentiate if no issues were faced during the derivation of facial parameters, or if the reliability of derived facial parameters were low due to any of the above scenarios, this information of the facial parameters can be signalled in the bitstream and passed on to the decoder.

Upon receiving this signal, the decoder can either:

- continue using the current decoded low confidence facial parameters as it is, or

- perform its own error concealment, such as re-using or combining with previously decoded facial parameters.

Two options are proposed,

Option 1: single low confidence flag that represents confidence for all the facial parameters in current GFV SEI message.

Option 2: multiple low confidence flags, each flag represents confidence for coordinate information of keypoints and each gfv_matrix_type.

## Proposed changes to GFV SEI message [Option 1]

Changes relative to JVET-AG2032 [1].

### Proposed changes to syntax

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_cnt | ue(v) |
| gfv_base_pic_flag /*indicate if current decoded output picture is a base picture*/ | u(1) |
| if( gfv_base_pic_flag ) { /*specify TranslatorNN( )*/ |  |
| gfv_nn_present_flag | u(1) |
| … |  |
| } else /* current decoded output picture is a driving picture*/ |  |
| gfv_drive_pic_fusion_flag /*indicate if DrivePicture is input to GenerativeNN( )*/ | ue(v) |
| gfv_low_confidence_face_parameter_flag | u(1) |
| … |  |
| } |  |

### Proposed changes to semantics

gfv_low_confidence_face_parameter_flag equal to 1 indicates the facial parameters have been derived with low confidence. gfv_low_confidence_face_parameter_flag equal to 0 indicates the confidence information of the facial parameters is not specified.

## Proposed changes to GFV SEI message [Option 2]

Changes relative to JVET-AG2032 [1].

### Proposed changes to syntax

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_cnt | ue(v) |
| gfv_base_pic_flag /*indicate if current decoded output picture is a base picture*/ | u(1) |
| if( gfv_base_pic_flag ) { /*specify TranslatorNN( )*/ |  |
| gfv_nn_present_flag | u(1) |
| … |  |
| } else /* current decoded output picture is a driving picture*/ |  |
| gfv_drive_pic_fusion_flag /*indicate if DrivePicture is input to GenerativeNN( )*/ | ue(v) |
| gfv_coordinate_present_flag | u(1) |
| if( gfv_coordinate_present_flag ) { |  |
| gfv_low_confidence_coordinate_flag | u(1) |
| gfv_coordinate_precision_factor_minus1 | ue(v) |
| … |  |
| } |  |
| gfv_matrix_present_flag | u(1) |
| if(gfv_matrix_present_flag ) { |  |
| gfv_matrix_element_precision_factor_minus1 | ue(v) |
| gfv_num_matrix_types_minus1 | ue(v) |
| if( !gfv_base_pic_flag ) |  |
| gfv_matrix_pred_flag | u(1) |
| for( i = 0; i <= num_matrix_types_minus1; i++ ) { |  |
| gfv_matrix_type_idx[ i ] | u(6) |
| gfv_low_confidence_matrix_flag[ i ] | u(1) |
| … |  |
| } |  |
| } |  |
| … |  |
| } |  |

### Proposed changes to semantics

gfv_low_confidence_coordinate_flag equal to 1 indicates the coordinates of keypoints have been derived with low confidence. gfv_low_confidence_coordinate_flag equal to 0 indicates the confidence information of the coordinates of keypoints is not specified.

gfv_low_confidence_matrix_flag[ i ] equal to 1 indicates the matrix parameters for the i-th matrix type have been derived with low confidence. gfv_low_confidence_matrix_flag[ i ] equal to 0 indicates the confidence information of matrix parameters for the i-th matrix type is not specified.

# Aspect 2: Copy facial parameters flag

## Problem Statement

In the generative face video SEI message of JVET-AG2032, the facial parameters need to be sent every time there is a GFV SEI. In the case when facial parameters are duplicated for more than one frame, current GFV SEI can only signal the facial parameters multiple times. This causes signalling of unnecessary data.

## Proposal

It is proposed to incorporate a flag to indicate the copying of facial parameters. With this flag, the encoder just needs to set the flag to 1 to inform the decoder to re-use previously decoded facial parameters.

For example, in error concealment cases, the encoder fails to detect a face resulting in missing attributes. Here, the encoder can just set the flag to 1 to inform the decoder to re-use facial parameters from previously decoded GFV SEI message.

In another example, if the encoder detects similar facial parameters in the next picture, it can choose to set the flag to 1 in the next GFV SEI message to skip sending the facial parameters and inform decoder to re-use the facial parameters from the last decoded GFV SEI message.

## Proposed changes to GFV SEI message

Changes relative to JVET-AG2032 [1].

### Proposed changes to syntax

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_cnt | ue(v) |
| gfv_base_pic_flag /*indicate if current decoded output picture is a base picture*/ | u(1) |
| … |  |
| gfv_coordinate_present_flag | u(1) |
| if( gfv_coordinate_present_flag ) { |  |
| gfv_copy_coordinate_flag | u(1) |
| if (! gfv_copy_coordinate_flag) { /* decode facial parameters */ |  |
| gfv_coordinate_precision_factor_minus1 | ue(v) |
| … |  |
| } |  |
| } |  |
| … |  |
| gfv_matrix_present_flag | u(1) |
| if(gfv_matrix_present_flag ) { |  |
| gfv_copy_matrix_flag | u(1) |
| if (! gfv_copy_matrix_flag) { /* decode facial parameters */ |  |
| gfv_matrix_element_precision_factor_minus1 |  |
| … |  |
| } |  |
| } |  |
| … |  |
| } |  |

### Proposed changes to semantics

gfv_copy_coordinate_flag equal to 1 indicates to use coordinates of keypoints from previously decoded generative face video SEI message in output order in the CVS. gfv_copy_coordinate_flag equal to 0 indicates not to use coordinates of keypoints from previously decoded generative face video SEI message in output order in the CVS.

gfv_copy_matrix_flag equal to 1 indicates to use matrix parameters from previously decoded generative face video SEI message in output order in the CVS. gfv_copy_matrix_flag equal to 0 indicates not to use matrix parameters from previously decoded generative face video SEI message in output order in the CVS.

It is a requirement of bitstream conformance that when gfv_base_pic_flag is equal to 1 and gfv_coordinate_present_flag is equal to 1, gfv_copy_coordinate_flag shall be equal to 0.

It is a requirement of bitstream conformance that when gfv_base_pic_flag is equal to 1 and gfv_matrix_present_flag is equal to 1, gfv_copy_matrix_flag shall be equal to 0.

It is a requirement of bitstream conformance that when gfv_coordinate_present_flag of previously decoded generative face video SEI message is equal to 0, gfv_copy_coordinate_flag shall be equal to 0.

It is a requirement of bitstream conformance that when gfv_matrix_present_flag of previously decoded generative face video SEI message is equal to 0, gfv_copy_matrix_flag shall be equal to 0.

The following steps will be executed for gfv_copy_coordinate_flag:

if ( gfv_coordinate_present_flag) { 
     if ( gfv_copy_coordinate_flag ) {
         num_kp_minus1 = previous_num_kp_minus1
         for (I = 0; I <= previous_num_kp_minus1; i++) {
	         coordinateX[ i ] = previous_coordinateX[ i ] 
            coordinateY[ i ] = previous_coordinateY[ i ]
            coordinateZ[ i ] = previous_coordinateZ[ i ] 
          }
     }
     else {
         previous_num_kp_minus1 = num_kp_minus1
         for (i = 0; i <= num_kp_minus1; i++) {
	         previous_coordinateX[ i ] = coordinateX[ i ]
            previous_coordinateY[ i ] = coordinateY[ i ]
            previous_coordinateZ[ i ] = coordinateZ[ i ]
          }
     }
}

The following steps will be executed for gfv_copy_matrix_flag:

if ( gfv_matrix_present_flag ) { 
     if ( gfv_copy_matrix_flag ) {
         num_matrix_types_minus1 = previous_num_matrix_types_minus1
         for( i = 0; i <= previous_num_matrix_types_minus1; i++ ) {
              numMatrices[ i ] = previous_numMatrices[ i ]
              for( j = 0; j < previous_numMatrices[ i ]; j++ ) {
                  matrixHeight[ i ] = previous_matrixHeight[ i ]
                  for( k = 0; k < previous_matrixHeight[ i ]; k++ ) {
                      matrixWidth[ i ] = previous_matrixWidth[ i ]
                      for( m = 0; m < previous_matrixWidth[ i ]; m++ ) { 
                          matrixElementVal[ i ][ j][ k ][ m ] = previous_matrixElementVal[ i ][ j][ k ][ m ]
                      }
                  }
               }
           }
      }
      else {
         previous_num_matrix_types_minus1 = num_matrix_types_minus1
         for( i = 0; i <= num_matrix_types_minus1; i++ ) {
              previous_numMatrices[ i ] = numMatrices[ i ]
              for( j = 0; j < numMatrices[ i ]; j++ ) {
                  previous_matrixHeight[ i ] = matrixHeight[ i ]
                  for( k = 0; k < matrixHeight[ i ]; k++ ) {
                      previous_matrixWidth[ i ] = matrixWidth[ i ]
                      for( m = 0; m < matrixWidth[ i ]; m++ ) { 
                          previous_matrixElementVal[ i ][ j][ k ][ m ] = matrixElementVal[ i ][ j][ k ][ m ]
                      }
                  }
               }
           }
      }
}

# Conclusion

This contribution proposes modifications to the GFV SEI message to include flags indicating copying of facial parameters from previously decoded GFV SEI message, and also information related to the confidence of facial parameters.

It is suggested to adopt the proposed modifications into the generative face video SEI message.

# References

- S. McCarthy, J. Chen, S. Deshpande, M. Hannuksela, “Technologies under consideration for future extensions of VSEI (version 3)”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AG2032, 33rd Meeting, by teleconference, 17–26 January 2024.

# Patent rights declaration(s)

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).