| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0056-v2 |
| --- | --- |

| Title: | EE2-4.1: Multiple Transform Set Selection for LFNST/NSPT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Yue Yu Haoping Yu Dong Wang | Tel: Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-4.1. In this test, the multiple transform set selection (MTSS) method for intra blocks coded with LFNST/NSPT is investigated. The focus of this test is to reduce the encoding time while maintaining the coding gain. Three sub-tests are included in the report.

In test_a, MTSS without any restriction is tested.

In test_b, a configurable restriction is tested. An SPS syntax element is signaled to indicate the minimum CU size that MTSS can be applied to. The CU size is measured with number of pixels. In AI configuration, the minimum CU size is set as 256. In RA configuration, the minimum CU size is set as 0, i.e. MTSS can be applied to any CU size.

In test_c, a fixed restriction is tested. MTSS can be applied to CUs with number of pixels equal or larger than 256.

On top of ECM-12.0, simulation results are reported as follows:

test_a (no restriction):

AI {-0.16% Y, 0.10% U, 0.12% V, 105.9% EncT, 100.9% DecT }

RA {*-0.09% Y, -0.01% U, -0.09% V, 101.0% EncT, 99.7% DecT }

* test_a RA results are inferred from test_b RA.

test_b (configurable restriction, AI:256, RA:0):

AI {-0.08% Y, 0.04% U, 0.04% V, 100.8% EncT, 100.5% DecT }

RA {-0.09% Y, -0.01% U, -0.09% V, 101.0% EncT, 99.7% DecT }

test_c (fixed restriction, 256):

AI {-0.08% Y, 0.04% U, 0.04% V, 100.7% EncT, 99.6% DecT}

RA {-0.04% Y, -0.06% U, -0.07% V, 100.4% EncT, 99.9% DecT}

Introduction

The multiple transform set selection (MTSS) method for intra blocks coded with LFNST/NSPT was proposed in JVET-AG0062 [1]. The proposed method was further investigated in EE2.

In the ECM-12.0 [2], the LFNST/NSPT transform set is implicitly decided by an intra prediction mode (IPM). For a block coded with PLANAR, DC or directional modes, the IPM is set as the intra prediction mode itself. For a DIMD coded block, the IPM is set as the best mode derived by the HoG (histogram of gradients) of neighboring reconstructed pixels. For a TIMD coded block, the IPM is set as the best mode derived by the TIMD process. For an SGPM coded block, the IPM is set as the mode corresponding to the split mode of SGPM. For a MIP, EIP or ITMP coded block, the IPM is set as the best mode derived by the HoG of the prediction. EIP (Extrapolation-filter based intra prediction) is adopted in ECM in the JVET-AG meeting.

In the MTSS method proposed in JVET-AG0062, CUs coded with DIMD, TIMD, MIP, SGPM and ITMP modes can choose an alternative LFNST/NSPT transform set. The transform set selection is signaled in the bit-stream. The focus of this EE test is to reduce the encoding time while maintaining the coding gain.

Proposed method

As EIP was adopted in ECM-12.0, it is proposed to apply MTSS also to EIP. The proposed MTSS method allows CUs coded with DIMD, TIMD, SGPM, MIP, EIP and ITMP to select one LFNST/NSPT transform set out of 2 candidate sets. If the current block is coded with DIMD, TIMD, SGPM, MIP, EIP, ITMP and LFNST/NSPT is used, one more bin is employed to indicate whether the 1st or the 2nd candidate transform set is selected.

The 1st candidate transform set remains the same as the current ECM.

To derive the 2nd candidate:

For TIMD, if fusion is applied, the 2nd TIMD IPM is first considered;

For SGPM, the 2 IPMs that SGPM uses are first considered;

For MIP and EIP, PLANAR is first considered;

The 1st and 2nd candidate should be different transform sets. If the 2nd candidate is not yet decided by the above process, it is derived by the HoG of neighboring reconstructed pixels.

To reduce the encoding time, CU size restriction is used for MTSS. The CU size is measured with number of pixels within the CU. MTSS can be applied to CUs with size equal or larger than the threshold.

Test results

EE2-4.1 was implemented on top of the ECM-12.0 software [2] and tested based on the common test condition [3]. Three sub-tests are included in the report.

In test_a, MTSS without any restriction is tested.

In test_b, a configurable restriction is tested. An SPS syntax element is signaled to indicate the minimum CU size that MTSS can be applied to. In AI configuration, the minimum CU size is set as 256. In RA configuration, the minimum CU size is set as 0, i.e. MTSS can be applied to any CU size.

In test_c, a fixed restriction is tested. MTSS can be applied to CUs with number of pixels equal or larger than 256.

The test results are summarized below.

test_a (no restriction):

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.24% | 0.13% | 0.04% | 105.3% | 100.6% | 98.2% | 100.1% |
| Class A2 | -0.13% | 0.09% | 0.05% | 105.7% | 100.8% | 100.6% | 100.1% |
| Class B | -0.16% | 0.09% | 0.14% | 106.5% | 101.2% | 101.1% | 100.1% |
| Class C | -0.11% | 0.08% | 0.25% | 105.7% | 101.1% | 100.1% | 100.1% |
| Class E | -0.18% | 0.14% | 0.07% | 106.2% | 100.5% | 100.1% | 100.0% |
| Overall | -0.16% | 0.10% | 0.12% | 105.9% | 100.9% | 100.1% | 100.1% |
| Class D | -0.15% | 0.08% | 0.03% | 105.6% | 101.0% | 100.2% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over anchor |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | -0.11% | 0.56% | 100.7% | 99.3% | 100.0% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Till V2, RA simulations are not finished. The results can be inferred to be almost the same as test_b RA, because there is only an SPS syntax element difference between them.

test_b (configurable restriction, AI:256, RA:0):

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.20% | 0.15% | 0.00% | 101.2% | 99.8% | 98.7% | 100.1% |
| Class A2 | -0.09% | 0.06% | 0.09% | 100.6% | 100.1% | 101.1% | 100.1% |
| Class B | -0.06% | 0.03% | 0.01% | 100.9% | 100.8% | 100.1% | 100.1% |
| Class C | -0.01% | 0.05% | 0.08% | 100.3% | 101.2% | 100.1% | 100.0% |
| Class E | -0.05% | -0.07% | 0.02% | 101.0% | 100.4% | 100.1% | 100.0% |
| Overall | -0.08% | 0.04% | 0.04% | 100.8% | 100.5% | 100.0% | 100.1% |
| Class D | -0.03% | 0.18% | 0.03% | 98.6% | 100.0% | 100.1% | 100.0% |
| Class F | 0.00% | 0.14% | 0.11% | 99.2% | 98.6% | 100.1% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.20% | -0.20% | -0.11% | 101.7% | 100.0% | 100.0% | 100.1% |
| Class A2 | -0.07% | -0.03% | -0.09% | 100.8% | 100.6% | 99.9% | 99.9% |
| Class B | -0.09% | 0.20% | -0.16% | 101.3% | 99.3% | 100.1% | 99.9% |
| Class C | -0.01% | -0.11% | 0.00% | 100.4% | 99.4% | 99.9% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.09% | -0.01% | -0.09% | 101.0% | 99.7% | 100.0% | 100.0% |
| Class D | 0.00% | -0.11% | 0.56% | 100.4% | 99.8% | 99.9% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

test_c (fixed restriction, 256):

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.20% | 0.14% | 0.00% | 100.9% | 99.9% | 98.8% | 100.2% |
| Class A2 | -0.09% | 0.06% | 0.09% | 100.6% | 99.5% | 101.5% | 100.2% |
| Class B | -0.06% | 0.03% | 0.01% | 101.2% | 100.3% | 99.4% | 100.2% |
| Class C | -0.02% | 0.04% | 0.08% | 99.2% | 98.7% | 100.2% | 100.1% |
| Class E | -0.05% | -0.07% | 0.02% | 101.6% | 99.5% | 100.1% | 100.1% |
| Overall | -0.08% | 0.04% | 0.04% | 100.7% | 99.6% | 99.9% | 100.2% |
| Class D | -0.03% | 0.18% | 0.03% | 101.6% | 101.7% | 100.2% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.13% | -0.16% | -0.18% | 100.6% | 99.7% | 100.1% | 100.3% |
| Class A2 | -0.03% | 0.01% | 0.02% | 100.0% | 100.3% | 100.1% | 100.4% |
| Class B | -0.04% | -0.09% | -0.11% | 100.5% | 99.6% | 100.1% | 100.1% |
| Class C | 0.01% | 0.00% | -0.02% | 100.2% | 99.9% | 100.0% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.06% | -0.07% | 100.4% | 99.9% | 100.1% | 100.2% |
| Class D | -0.02% | 0.12% | 0.24% | 99.8% | 99.7% | 100.2% | 100.1% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Conclusions

It is recommended to adopt EE2-4.1 test_b or test_c.

References

- F. Wang, Y. Yue, H. Yu, D. Wang, “Non-EE2: Multiple Transform Set Selection for LFNST/NSPT”, JVET-AG0062, January 2024

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October 2023

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).