| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0057-v1 |
| --- | --- |

| Title: | EE2-5.1: Adaptive precision for CCALF coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Song, Yue Yu, Haoping Yu, Dong Wang Nan Hu, Marta Karczewicz, Vadim Seregin, Hongtao Wang 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | songnan@oppo.com nanh@qti.qualcomm.com martak@qti.qualcomm.com vseregin@qti.qualcomm.com hongtaow@qti.qualcomm.com |
| Source: | OPPO and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In Cross Component Adaptive Loop Filter (CCALF) of ECM-12.0, 7 bits are used to represent the fractional part of a CCALF coefficient and each coefficient can only be a value in the form of power of 2. In this contribution, the results of EE2-5.1 are reported, which investigates the impact of using an adaptive number of bits to represent the fractional part of a CCALF coefficient and removing the power of two constraint for CCALF coefficients. In test EE2-5.1a, the number of bits used to represent the fractional part of a CCALF coefficient is adaptively selected from the range of 7 to 10, inclusively. In test EE2-5.1b, the number of bits used to represent the fractional part of a CCALF coefficient is adaptively selected from the range of 5 to 8, inclusively. In test EE2-5.1c, the power of 2 constraint is removed. In test EE2-5.1d, the combination of EE2-5.1a and EE2-5.1c is tested. In test EE2-5.1e, the combination of EE2-5.1b and EE2-5.1c is tested. On top of ECM-12.0 with the common test conditions, simulation results of the proposed methods are reported as follows.

Test EE2-5.1a:

|  | BD-rate Y | BD-rate U | BD-rate V | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra |  |  |  |  |  |  |  |
| Random access |  |  |  |  |  |  |  |
| Low delay B |  |  |  |  |  |  |  |

Test EE2-5.1b:

|  | BD-rate Y | BD-rate U | BD-rate V | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra | 0.00% | -0.09% | -0.13% | 99.6% | 99.7% | 99.7% | 100.0% |
| Random access |  |  |  |  |  |  |  |
| Low delay B |  |  |  |  |  |  |  |

Test EE2-5.1c:

|  | BD-rate Y | BD-rate U | BD-rate V | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra |  |  |  |  |  |  |  |
| Random access |  |  |  |  |  |  |  |
| Low delay B |  |  |  |  |  |  |  |

Test EE2-5.1d:

|  | BD-rate Y | BD-rate U | BD-rate V | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra |  |  |  |  |  |  |  |
| Random access |  |  |  |  |  |  |  |
| Low delay B |  |  |  |  |  |  |  |

Test EE2-5.1e:

|  | BD-rate Y | BD-rate U | BD-rate V | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra | -0.01% | -0.19% | -0.20% | 99.9% | 101.1% | 100.1% | 100.0% |
| Random access |  |  |  |  |  |  |  |
| Low delay B |  |  |  |  |  |  |  |

# Introduction

In ECM-12.0, a CCALF coefficient is represented by a fixed-point 8-bit decimal number. The fractional part is represented by a 7-bit number and 1 bit is used for the sign. In addition, a CCALF coefficient must be a value in the form of a power of two.

In JVET-AG0065 and JVET-AG0233, the number of bits used to represent the fractional part of a CCALF coefficient is adaptively selected from the range of 7 to 10, inclusively. In addition, the power of 2 constraint is removed.

# Proposed method

EE2-5.1 investigates the impact of using an adaptive number of bits to represent the fractional part of a CCALF coefficient and removing the constraint power of 2 for a CCALF coefficient. In test EE2-5.1a, the number of bits used to represent the fractional part of a CCALF coefficient is adaptively selected from the range of 7 to 10, inclusively. In test EE2-5.1b, the number of bits used to represent the fractional part of a CCALF coefficient is adaptively selected from the range of 5 to 8, inclusively. In test EE2-5.1c, the power of 2 constraint is removed. In test EE2-5.1d, a combination of EE2-5.1a and EE2-5.1c is tested. In test EE2-5.1e, a combination of EE2-5.1b and EE2-5.1c is tested.

# Simulation results

The proposed method was implemented on top of ECM-12.0 and tested following the common test conditions described in JVET-AF2017. Tables 1~ 5 report the simulation results of tests EE2-5.1a ~ 5.1e, respectively.

Table 1. Results of test EE2-5.1a.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.00% | -0.26% | -0.40% | 100.2% | 100.4% | 100.1% | 99.8% |
| Class C | 0.01% | -0.14% | -0.26% | 99.7% | 100.3% | 100.0% | 100.0% |
| Class E | 0.01% | -0.32% | -0.27% | 100.5% | 99.3% | 100.0% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.01% | -0.20% | -0.41% | 99.8% | 100.1% | 99.8% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | -0.32% | -0.61% | 100.5% | 100.1% | 100.2% | 100.1% |
| Class C | 0.02% | -0.14% | -0.29% | 99.6% | 99.8% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.01% | -0.48% | -0.83% | 99.9% | 99.9% | 100.0% | 100.0% |
| Class F | 0.01% | -0.38% | -0.58% | 100.2% | 99.6% | 100.1% | 99.9% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.04% | -0.36% | -0.14% | 100.0% | 99.9% | 99.5% | 99.8% |
| Class E | -0.03% | -1.24% | -0.98% | 100.5% | 100.1% | 97.9% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F | -0.10% | -0.49% | -0.70% | 100.2% | 100.3% | 99.7% | 99.9% |
| Class TGM |  |  |  |  |  |  |  |

Table 2. Results of test EE2-5.1b.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.00% | -0.06% | 99.5% | 100.3% | 100.0% | 100.0% |
| Class A2 | 0.00% | -0.02% | 0.02% | 99.7% | 99.7% | 100.0% | 100.0% |
| Class B | 0.00% | -0.09% | -0.22% | 99.5% | 99.5% | 99.0% | 100.0% |
| Class C | 0.00% | -0.07% | -0.13% | 100.0% | 98.8% | 100.0% | 100.0% |
| Class E | 0.00% | -0.28% | -0.23% | 99.0% | 100.4% | 100.0% | 100.0% |
| Overall | 0.00% | -0.09% | -0.13% | 99.6% | 99.7% | 99.7% | 100.0% |
| Class D | 0.01% | -0.04% | -0.32% | 99.5% | 98.1% | 99.9% | 100.0% |
| Class F | -0.01% | -0.19% | -0.23% | 97.8% | 99.3% | 100.0% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.01% | -0.08% | -0.13% | 99.4% | 100.0% | 100.1% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.01% | -0.19% | -0.58% | 99.6% | 100.5% | 99.9% | 100.0% |
| Class F | -0.01% | -0.18% | -0.23% | 99.1% | 99.6% | 99.9% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.00% | -0.11% | 0.11% | 99.5% | 101.0% | 101.8% | 100.1% |
| Class E | 0.12% | -0.46% | -1.09% | 99.3% | 101.2% | 99.9% | 100.3% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.11% | -0.09% | -0.82% | 98.5% | 100.7% | 99.8% | 99.9% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 3. Results of test EE2-5.1c.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | -0.14% | -0.27% | 99.7% | 99.8% | 99.4% | 99.9% |
| Class C | -0.01% | -0.24% | -0.19% | 99.9% | 100.1% | 100.0% | 100.0% |
| Class E | -0.01% | -0.18% | -0.61% | 99.4% | 100.3% | 100.0% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.02% | -0.76% | -0.64% | 100.3% | 99.7% | 99.8% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.02% | -0.17% | -0.27% | 100.1% | 99.9% | 100.0% | 100.2% |
| Class C | 0.03% | -0.34% | -0.22% | 100.3% | 99.7% | 100.1% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.01% | -0.69% | -0.49% | 99.6% | 100.4% | 100.0% | 100.0% |
| Class F | 0.00% | -0.17% | -0.19% | 100.3% | 99.9% | 99.8% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.09% | -0.22% | 0.48% | 100.0% | 100.3% | 100.3% | 100.0% |
| Class E | 0.05% | -0.15% | -1.11% | 100.1% | 99.9% | 97.0% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | -0.80% | -0.97% | 100.1% | 99.5% | 100.0% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 4. Results of test EE2-5.1d.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.00% | -0.34% | -0.36% | 100.2% | 99.7% | 100.0% | 100.0% |
| Class E | 0.00% | -0.32% | -0.70% | 99.5% | 99.4% | 100.0% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.03% | -0.90% | -0.83% | 100.3% | 100.4% | 99.8% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.04% | -0.50% | -0.44% | 99.8% | 99.8% | 100.2% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.01% | -1.00% | -1.14% | 100.5% | 99.6% | 100.0% | 100.0% |
| Class F | 0.04% | -0.68% | -0.67% | 99.9% | 99.8% | 100.0% | 100.1% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.08% | -0.24% | -0.09% | 100.2% | 99.8% | 98.6% | 99.8% |
| Class E | 0.00% | -0.30% | -1.71% | 99.9% | 100.5% | 98.0% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.07% | -0.67% | -0.48% | 99.8% | 99.5% | 100.5% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 5. Results of test EE2-5.1e.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.16% | -0.04% | 99.6% | 101.1% | 100.3% | 100.0% |
| Class A2 | -0.01% | 0.03% | 0.19% | 99.8% | 100.6% | 100.0% | 100.0% |
| Class B | -0.01% | -0.16% | -0.28% | 99.9% | 100.9% | 100.0% | 100.0% |
| Class C | -0.01% | -0.24% | -0.24% | 99.2% | 100.5% | 100.0% | 100.0% |
| Class E | -0.01% | -0.41% | -0.56% | 101.1% | 102.6% | 100.0% | 100.0% |
| Overall | -0.01% | -0.19% | -0.20% | 99.9% | 101.1% | 100.1% | 100.0% |
| Class D | 0.02% | -0.65% | -0.81% | 99.7% | 98.8% | 100.2% | 100.0% |
| Class F | -0.02% | -0.21% | -0.14% | 101.4% | 101.8% | 100.0% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.01% | -0.20% | -0.39% | 99.7% | 99.7% | 99.7% | 100.0% |
| Class C | 0.02% | -0.37% | -0.41% | 100.1% | 99.5% | 99.6% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.04% | -0.02% | -0.21% | 99.4% | 98.2% | 99.2% | 100.2% |
| Class E | 0.09% | -0.38% | -0.80% | 99.0% | 99.7% | 100.0% | 100.6% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.04% | -1.16% | -0.76% | 101.2% | 101.1% | 100.1% | 99.9% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Conclusion

This contribution reports the results of EE2-5.1. It is recommended to adopt Test 5.1a.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).