| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0059-v2 |
| --- | --- |

| Title: | AHG12: Temporal BV for IBC merge/AMVP list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu, Yue Yu, Haoping Yu, Dong Wang (OPPO) Na Zhang, Kai Zhang, Li Zhang (Bytedance Inc.) | Tel: Email: | xuluhang@oppo.com |
| Source: | OPPO Bytedance Inc. |  |  |

_____________________________

- Abstract

This proposal adds some temporal BV candidates to the IBC merge/AMVP list after the non-adjacent BV candidates. The proposed method is implemented on top of the ECM-12.0 reference software. The simulation results are reported as follows:

RA: Class F -0.03%/-0.01%/-0.14%; Class TGM -0.58%/-0.52%/-0.66%.

LB: Class F -0.xx%/-0.xx%/-0.xx%; Class TGM -0.xx%/-0.xx%/-0.xx%.

Introduction

In ECM, the IBC merge/AMVP list is constructed from adjacent spatial candidates, non-adjacent spatial candidates, HBVP candidates, ARBVP candidates, pairwise average candidates, and some predefined BV candidates. However, temporal BV candidates are not utilized. On the other hand, the temporal motion vector prediction (TMVP) is utilized for the inter merge/AMVP mode. In JVET-AD0095 [1], an EE test was conducted to test adding some temporal BV candidates to the IBC merge/AMVP list. In this proposal, more temporal positions are proposed to improve the coding performance of the IBC merge/AMVP mode.

Proposed methods

In this contribution, temporal BV candidates are added to the IBC merge/AMVP list after the non-adjacent BV candidates. The temporal BVs are derived from collocated positions and shifted collocated positions in the collocated pictures. The IBC merge/AMVP list size, the number of applying template matching process and the number of full RDO processes are kept unchanged in the proposed method.

Collocated positions

There are five collocated positions defined in the proposed method. The temporal BVs can be obtained from the collocated positions in the collocated pictures.

Figure-1. Five collocated positions.

Shifted collocated positions

Offsets are used as guidance to change the collocated positions to form the shifted collocated positions. The temporal BVs can be obtained from the shifted collocated positions in the collocated pictures. The offsets can be obtained by:

- Fetching a MV pointing to a collocated picture from one of the five neighboring positions in Figure-2 and using the MV as an offset of the collocated positions for the collocated picture,

- Fetching a MV pointing to a reference picture from one of the five neighboring positions in Figure-2, scaling the MV to point to the collocated picture based on the POC distance between the reference picture and the current picture and the POC distance between the collocated picture and the current picture , and using the scaled MV as an offset of the collocated positions for the collocated picture,

- Fetching a MV pointing to a collocated picture from one of the five neighboring positions in Figure-2 and calculating a mirrored MV pointing to  the other collocated picture. Then using the mirrored MV as an offset of the collocated positions for the other collocated picture.

Figure-2. Five neighboring positions.

Collocated pictures

Two collocated pictures are utilized which are the two reference pictures with the first two least POC distances relative to the current picture, which is the same as that of TMVP.

Experimental Results

The proposed method has been implemented on top of ECM-12.0 [2]. Testing results of the proposed method under the RA/LB configurations are summarized in Table 1. The simulation times are not accurate.

Table 1: Coding performance of the proposed method.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.03% | -0.01% | -0.14% | 102.5% | 104.4% | 99.7% | 100.1% |
| Class TGM | -0.58% | -0.52% | -0.66% | 100.5% | 101.1% | 99.7% | 99.8% |

- Conclusions

In this contribution, the temporal BV candidates are proposed to be added to the IBC merge/AMVP list. The proposed method can bring some coding gain with negligible complexity changes.. It is recommended to study this method in the next round of EE.

References

[1] N. Zhang, K. Zhang, L. Zhang, “EE2-3.3: Temporal block vector prediction,” JVET-AD0095, Apr. 2023.

[2] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-12.0

- Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).