| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0061-v1 |
| --- | --- |

| Title: | Non-EE2: CCALF with Chroma SAO input |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhenyu Dai Nan Song Yue Yu Haoping Yu Dong Wang | Tel: Email: | daizhenyu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes to use chroma SAO’s output with new filter taps in CCALF. The proposed method is implemented on top of ECM-12.0, and the coding performance is reported as follows:

AI: { x.xx % Y, x.xx % U, x.xx % V, x% EncT, x% DecT }

RA: { x.xx % Y, x.xx % U, x.xx % V, x% EncT, x% DecT }

Introduction

In the current ECM-12.0 design, an online-trained CCALF filter is constructed by 23 luma spatial reconstruction taps and 5 luma residual taps, which is illustrated in Figure 1.

Fig. 1: The currently used filter shape of CCALF

Proposed method

In the proposed method, 4 chroma reconstruction taps in a 3x3 cross shape are added for CCALF, as shown in Figure 2. The extended taps take the co-located and neighbouring chroma samples from the SAO’s output as input. All the current luma spatial reconstruction taps and luma residual taps in CCALF are kept unchanged. The coefficient signaling mechanism is the same as the current design in ECM.

Fig. 2: The newly introduced filter shape for CCALF

Experiment results

The proposed method is implemented on top of ECM-12.0[1]. The simulations were carried out by following the ECM common test conditions[2].

Table 1. The proposed method on top of ECM-12.0 (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -1.37% | -1.19% |  |  |
| Class C | 0.01% | -0.24% | -0.17% |  |  |
| Class E | 0.04% | -0.43% | -1.56% |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.01% | -0.18% | -0.19% |  |  |
| Class F | 0.03% | -0.50% | -0.41% |  |  |

Table 2. The proposed method on top of ECM-12.0 (RA)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.03% | -0.22% | -0.17% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.02% | -0.41% | -0.24% |  |  |
| Class F |  |  |  |  |  |

Conclusions

This contribution proposes to use the chroma SAO’s output with new filter taps in CCALF. Simulation results show that the proposed method can improve the coding efficiency with minor complexity increases. It is suggested to further study this method in the next round of EE2.

References

- ECM12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0.

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October, 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).