| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0065-v1 |
| --- | --- |

| Title: | EE2-2.7: Relaxing line buffer restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Kai Zhang, Li Zhang | Email: | {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results for EE2-2.7 which proposes relaxing the top line of CTU restriction to align with the principle of other coding tools in ECM that can access more than one line of reconstructed reference samples for CUs on the CTU top boundary. The impact on coding efficiency and running time of the tests over ECM-12.0 are reported as below for {Y, U, V, EncT, DecT}:

AI(Y/U/V): -0.03%/-0.03%/-0.01%, EncT 100.3%, DecT 99.7%

RA(Y/U/V): -0.02%/0.03%/-0.05%, EncT 99.9% , DecT 100.0%

# Introduction

In ECM-12.0, coding tools such as CCCM, GLM, DIMD template, TIMD template, etc. can access more than one line of reconstructed reference samples for CUs on CTU top boundary, but a few coding tools still follow the top line of CTU restriction as in VVC, as listed in Table 1.

Table 1. The line buffer requirement of coding tools in ECM-12.0

| Coding methods in ECM-12.0 | Require more than one line buffer? |
| --- | --- |
| CCCM/NS-CCCM/GL-CCCM/MDF-CCCM | Yes |
| GLM with gradients and downsampled luma | Yes |
| DIMD template | Yes |
| TIMD template | Yes |
| Template usage such as interTM merge | Yes |
| Intra MPM: non-adjacent spatial candidates | Yes |
| Inter/IBC/affine motion: non-adjacent spatial candidates | Yes |
| Intra/Inter CCP merge: non-adjacent spatial candidates | Yes |
| intraTMP/IBC search region | Yes |
| Extended MRL | No |
| Template based MRL | No |
| Intra luma fusion | No |

# Proposed Method

In JVET-AG0120 [1], the top line of CTU restriction is relaxed for relevant coding tools in ECM. Specifically, the MRL related tools and intra luma fusion can access more than one line of reconstructed reference samples for CUs on CTU top boundary. The method of relaxing the line buffer restriction was studied in EE2 as Test 2.7.

# Experimental results

The proposed method is implemented on top of ECM-12.0 [2] and tested based on the common test conditions [3]. The simulation results of EE2-2.7 are summarized in the following Table.

Table 2: Coding performance of EE2-2.7

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.03% | 0.12% | 100.8% | 99.9% | 99.4% | 100.0% |
| Class A2 | -0.02% | 0.06% | 0.01% | 100.0% | 99.9% | 101.1% | 100.0% |
| Class B | -0.04% | -0.08% | -0.06% | 100.2% | 99.6% | 100.9% | 100.0% |
| Class C | -0.03% | -0.07% | 0.02% | 100.4% | 99.5% | 100.9% | 100.0% |
| Class E | -0.03% | -0.05% | -0.09% | 100.4% | 99.7% | 98.8% | 100.0% |
| Overall | -0.03% | -0.03% | -0.01% | 100.3% | 99.7% | 100.4% | 100.0% |
| Class D | -0.03% | 0.21% | -0.10% | 100.2% | 99.5% | 100.3% | 100.0% |
| Class F | -0.02% | -0.04% | -0.07% | 100.1% | 100.3% | 101.2% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.04% | -0.14% | 99.9% | 100.3% | 100.1% | 100.1% |
| Class A2 | 0.00% | 0.05% | 0.04% | 99.1% | 100.1% | 99.8% | 100.2% |
| Class B | -0.02% | 0.06% | -0.13% | 100.4% | 99.9% | 100.0% | 99.8% |
| Class C | 0.00% | 0.02% | 0.05% | 100.0% | 99.7% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | 0.03% | -0.05% | 99.9% | 100.0% | 100.0% | 100.0% |
| Class D | 0.03% | -0.01% | 0.25% | 99.8% | 98.9% | 100.3% | 100.0% |
| Class F | -0.02% | 0.04% | 0.04% | 100.6% | 100.1% | 100.0% | 99.8% |

# Conclusions

This contribution reports results of EE2-2.7 which proposes improvements to the intra prediction. The results show that the gain of the intra prediction can be further improved. It is suggested to relax the line buffer restriction in ECM.

# References

- Z. Deng, K. Zhang, L. Zhang, “Non-EE2: On line buffer restriction”, JVET-AG0120, Jan. 2024.

- ECM12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct, 2023.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).