| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0066-v2 |
| --- | --- |

| Title: | EE2-3.3: Inter CCP merge mode with zero luma CBF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Kai Zhang, Li Zhang | Email: | {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results for EE2-3.3 which was proposed in JVET-AG0200 [1]. In this test, inter CCP merge mode is extended to zero luma CBF. A CU-level syntax is signalled to indicate this mode when merge skip mode is applied, and when root CBF is equal to 0. The impact on coding efficiency and runtimes of the tests over ECM-12.0 are reported as below {for Y, U, V, EncT, DecT}:

Overall:

RA(Y/U/V): 0.00%/0.00%/0.00%, EncT 99.7%, DecT 99.8%

LDB(Y/U/V): 0.00%/0.00%/0.00%, EncT 100.4%, DecT 100.3%

Class F:

RA(Y/U/V): 0.01%/-0.04%/-0.22%, EncT 100.2%, DecT 100.1%

LDB(Y/U/V): -0.01%/-0.34%/-0.59%, EncT 100.3%, DecT 99.6%

Class TGM:

RA(Y/U/V): -1.59%/-4.15%/-4.05%, EncT 100.5%, DecT 99.2%

LDB(Y/U/V): -0.93%/-5.30%/-5.31%, EncT 100.4%, DecT 98.7%

# Introduction

The inter CCP merge mode in ECM-12.0 blends the inter motion-compensated chroma prediction with a CCP model estimated chroma prediction, and the CCP model can be either calculated from adjacent neighboring samples or inherited from a previous CCP coded block. The inter CCP merge mode follows the same restriction of inter CCCM mode that requires the luma CBF to be non-zero.

In JVET-AG0200 [1], it was proposed to allow the inter CCP merge mode to a CU even if the luma CBF is equal to 0. As such, the inter CCP merge mode flag can be signalled when the luma CBF or the rootCBF is equal to 0.

# Proposed Method

In this test, the inter CCP merge mode is extended to CUs with zero luma CBFs.

Specifically, if the root CBF is equal to 0, a CU level flag is signalled to indicate the usage of inter CCP merge mode. Otherwise, a TU level flag is signalled as in ECM-12.0. Whether to allow the proposed method for inter AMVP-coded blocks or inter MERGE-coded blocks with W×H less than 16 or greater than 1024 is determined based on the selection ratio of blocks coded in the inter CCP merge mode in the previous coded picture. The encoder may also disable the proposed method for the trade-off purpose.

# Experimental results

The proposed method is implemented on top of ECM-12.0 [3] and tested based on the common test conditions [4]. The simulation results of EE2-3.3 is summarized in the following Table.

Table 1: Coding performance of EE2-3.3

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.00% | 0.00% | 100.1% | 100.3% | 100.0% | 100.0% |
| Class A2 | 0.00% | 0.00% | 0.00% | 98.7% | 99.1% | 99.3% | 100.1% |
| Class B | 0.00% | 0.00% | 0.00% | 100.1% | 99.9% | 100.0% | 99.9% |
| Class C | 0.00% | 0.00% | 0.00% | 99.8% | 99.7% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99.7% | 99.8% | 99.9% | 100.0% |
| Class D | 0.00% | 0.00% | 0.00% | 100.0% | 99.4% | 99.9% | 100.0% |
| Class F | 0.01% | -0.04% | -0.22% | 100.2% | 100.1% | 100.1% | 99.8% |
| Class TGM | -1.59% | -4.15% | -4.05% | 100.5% | 99.2% | 99.9% | 100.7% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100.1% | 100.1% | 99.8% | 99.8% |
| Class C | 0.00% | 0.00% | 0.00% | 101.1% | 100.8% | 100.0% | 99.8% |
| Class E | 0.00% | 0.00% | 0.00% | 99.9% | 100.0% | 99.8% | 100.1% |
| Overall | 0.00% | 0.00% | 0.00% | 100.4% | 100.3% | 99.9% | 99.8% |
| Class D | 0.00% | 0.00% | 0.00% | 100.3% | 100.2% | 99.8% | 100.1% |
| Class F | -0.01% | -0.34% | -0.59% | 100.3% | 99.6% | 98.3% | 99.4% |
| Class TGM | -0.93% | -5.30% | -5.31% | 100.4% | 98.7% | 100.4% | 98.6% |

# Conclusions

This contribution reports results of EE2-3.3 which proposes improvements to the inter CCP merge mode. The results show that the gain of the inter CCP merge mode in SCC sequences can be further improved. It is suggested to adopt the proposed method to ECM.

# References

- Z. Deng, K. Zhang, L. Zhang, “Non-EE2: Inter CCP merge mode with zero luma CBF”, JVET-AG0200, Jan. 2024.

- ECM12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct, 2023.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).