| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0047-v3 |
| --- | --- |

| Title: | AHG15: ECM-12.0 Tool assessment on class G candidate sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sebastian Schwarz, Miska M. Hannuksela Nokia Technologies Munich, Germany | Email: | {sebastian.schwarz, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

At the 33rd JVET meeting in January 2024, AHG15 collected several candidate sequences for gaming content compression. To support further evaluation, this contribution provides ECM-12.0 tool assessment results following the latest AHG7 configurations. Based on the tool assessment, it is proposed to remove the following sequences from the gaming content candidate sequences: DesertTown1, SunTemple2, Baolei-Man, Baolei-Baloon Baolei-Woman, and Jianling-Temple. It is asserted that these six sequences have similar counterparts more sensitive to ECM coding tools still present in the list of candidate sequences; thus, their removal should have little effect on the coverage of use cases.

# Introduction

AHG15 selected 27 sequences for further evaluation. The sequences are grouped in two groups, G1 with 7 sequences and G2 with 20 sequences. For the ECM-12.0 tool assessment, all G1 sequences plus 13 G2 sequences were selected (“3GPP SA4 S5 sequences”). For each sequence, the first 64 frames according to [1] were encoded with ECM-12.0 following AHG15 conditions for AI, RA, LDB, and LDP.

Additionally, the same sequences were encoded following the AHG7 tool assessment settings with 5 different tool groups switched off [2]:

- Group 1 includes inter template matching tools.

- Group 2 includes coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process.

- Group 3 includes intra and IBC template matching (with search) related tools.

- Group 4 includes tools that require more processing on the neighboring reconstructed samples than VVC.

- Group 5 includes all the tools in the groups 1 to 4.

# ECM12.0 tool assessment overview

This section summarises the ECM-12.0 tool assessment on class G candidate sequences. Detailed per-sequence results are attached to this contribution.

## Group 1 off

For group 1, RA and LDB configurations had some issues running the decoder. We will provide the corrected results once available.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -0.01% | -0.01% | -0.01% | 99.77% | 99.91% | 100.0% |
| Class G2 | -0.01% | -0.01% | -0.01% | 99.79% | 99.87% | 100.00% |
| Overall | -0.01% | -0.01% | -0.01% | 99.78% | 99.89% | 100.00% |
| JVET CTC* | -0.01% | -0.01% | -0.01% | 99.9% | 99.9% | 100% |
|  | Random Access Main 10** |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 4.41% | 3.68% | 4.59% | 84.38% |  |  |
| Class G2 | 3.27% | 3.35% | 3.16% | 83.74% |  |  |
| Overall | 3.67% | 3.47% | 3.66% | 83.97% |  |  |
| JVET CTC* | 4.13% | 4.00% | 4.15% | 84.4% | 74.1% | 100% |
|  | Low delay B Main 10** |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 1.86% | 2.73% | 3.44% | 77.50% |  |  |
| Class G2 | 2.94% | 3.19% | 3.14% | 79.43% |  |  |
| Overall | 2.56% | 3.03% | 3.25% | 78.75% |  |  |
| JVET CTC* | 4.78% | 5.06% | 4.96% | 76.6% | 66.5% | 100% |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 0.73% | 1.92% | 2.81% | 81.87% | 82.46% | 100.2% |
| Class G2 | 2.20% | 2.22% | 2.28% | 82.91% | 77.59% | 100.20% |
| Overall | 1.68% | 2.12% | 2.46% | 82.54% | 79.26% | 100.21% |

* Taken from ECM-12.0 AHG7 report JVET-AH0007 [3].

** For Group 1 off - RA10 and LDB10 we observed decoder errors at several test points (MD5 mismatches, decoder crashes “getGeoBlendCand( mergeIdx=1 ) failed.”).

## Group 2 off

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 0.34% | 0.80% | 0.70% | 92.12% | 94.65% | 99.7% |
| Class G2 | 0.44% | 0.50% | 0.58% | 91.38% | 95.45% | 99.76% |
| Overall | 0.41% | 0.60% | 0.62% | 91.64% | 95.17% | 99.73% |
| JVET CTC* | 0.42% | 0.65% | 0.71% | 92.1% | 95.4% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 3.35% | 0.76% | 2.98% | 85.99% | 100.07% | 104.1% |
| Class G2 | 1.88% | 1.42% | 0.90% | 84.55% | 94.72% | 104.33% |
| Overall | 2.40% | 1.19% | 1.63% | 85.05% | 96.56% | 104.26% |
| JVET CTC* | 2.21% | 1.40% | 1.74% | 83.8% | 96.8% | 99% |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 2.60% | 3.99% | 4.61% | 82.14% | 99.02% | 101.2% |
| Class G2 | 1.90% | 3.40% | 2.80% | 80.20% | 92.98% | 101.87% |
| Overall | 2.15% | 3.61% | 3.44% | 80.87% | 95.05% | 101.64% |
| JVET CTC* | 2.31% | 3.22% | 3.79% | 76.0% | 92.8% | 100% |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 2.63% | 4.51% | 5.08% | 83.05% | 98.90% | 99.7% |
| Class G2 | 1.95% | 3.25% | 3.27% | 79.33% | 90.78% | 100.01% |
| Overall | 2.19% | 3.69% | 3.90% | 80.61% | 93.54% | 99.89% |

* Taken from ECM-12.0 AHG7 report JVET-AH0007 [3].

## Group 3 off

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 2.08% | 4.70% | 4.10% | 75.60% | 76.63% | 99.9% |
| Class G2 | 3.05% | 4.42% | 3.98% | 77.52% | 72.30% | 99.77% |
| Overall | 2.71% | 4.52% | 4.02% | 76.84% | 73.78% | 99.82% |
| JVET CTC* | 2.09% | 4.07% | 4.50% | 74.5% | 77.2% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 1.15% | 4.59% | 4.21% | 96.25% | 99.29% | 100.0% |
| Class G2 | 1.92% | 4.29% | 3.82% | 94.91% | 96.65% | 99.82% |
| Overall | 1.65% | 4.39% | 3.96% | 95.38% | 97.56% | 99.89% |
| JVET CTC* | 1.08% | 2.87% | 3.48% | 93.7% | 98.1% | 100% |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 0.88% | 2.04% | 1.85% | 95.34% | 98.25% | 99.2% |
| Class G2 | 1.80% | 3.45% | 3.69% | 94.39% | 96.98% | 100.18% |
| Overall | 1.47% | 2.95% | 3.05% | 94.73% | 97.42% | 99.83% |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 0.91% | 2.44% | 2.00% | 92.81% | 98.03% | 99.7% |
| Class G2 | 2.04% | 3.93% | 4.40% | 91.49% | 97.57% | 99.93% |
| Overall | 1.64% | 3.41% | 3.56% | 91.95% | 97.73% | 99.86% |

* Taken from ECM-12.0 AHG7 report JVET-AH0007 [3].

## Group 4 off

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 0.69% | 4.11% | 3.62% | 89.86% | 96.97% | 100.0% |
| Class G2 | 0.59% | 2.94% | 2.49% | 86.82% | 96.61% | 99.98% |
| Overall | 0.63% | 3.35% | 2.89% | 87.87% | 96.74% | 100.00% |
| JVET CTC* | 0.86% | 3.58% | 3.48% | 84.9% | 97.1% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 0.25% | 1.71% | 2.07% | 95.93% | 100.36% | 100.2% |
| Class G2 | 0.35% | 2.64% | 1.59% | 94.83% | 98.89% | 100.09% |
| Overall | 0.32% | 2.31% | 1.76% | 95.21% | 99.40% | 100.13% |
| JVET CTC* | 0.41% | 2.03% | 1.97% | 93.4% | 99.9% | 100% |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 0.21% | 1.06% | 0.97% | 98.00% | 99.54% | 99.6% |
| Class G2 | 0.29% | 2.20% | 1.29% | 97.76% | 99.78% | 100.49% |
| Overall | 0.27% | 1.80% | 1.18% | 97.85% | 99.69% | 100.19% |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 0.04% | 1.15% | 0.84% | 97.35% | 99.41% | 100.1% |
| Class G2 | 0.25% | 2.21% | 1.47% | 96.28% | 99.66% | 100.13% |
| Overall | 0.17% | 1.84% | 1.25% | 96.65% | 99.57% | 100.11% |

* Taken from ECM-12.0 AHG7 report JVET-AH0007 [3].

## Group 1-4 off

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 3.92% | 13.39% | 10.76% | 60.13% | 67.38% | 99.6% |
| Class G2 | 5.12% | 9.76% | 8.45% | 59.14% | 63.74% | 99.40% |
| Overall | 4.70% | 11.03% | 9.26% | 59.48% | 64.99% | 99.46% |
| JVET CTC* | 4.83% | 11.14% | 11.39% | 55.0% | 64.7% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 10.76% | 13.37% | 14.40% | 66.06% | 75.01% | 103.6% |
| Class G2 | 8.88% | 12.89% | 12.53% | 62.42% | 70.14% | 103.87% |
| Overall | 9.54% | 13.06% | 13.18% | 63.67% | 71.80% | 103.76% |
| JVET CTC* | 9.26% | 13.19% | 14.95% | 58.8% | 71.1% | 99% |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 7.66% | 12.02% | 14.02% | 60.25% | 70.99% | 100.0% |
| Class G2 | 8.00% | 12.85% | 12.23% | 57.89% | 65.18% | 100.78% |
| Overall | 7.88% | 12.56% | 12.86% | 58.71% | 67.16% | 100.52% |
| JVET CTC* | 8.25% | 13.10% | 13.05% | 49.7% | 61.6% | 100% |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | 7.23% | 13.19% | 13.82% | 61.98% | 73.29% | 99.4% |
| Class G2 | 7.72% | 12.60% | 12.02% | 57.35% | 63.53% | 99.57% |
| Overall | 7.54% | 12.80% | 12.65% | 58.93% | 66.79% | 99.50% |

* Taken from ECM-12.0 AHG7 report JVET-AH0007 [3].

# ECM12.0 tool assessment per-sequence

Per-sequence analysis of the ECM-12.0 tool assessment results identifies which sequences are sensitive to which coding tool groups. We propose to use this information to reduce the number of candidate sequences in each class.

## Class G1 sequences

In class G1 are currently 7 candidate sequences, the table below summarises their individual performance results (AI10, RA10, LDB10) in Luma-BDBR [%].

|  | AI10 |  |  |  |  | RA10 |  |  |  |  | LDB10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | O1 | O2 | O3 | O4 | O14 | O1 | O2 | O3 | O4 | O14 | O1 | O2 | O3 | O4 | O14 |
| APRG | -0.01 | 0.39 | 0.59 | 1.15 | 2.77 | 3.60 | 5.18 | 0.22 | 0.17 | 11.48 | 0.20 | 3.36 | 0.35 | 0.23 | 6.70 |
| DarkTree | 0.00 | 0.20 | 1.23 | 0.47 | 2.17 | 4.76 | 0.84 | 0.71 | 0.21 | 6.68 | 4.94 | 0.99 | 0.83 | 0.72 | 6.60 |
| DesertTown1 | 0.00 | 0.37 | 1.37 | 0.56 | 3.00 | 3.38 | 2.70 | 0.75 | 0.38 | 8.27 | 0.80 | 1.24 | 0.79 | -0.01 | 5.70 |
| DesertTown2 | 0.00 | 0.31 | 1.75 | 0.47 | 3.26 | 4.67 | 4.04 | 1.14 | 0.28 | 11.00 | 1.55 | 1.53 | 0.71 | -0.11 | 6.14 |
| Level1 | 0.00 | 0.29 | 1.90 | 0.73 | 3.89 | 4.36 | 1.10 | 0.81 | 0.27 | 7.79 | 4.30 | 1.58 | 0.66 | 0.12 | 7.45 |
| SunTemple1 | -0.02 | 0.36 | 4.20 | 0.72 | 6.39 | 6.37 | 6.21 | 2.67 | 0.58 | 17.87 | 1.02 | 6.51 | 1.61 | 0.19 | 12.82 |
| SunTemple2 | -0.02 | 0.43 | 3.54 | 0.69 | 5.95 | 3.75 | 3.38 | 1.76 | -0.10 | 12.21 | 0.18 | 2.99 | 1.19 | 0.35 | 8.19 |

From the table above, we would propose to remove sequences DesertTown1 and SunTemple2 from the current set of candidate sequences, reducing the number of sequences in class G1 to 5.

## Class G2 sequences

The table below summarises the individual performance results (AI10, RA10, LDB10) of the 13 evaluated sequences in Luma-BDBR [%].

|  | AI10 |  |  |  |  | RA10 |  |  |  |  | LDB10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | O1 | O2 | O3 | O4 | O14 | O1 | O2 | O3 | O4 | O14 | O1 | O2 | O3 | O4 | O14 |
| AOV | 0.00 | 0.55 | 2.29 | 0.90 | 5.02 | 4.01 | 1.80 | 0.96 | 0.50 | 8.40 | 3.86 | 1.92 | 0.71 | 0.11 | 7.88 |
| Baolei-Balloon* | 0.00 | 0.63 | 1.45 | 0.84 | 3.53 | 4.95 | 3.45 | 0.05 | 0.55 | 12.13 | 4.42 | 2.99 | 0.54 | 0.23 | 10.00 |
| Baolei-Man | 0.00 | 0.56 | 1.70 | 0.85 | 3.81 | 3.09 | 1.56 | 0.54 | 0.40 | 6.70 | 2.04 | 1.40 | 0.47 | 0.08 | 5.63 |
| Baolei-Woman | 0.00 | 0.49 | 1.89 | 0.66 | 4.01 | 3.44 | 4.16 | 0.85 | 0.21 | 9.46 | 2.81 | 4.11 | 0.74 | 0.08 | 8.88 |
| Baolei-Yard* | 0.00 | 0.67 | 2.34 | 0.81 | 5.08 | 4.95 | 4.33 | 1.00 | 0.27 | 12.61 | 3.93 | 4.31 | 1.35 | 0.52 | 11.19 |
| CS-GO | -0.03 | 0.56 | 5.62 | 0.23 | 8.43 | 1.26 | 2.13 | 7.88 | 0.40 | 15.59 | 1.03 | 2.34 | 8.33 | 1.06 | 13.68 |
| Heroes-of-the-storm | 0.00 | 0.39 | 2.70 | 0.68 | 5.01 | 1.53 | 1.15 | 1.76 | 0.44 | 6.27 | 1.97 | 1.45 | 1.78 | 0.34 | 6.32 |
| Jianling-Beach | 0.00 | 0.27 | 2.35 | 0.44 | 3.44 | 2.50 | 1.16 | 1.16 | 0.32 | 5.59 | 2.03 | 1.24 | 0.98 | 0.19 | 5.10 |
| Jianling-Temple | 0.00 | 0.12 | 0.85 | 0.38 | 1.60 | 2.97 | 0.69 | 0.32 | 0.19 | 4.55 | 3.02 | 0.69 | 0.23 | 0.04 | 4.43 |
| MineCraft | 0.00 | 0.39 | 8.13 | 0.37 | 10.34 | 4.70 | 0.73 | 3.62 | 0.09 | 10.69 | 3.77 | 1.08 | 3.07 | 0.23 | 8.86 |
| Project-CARS | -0.01 | 0.40 | 3.80 | 0.60 | 5.67 | 3.99 | 1.33 | 1.90 | 0.28 | 8.31 | 4.20 | 1.58 | 1.38 | 0.14 | 8.18 |
| StarCraft | 0.00 | 0.36 | 3.73 | 0.61 | 5.79 | 4.26 | 1.42 | 2.40 | 0.59 | 9.75 | 3.93 | 1.35 | 1.87 | 0.24 | 8.39 |
| World-of-WarCraft | -0.01 | 0.38 | 2.86 | 0.34 | 4.76 | 0.84 | 0.55 | 2.51 | 0.36 | 5.45 | 1.24 | 0.25 | 1.91 | 0.55 | 5.47 |

* UHD sequence.

From the table above, we would propose to remove sequences Baolei-Man, Baolei-Baloon and Baolei-Woman, as well as Jianling-Temple.

# Summary & Conclusion

This contribution provides ECM-12.0 tool assessment results on 20 gaming content candidate sequences. Based on the per-sequence evaluation, we propose to reduce the current set of candidate sequences by six sequences. These six sequences have similar counterparts more sensitive to ECM coding tools still present in the list of candidate sequences; thus, their removal should have little effect on the coverage of use cases.

# References

[1]	S. Puri, J. Sauer, R. Chernyak, A. Duenas, L. Wang, „JVET AHG report: Gaming content compression (AHG15)”, JVET-AG0015, January 2024.
https://jvet-experts.org/doc_end_user/current_document.php?id=13788

[2]	X. Li, L.-F. Chen, Z. Deng, J. Gan, E. François, H.-J. Jhu, X. Li, H. Wang, „JVET AHG report: ECM tool assessment (AHG7)”, JVET-AG0007, January 2024.
https://jvet-experts.org/doc_end_user/current_document.php?id=13781

[3]	X. Li, L.-F. Chen, Z. Deng, J. Gan, E. François, H.-J. Jhu, X. Li, H. Wang, „JVET AHG report: ECM tool assessment (AHG7)”, JVET-AH0007, April 2024.
https://jvet-experts.org/doc_end_user/current_document.php?id=14050

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).