| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0070-v1 |
| --- | --- |

| Title: | AHG9: On an extension of AI text data SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh, Zheming Fan and Tomohiro Ikai, Sharp Corporation, Luoxu Jin and Hiroshi Watanabe, Waseda University | Email: | chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation and Waseda University |  |  |

_____________________________

- Abstract

Currently, generative AI is one of the most exciting technologies. As a visual quality improvement for decoded video sequences using Stable Diffusion was experimented with, a different result could be obtained compared to conventional methods. This contribution proposes to extend AI text data SEI, which was discussed at the past two meetings. In generative AI represented Stable Diffusion, multiple models with many parameters are combined, and their specifications are fluid. The number of descriptions is not tiny, either. Therefore, this contribution recommends a method of defining them should be identified by URI, which is defined outside the codec. This methodology is the same as that of the NNPFC SEI message, and it is also possible to send related information.

Problem statement

Several studies of picture coding based on generative AI methods [1] [2] are currently underway. Stable Diffusion is one of the most famous generative image methods, and many related tools have been developed. In this contribution, those methods are applied to improve the visual quality of the decoded image. The next page shows examples of experimental results. The test sequence is BlowingBubbles (416x240, 50fps 10sec), encoded and decoded with QP equal to 37 and RA10 condition by VTM-23.1. Figure 1 shows a quadruple upsampling picture using Lanczos 3 kernel, and a quadruple-resolution picture using Stable Diffusion WebUI, including ControlNet and Deforum. Although the signal may differ from the original image, generating a picture different from noise reduction was possible.

The problem is that not only prompts but also many parameters and multiple large sizes of model data are needed for the generative processing. In this experiment, the bitstream size (10sec) is 182KB, and the setting text file size is 10KB. The size of the model (yayoi_mix_v25-fp16) for Stable Diffusion is about 2GB, and the sum size of the two models for ControlNet (control_v11f1e_sd15_tile_fp16 and control_v11p_sd15_lineart_fp16) models is about 1.4GB. It may be too large to indicate that information by only text. There were proposals for generative AI [3] [4] at the past two meetings. Those proposals were SEI messages sending text information for generative AI. Therefore, this contribution recommends that a method of defining them should be identified by URI, which is defined outside the codec. The methodology by URI is the same as that of the NNPFC SEI message. It is also possible to send some related text information.

(a)	Quadruple upsampling picture using Lanczos 3 kernel

(b)	Quadruple resolution picture using Stable Diffusion.

Figure 1:	These pictures are 270th of BlowingBubbles (416x240), encoded and decoded with QP equal to 37 and RA10 condition by VTM-23.1. The picture (a) is upsampled to quadruple resolution using Lanczos 3 kernel by HDRTools. The picture (b) is generated to quadruple resolution (1664x960) using Stable Diffusion Webui (AUTOMATIC1111) version 1.7.0, ControlNet version 1.1.436, and Deforum extension.

Proposal

The parts highlighted in yellow are our proposed specifications.

AI text data SEI message syntax

| ai_text_data( payload_size ) | Descriptor |
| --- | --- |
| ait_data_cancel_flag | u(1) |
| if (!ait_data_cancel_flag) { |  |
| ait_data_persistence_flag | u(1) |
| ait_data_mode_idc | ue(v) |
| while( !byte_aligned( ) ) |  |
| ait_data_alignment_zero_bit_a | u(1) |
| if( ait_data_mode_idc = = 0 ) |  |
| ait_data_string | st(v) |
| else if( ait_data_mode_idc = = 1 ) { |  |
| ait_data_tag_uri | st(v) |
| ait_data_uri | st(v) |
| } |  |
| } |  |
| } |  |

AI text data SEI message semantics

The AI text data SEI message indicates text prompts to guide a generative AI process to modify a video stream.

ait_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous AI text data SEI message in output order. ait_cancel_flag equal to 0 indicates that AI text data information follows.

ait_data_persistence_flag specifies the persistence of the AI text data SEI message for the current layer.

ait_data_persistence_flag equal to 0 specifies that the AI text data SEI message applies to the current decoded picture only.

ait_data_persistence_flag equal to 1 specifies that the AI text data SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an AI text data SEI message is output that follows the current picture in output order.

ait_data_mode_idc, when equal to 0, indicates that ait_data_mode_idc equal to 0 indicates that the generative AI data is identified by text data. ait_data_mode_idc equal to 1 indicates that the generative AI data is identified by the URI indicated by ati_data_uri with the format identified by the tag URI ait_data_tag_uri.

ait_data_string is a text string containing a command prompt to be interpreted by a generative AI engine. The text prompts are encoded as specified by ISO/IEC 10646: Information technology - Universal Coded Character Set (UCS).

ait_data_alignment_zero_bit_a shall be equal to 0.

ait_data_tag_uri contains a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the format and associated information about the generative AI data specified by ait_data_uri.

ait_data_uri contains a URI with syntax and semantics as specified in IETF Internet Standard 66 identifying the generative AI data.

Acknowledgements

These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.

Reference

[1]	H. Watanabe, L. Jin, T. Hayami, T. Chujoh, T. Aono,Y. Yasugi, S. Hong, Z. Fan and T. Ikai, Prompt-based Image Coding with Edge Information, PCSJ/IMPS 2023, P1-12, Nov. 2023.

[2]	L. Jin, T. Hayami, H. Watanabe, T. Chujoh, T. Aono,Y. Yasugi, S. Hong, Z. Fan and T. Ikai, Post-processing Based Image Coding via Stable Diffusion, PCSJ/IMPS 2023, P3-08, Nov. 2023.

[3]	A. Hinds, and S. Wenger, AHG9: SEI message for text for generative AI, JVET-AF0145, Oct. 2023.

[4]	A. Hinds, G. Teniou and S. Wenger, AHG9: Text prompt for generative AI SEI, JVT-AG0167, Jan. 2024.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Waseda University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).