| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0073-v1 |
| --- | --- |

| Title: | EE2-3.4: Adaptive GPM blending |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In EE2 Test-3.4, an adaptive GPM blending width candidate list determination method is presented, where the selection of GPM blending width list is determined based on the block dimension. It is reported that on top of ECM-12.0, simulation results of the proposed method are as below:

RA: -0.02 % (Y), -0.06 % (U), -0.08 % (V), xxx% (EncT), xxx% (DecT)

LB: -0.01 % (Y), -0.11 % (U), -0.14 % (V), xxx% (EncT), xxx% (DecT) .

- Introduction

In ECM-12.0, the prediction samples within a certain region along the GPM split boundary are generated by blending the two prediction signals. The blending width is determined from a predefined set with five fixed candidates, i.e., {τ/4, τ/2, τ, 2τ, 4τ}, and up to 8 samples on each side of the boundary are blended by a weighted sum. A CU level index is signaled to specify the blending width index.

- Proposed method

In EE2 Test-3.4, an adaptive GPM blending method with two alternative blending width candidate lists is investigated. In particular, list #0 ({τ/4, τ/2, τ, 2τ, 4τ}) comprises the same blending candidates as in ECM-12.0, and list #1 ({τ/2, τ, 2τ, 4τ, 8τ}) is designed for larger blocks, where at most 16 samples are blended on each side of the split boundary. The selection of the candidate list is adaptively determined based on the block size. If min(width, height) < 32, list #0 is used. Otherwise list #1 is used.

- Experimental results

The proposed method has been implemented on top of ECM-12.0 [1]. Results under RA/LDB configurations are presented in Table 1. The simulations were performed in accordance with the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method on ECM-12.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | -0.03% | 0.03% |  |  |  |  |
| Class A2 | 0.00% | 0.03% | -0.04% |  |  |  |  |
| Class B | -0.05% | -0.12% | -0.12% |  |  |  |  |
| Class C | 0.01% | -0.07% | -0.12% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.06% | -0.08% |  |  |  |  |
| Class D | -0.01% | 0.13% | 0.07% |  |  |  |  |
| Class F | -0.01% | 0.04% | 0.00% |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | 0.15% | -0.07% |  |  |  |  |
| Class C | 0.01% | 0.00% | -0.08% |  |  |  |  |
| Class E | -0.03% | -0.68% | -0.33% |  |  |  |  |
| Overall | -0.01% | -0.11% | -0.14% |  |  |  |  |
| Class D | -0.17% | 0.19% | -0.13% |  |  |  |  |
| Class F | -0.09% | -0.71% | -0.90% |  |  |  |  |

- Conclusions

An adaptive GPM blending method is presented in this contribution. Simulation results on top of ECM-12.0 indicate 0.02%/0.01% coding gain in RA/LDB configurations without complexity fluctuation. It is recommended to include the proposed method in the next version of ECM.

- References

[1] ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).