| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0074-v2 |
| --- | --- |

| Title: | Objective Results of VVC Multilayer Reference Implementation (VTM-21.0) on SDR UHD (4K) Content from JVET CTC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Olena Chubach, Ching-Yeh Chen, Tzu-Der Chuang, Yi-Wen Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Email: | {olena.chubach, chingyeh.chen, peter.chuang, yiwen-yw.chen, cw.hsu, yuwen.huang, shawmin.lei} mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution presents information about running tests on spatial scalability using VVC Reference software (VTM-21.0) and initial objective evaluation results. Tests were performed for 4K content, using sequences used for JVET Common Testing Conditions in Classes A1 and A2.

Overall, about +18% PSNR_YCbCr BD-rate is achieved compared to the Single Layer results, which is consistent to other tests performed in JVET using sequences outside of JVET Common Testing Conditions. For the two sequences, Campfire and FoodMarket4, the average PSNR_Y BD-rate is about +11%. At the same time, as can be seen from the results presented in JVET-AB0045, for the same sequences, about -4% PSNR_Y BD-rate can be obtained at lower bitrates.

Compared to the Simulcast, -35.53% PSNR_ YCbCr BD-rate can be obtained on average.

Introduction

VVC (Versatile Video Coding), also known as H.266, was designed to provide improved compression efficiency and to support a wide range of applications and devices. It represents a significant advancement in video coding technology, offering a versatile solution for the evolving demands of modern multimedia applications.

Among other features, VVC allows for scalable coding (multilayer, ML), aiming to provide even more efficient scalability options than its predecessors, with support for Spatial Scalability, Temporal Scalability, Quality Scalability, etc. The scalability features of VVC make it possible to efficiently stream and store video content that can be dynamically adapted to a wide range of devices, network conditions, and user preferences.

A formal Verification Test (VT) of VVC on SDR and 360-degree video content was finalized in July 2021. Verification Tests for VVC multilayer are still under discussion in JVET.

Below is a quick summary of JVET documents containing reports for already performed tests:

- JVET-Y0047 This contribution reports the results a formal visual quality assessment of VVC in spatial scalability mode, following preparation work described in JVET-X0202. With equal bitrate for base HD and enhancement UHD layer, PSNR-Y based performance is ‑24% vs simulcast, while MOS-based performance is around ‑15%; other layer bitrate allocations are tested in JVET-Y0048.

- JVET-Y0048 This contribution reports experiments with various bitrate allocations, and reports gains and lower encoding time over single layer in some cases (PSNR-wise, VVC multilayer  can be on par or even outperform a little single layer & upscale)

- JVET-AB0045 The VVC 4:2:0 multilayer profile was tested in a spatially scalable configuration with two layers and a spatial resolution ratio of 2x and 3x both vertically and horizontally. Tests performed using various bitrate allocations between the base layer and the enhancement layer, and the best trade-off leading to optimal enhancement layer compression performance was investigated. Both objective and subjective evaluations have been carried out and show that with VVC, spatial scalability using dual-layer coding can be on par or outperform single-layer coding in specific conditions. It is noted that visual comparison is more often in favor of dual-layer coding than PSNR. Optimum enhancement layer (EL) rate is reported to be around 10% of base layer (BL) bitrate. A small visual benefit of dual-layer over single-layer was obtained. No tests against the upscaled content.

- JVET-AD0399 Results of an on-site expert viewing performed at the 30th JVET meeting in Antalya, TK, comparing the compression performance of dual layer coding with a downscaled base layer at scaling ratios 1.5 and 2 to single layer coding. The results indicate competitive visual quality for the reconstructed output of the dual layer bitstreams. By tendency, gains appear to be observable at the lower bitrates while stronger impairments were noted at the higher bitrates. Results are somewhat consistent with AB0045. No tests against upscaled content.

- JVET-AE0228 This document reports the results of an on-site expert viewing performed at the 31st JVET meeting in Geneva, CH. The evaluation targets at the spatial scalability category of the test plan document JVET-AD2021, comparing the compression performance of dual layer (DL) coding with a downscaled base layer at scaling ratio 2 to single layer coding at the low resolution with upscaling to the target resolution using the VVC interpolation filters (UP) as well as single layer coding at the target resolution (SL). A fixed BL-EL QP offset rule is chosen, resulting in close to 10% rate allocation for EL. Tests against upscaled content are included.

- JVET-AF0186 This document reports about some experiments related to visual testing of VVC spatial scalability, as planned in JVET-AE2021, and after observations from expert viewing tests made during the previous JVET meeting, where the upscaled anchor was often found better than the dual-layer test. After investigation, it was found that moving the inter-layer reference first in the reference picture lists (instead of last) improved the situation. In addition, video sequences from LCEVC verification tests have been tested.

- JVET-AF0311 This document reports the results of an on-site expert viewing performed at the 32nd JVET meeting in Hannover, DE. The evaluation targets at the spatial scalability category of the test plan document JVET-AE2021, comparing the compression performance of dual layer coding with a downscaled base layer at scaling ratio 2 to single layer coding at the low resolution with upscaling to the target resolution using the VVC interpolation filters as well as single layer (SL) coding at the target resolution. In this contribution, in most cases, the MOS only slightly increased with the inclusion of the enhancement layer. This might be attributed to the fact that other effects overlay the benefit of enhanced detail provided with the DL stream. With a comparably low number of valid scores, the results of this contribution should be considered as an indication of tendencies, and not regarded as a necessarily conclusive result.

Tests for quality ladder use case for streaming (local cache space savings)

- JVET-AC0145 This document discusses the quality ladder category of the VVC multi-layer visual tests planned in JVET-AB2021. It proposes a recommended quality ladder for VVC, and reports about experiments of dual-layer coding various pairs in the quality ladder.

- JVET-AE0227 Encodings at candidate resolutions and overlapping bit rates were performed, PSNR and VMAF metrics of upscaled decoded pictures are reported, along with visual checks to estimate expected scores and to compare resolutions at comparable bitrate. The intent is to provide data to help define rate points and resolutions of the above-mentioned anchor.

Testing Conditions and Settings Used for VVC ML Evaluation

In the current test, 4K sequences in JVET Common Testing Conditions (CTC) with random access (RA) configuration are encoded.

VTM-21.0 reference software implementation of H.266/VVC is used as an anchor and for running the test.

Following simulations are performed:

- Test1. H.266/VVC SL: VTM-21.0 (single layer encoding of 4K content with QP=22, 27, 32, 37)

- Test2. H.266/VVC ML: VTM-21.0 (2x spatial scalability)

Test2 is run for enhancement layer QPs (QP_EL) between 23 and 47. The corresponding base layer is coded with QP_BL obtained as: QP_BL = QP_EL – 6, for each value of the QP_EL.

After that, the closest rates of Test2 (VVC ML) are chosen for the corresponding rates of Test1.

Two modifications were applied to the VTM-21.0 before running the tests:

- The patch provided in JVET-AF0186 was applied, putting inter-layer reference picture to the first position in the reference picture list

- Modifications in the MR !2686 were applied to upscale the pictures while decoding (using the normative RPR filters)

For each simulation point, two uncompressed sequences are used as inputs: one in UHD resolution, and another one – in HD resolution. Sequences at UHD resolution (3840×2160), are downscaled to HD (1920×1080) using the TAppDownConvertStatic tool from the SHM repository. The following example command lines are used for UHD BT.709-color content (chroma location 0):

TAppDownConvertStatic 3840 2160 <input_UHD.yuv> 1920 1080 <output_HD.yuv> 10 10

Following settings and configurations were used:

- resHD_60.cfg and resUHD_60.cfg - two configuration files, for HD and UHD content, respectively, which contain information that may vary depending on the resolution, frame rate, length, HDR, etc. Example of the configuration file for HD content is as follows:

#======== File I/O ===============

InputFile                : input_video_1920x1080_60fps_10bit_420.yuv

InputBitDepth            : 10           # Input bitdepth

InputChromaFormat       : 420          # Ratio of luminance to chrominance samples

FrameRate               : 60          # Frame Rate per second

FrameSkip               : 0           # Number of frames to be skipped in input

SourceWidth             : 1920        # Input  frame width

SourceHeight             : 1080       # Input  frame height

FramesToBeEncoded       : 600        # Number of frames to be coded

Level                    : 4.1

- two_layers.cfg – configuration file that contains parameters settings common for layers and specific for each layer (the reference picture list is automagically adapted by the VTM, depending on the multi-layer parameters found in this configuration file):

#======== Layers ===============

MultiLayerEnabledFlag       : 1

MaxLayers                     : 2

MaxSublayers                  : 7

DefaultPtlDpbHrdMaxTidFlag : 0

AllIndependentLayersFlag    : 0

AllowablePredDirection      : 0 0 0 0 0    # equal to 0 specifies the picture in the i-th temporal layer is allowed to use both inter-layer and intra-layer predictions

# equal to 1 specifies the picture in the i-th temporal layer is allowed to use inter-layer predition only

# equal to 2 specifies the picture in the i-th temporal layer is allowed to use intra-layer predition only

#======== OLSs ===============

EachLayerIsAnOlsFlag        : 0

OlsModeIdc                    : 0

NumPTLsInVPS                  : 2

#======== Layer-0 ===============

LayerId0                       : 0

#======== Layer-1 ===============

LayerId1                       : 1

NumRefLayers1                 : 1

RefLayerIdx1                  : 0

#======== OLS-0 ===============

OlsPTLIdx0                    : 0

#======== OLS-1 ===============

LevelPTL1                     : 6.1

OlsPTLIdx1                    : 1

- encoder_randomaccess_vtm.cfg - configuration file corresponding to the CTC random-access configuration.

Example encoder command line (configuration parameters prefixed by « -l0 » or « -l1 » are layer-specific. Non-prefixed parameters are common):

EncoderApp -c random_access.cfg  -l0 -q 38 -l0 -c resHD_60.cfg -l0 -i input_video_1920x1080_60p_420_10b.yuv  -l1 -q 45 -l1 -c resUHD_60.cfg -l1 -i input_video_3840x2160_60p_420_10b.yuv --DMVREncMvSelect=1 -l1 --Level=6.1 -c two_layers.cfg -ip 64 -fs 0 -b Procession_1920x1080_60p_420_10b_3845_P1.bin  --InternalBitDepth=10 --OutputBitDepth=10 --SEIDecodedPictureHash=1  --Verbosity=6 --PrintHexPSNR=1  --PrintMSSSIM=1  -f 65 -o /dev/null >>  Procession_1920x1080_60p_420_10b_3845_P1.log

Decoder-related settings:

- to decode OLS0 or OLS1, use the “-p 0” or “-p 1” when decoding the bitstream, respectively

- when decoding, to upscale the lower (HD) resolution sequence to the original (UHD) resolution (using the modifications in the MR !2686), use following parameters:

  - --UpscaledOutput=2 --outputWidth xxx --outputHeight xxx

  - In VTM-22, need to set --UpscaleFilterForDisplay to 0, so that the normative RPR filters are used for upscaling (by default it is set to “1”, i.e., the 12-tap alternative filters are used)

Objective Results

In the current tests, PSNR_Y (PSNR computed for luma component), PSNR_Cb, and PSNR_Cr (PSNR computed for two chroma components) are used as the quality metric to calculate BD-rates.

Combined PSNR_YCbCr is obtained as follows:

PSNRYCbCr  = 0.75*PSNRY  + 0.125*PSNRCb  + 0.125*PSNRCr

Simulation results of Test2 (VVC ML) vs Test1 (VVC SL) presented in Table I. Positive values mean that test has a larger file size than the anchor, under the same PSNR value.

Table I. Simulation results of Test2 (VVC ML) vs Test1 (VVC SL) as anchor (BD-rate, QP_BL=QP_EL-6)

| Sequence | QP_EL | Random Access, Over VTM-21.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR_Y | PSNR_Cb | PSNR_Cr | PSNR_YCbCr | VMAF BD-rate |
| Tango2 | 23, 28, 32, 37 | -1.28% | -1.64% | 8.90% | 0.45% | 2.78% |
| FoodMarket4 | 23, 28, 32, 37 | 2.49% | 4.05% | 4.46% | 2.97% | 9.01% |
| Campfire | 23, 29, 33, 37 | 19.09% | 21.00% | 39.43% | 21.93% | 6.80% |
| CatRobot | 24, 29, 33, 38 | 25.56% | 43.70% | 55.16% | 30.83% | 14.51% |
| DaylightRoad2 | 24, 29, 33, 38 | 31.80% | 21.03% | 27.55% | 29.56% | 19.21% |
| ParkRunning3 | 24, 29, 33, 37 | 22.45% | 48.55% | 42.89% | 25.82% | 6.32% |
| Overall |  | 16.69% | 22.78% | 29.73% | 18.59% | 9.77% |

It is challenging to compare any results obtained in this contribution with the previously available, considering that CTC sequences are not used in the recent tests, and rates in this contribution are higher than rates previously used in the scalable tests.

Some results can be found in JVET-AB0045 for two sequences (Campfire and FoodMarket4) but for higher QP values (see Table II). It should be also noted that VVC encodings in JVET-AB0045 were performed by the VTM in version 16.0 (17.0 for 3x tests), in RA CTC configuration.

Table II: BD-rate for VTM dual layer (test) vs VTM single layer (reference) reported in JVET-AB0045

| Sequence | QPBL/EL | QPSL | BD-rate (PSNRY) | BD-rate (MS-SSIMY) | BD-rate (VMAF) |
| --- | --- | --- | --- | --- | --- |
| FoodMarket4 | 30/38, 34/44, 38/48, 42/50 | 36, 40, 44, 48 | -4.9 % | -9.5 % | -7.7 % |
| Campfire | 24/32, 28/36, 34/42, 38/46 | 29, 35, 40, 44 | -2.7 % | -8.7 % | -5.8 % |
| Overall |  |  | -3.8% | -9.1% | -6.75% |

If compared to simulcast, results are reported in Table III.

Table III. Simulation results of Test2 (VVC ML) vs VVC simulcast as anchor

| Sequence | QP_EL | Random Access, Over VTM-21.0 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | PSNR_Y | PSNR_Cb | PSNR_Cr | PSNR_YCbCr |
| Tango2 | 23, 28, 32, 37 | -50.36% | -50.56% | -45.39% | -49.52% |
| FoodMarket4 | 23, 28, 32, 37 | -48.21% | -47.42% | -47.17% | -47.97% |
| Campfire | 23, 29, 33, 37 | -30.39% | -29.10% | -19.56% | -28.69% |
| CatRobot | 24, 29, 33, 38 | -32.41% | -22.71% | -16.55% | -29.59% |
| DaylightRoad2 | 24, 29, 33, 38 | -28.26% | -34.04% | -30.77% | -29.50% |
| ParkRunning3 | 24, 29, 33, 37 | -29.80% | -14.86% | -18.47% | -27.89% |
| Overall |  | -36.57% | -33.12% | -29.65% | -35.53% |

Additional information for bitrates per-layer can be found in Table IV.

Table IV. Bitrates used for BL, total bitrate and EL bitrate percentage used for sequences from Test2 (VVC ML)

| Sequence |  | QP_BL |  |  | QP_EL |  | Random Access |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | BL bitrate, kbps |  |  | Total bitrate, kbps |  |  | EL bitrate, % |  |  |
| Tango2 |  |  | 17 | 23 |  | 13966.5731 |  |  | 15492.66 |  | 9.85 |  |  |  |
|  |  |  | 22 | 28 |  | 6256.5078 |  |  | 6456.96 |  | 3.10 |  |
|  |  |  | 26 | 32 |  | 3575.3535 |  |  | 3649.77 |  | 2.04 |  |
|  |  |  | 31 | 37 |  | 1855.6424 |  |  | 1897.12 |  | 2.19 |  |
| FoodMarket4 | 17 |  |  | 23 |  | 12906.6048 |  | 15138.48 |  | 14.74 |  |  |  |
|  | 22 |  |  | 28 |  | 6687.0352 |  | 7207.45 |  | 7.22 |  |  |  |
|  | 26 |  |  | 32 |  | 3824.2816 |  | 4030.72 |  | 5.12 |  |  |  |
|  | 31 |  |  | 37 |  | 1926.2368 |  | 2002.09 |  | 3.79 |  |  |  |
| Campfire |  |  | 17 | 23 |  | 19524.1840 |  | 39782.41 |  | 50.92 |  |  |  |
|  |  |  | 23 | 29 |  | 7935.5488 |  | 11630.41 |  | 31.77 |  |  |  |
|  |  |  | 27 | 33 |  | 4578.9336 |  | 6666.20 |  | 31.31 |  |  |  |
|  |  |  | 31 | 37 |  | 2806.5616 |  | 3584.80 |  | 21.71 |  |  |  |
| CatRobot |  |  | 18 | 24 |  | 12653.9184 |  | 17468.18 |  | 27.56 |  |  |  |
|  |  |  | 23 | 29 |  | 6231.2784 |  | 7610.46 |  | 18.12 |  |  |  |
|  |  |  | 27 | 33 |  | 3557.8368 |  | 4125.46 |  | 13.76 |  |  |  |
|  |  |  | 32 | 38 |  | 1831.4528 |  | 2019.38 |  | 9.31 |  |  |  |
| DaylightRoad2 |  |  | 18 | 24 |  | 13539.4896 |  | 19724.24 |  | 31.36 |  |  |  |
|  |  |  | 23 | 29 |  | 6199.0672 |  | 8070.19 |  | 23.19 |  |  |  |
|  |  |  | 27 | 33 |  | 3434.8016 |  | 4218.24 |  | 18.57 |  |  |  |
|  |  |  | 32 | 38 |  | 1728.5344 |  | 1986.60 |  | 12.99 |  |  |  |
| ParkRunning3 |  |  | 18 | 24 |  | 58041.9413 |  | 95680.04 |  | 39.34 |  |  |  |
|  |  |  | 23 | 29 |  | 26699.8227 |  | 37069.02 |  | 27.97 |  |  |  |
|  |  |  | 27 | 33 |  | 13689.6507 |  | 17748.28 |  | 22.87 |  |  |  |
|  |  |  | 31 | 37 |  | 6839.0813 |  | 8275.83 |  | 17.36 |  |  |  |

Additional results for the VVC ML vs VVC SL with QP_BL=QP_EL-5 can be found in Table V and Table VI.

Additional results for the VVC ML vs VVC SL with QP_BL=QP_EL-7 can be found in Table VII and Table VIII.

Additional results for the VVC ML vs VVC SL with QP_BL=QP_EL-8 can be found in Table IX and Table X.

Table V. Simulation results of VVC ML vs VVC SL as anchor (BD-rate, QP_BL=QP_EL-5)

| Sequence | QP_EL | Random Access, Over VTM-21.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR_Y | PSNR_Cb | PSNR_Cr | PSNR_YCbCr |
| Tango2 | 23, 27, 31, 36 | -0.08% | 0.10% | 9.88% | 1.69% |
| FoodMarket4 | 23, 27, 32, 36 | 3.80% | 5.22% | 5.85% | 4.26% |
| Campfire | 23, 28, 33, 37 | 21.16% | 19.12% | 43.56% | 23.27% |
| CatRobot | 24, 28, 33, 37 | 25.93% | 44.99% | 53.99% | 31.07% |
| DaylightRoad2 | 24, 29, 33, 37 | 30.21% | 25.27% | 28.38% | 28.95% |
| ParkRunning3 | 24, 28, 33, 37 | 24.64% | 41.28% | 37.75% | 26.83% |
| Overall |  | 17.61% | 22.66% | 29.90% | 19.35% |

Table VI. Bitrates used for BL, total bitrate and EL bitrate percentage used for sequences from VVC ML, QP_BL=QP_EL-5

| Sequence | QP_BL | QP_EL | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BL bitrate, kbps | Total bitrate, kbps | EL bitrate, % |
| Tango2 | 18 | 23 | 11645.9690 | 14277.20 | 18.43 |
|  | 22 | 27 | 6256.5078 | 6679.04 | 6.33 |
|  | 26 | 31 | 3575.3535 | 3715.87 | 3.78 |
|  | 31 | 36 | 1855.6424 | 1948.07 | 4.74 |
| FoodMarket4 | 18 | 23 | 11450.2144 | 14219.89 | 19.48 |
|  | 22 | 27 | 6687.0352 | 7538.63 | 11.30 |
|  | 27 | 32 | 3321.0688 | 3613.28 | 8.09 |
|  | 31 | 36 | 1926.2368 | 2095.56 | 8.08 |
| Campfire | 18 | 23 | 16800.6112 | 37954.67 | 55.74 |
|  | 23 | 28 | 7935.5488 | 12400.74 | 36.01 |
|  | 28 | 33 | 3978.1080 | 6366.62 | 37.52 |
|  | 32 | 37 | 2509.4688 | 3414.13 | 26.50 |
| CatRobot | 19 | 24 | 10871.0592 | 16271.39 | 33.19 |
|  | 23 | 28 | 6231.2784 | 8265.69 | 24.61 |
|  | 28 | 33 | 3105.9088 | 3832.52 | 18.96 |
|  | 32 | 37 | 1831.4528 | 2162.83 | 15.32 |
| DaylightRoad2 | 19 | 24 | 11340.1920 | 18060.95 | 37.21 |
|  | 24 | 29 | 5296.0752 | 7407.46 | 28.50 |
|  | 28 | 33 | 2988.1056 | 3898.82 | 23.36 |
|  | 32 | 37 | 1728.5344 | 2125.01 | 18.66 |
| ParkRunning3 | 19 | 24 | 48968.0533 | 89024.27 | 44.99 |
|  | 23 | 28 | 26699.8227 | 40785.76 | 34.54 |
|  | 28 | 33 | 11534.7893 | 16520.91 | 30.18 |
|  | 32 | 37 | 5859.2533 | 7677.30 | 23.68 |

Table VII. Simulation results of VVC ML vs VVC SL as anchor (BD-rate, QP_BL=QP_EL-7)

| Sequence | QP_EL | Random Access, Over VTM-21.0 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | PSNR_Y | PSNR_Cb | PSNR_Cr | PSNR_YCbCr |
| Tango2 | 24, 29, 33, 38 | -1.75% | -2.27% | 8.10% | -0.11% |
| FoodMarket4 | 24, 29, 33, 38 | 2.21% | 2.76% | 3.10% | 2.41% |
| Campfire | 23, 29, 34, 37 | 18.17% | 28.53% | 33.77% | 22.34% |
| CatRobot | 25, 29, 34, 38 | 26.28% | 41.82% | 54.85% | 31.22% |
| DaylightRoad2 | 24, 30, 34, 38 | 35.27% | 18.93% | 28.54% | 31.95% |
| ParkRunning3 | 24, 29, 34, 38 | 20.13% | 58.38% | 49.64% | 24.89% |
| Overall |  | 16.72% | 24.69% | 29.67% | 18.78% |

Table VIII. Bitrates used for BL, total bitrate and EL bitrate percentage used for sequences from VVC ML, QP_BL=QP_EL-7

| Sequence | QP_BL | QP_EL | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BL bitrate, kbps | Total bitrate, kbps | EL bitrate, % |
| Tango2 | 17 | 24 | 13966.5731 | 14586.25 | 4.25 |
|  | 22 | 29 | 6256.5078 | 6375.99 | 1.87 |
|  | 26 | 33 | 3575.3535 | 3623.34 | 1.32 |
|  | 31 | 38 | 1855.6424 | 1883.03 | 1.45 |
| FoodMarket4 | 17 | 24 | 12906.6048 | 14237.03 | 9.34 |
|  | 22 | 29 | 6687.0352 | 7032.06 | 4.91 |
|  | 26 | 33 | 3824.2816 | 3970.40 | 3.68 |
|  | 31 | 38 | 1926.2368 | 1973.38 | 2.39 |
| Campfire | 16 | 23 | 24778.8832 | 44046.43 | 43.74 |
|  | 22 | 29 | 9382.1024 | 12813.32 | 26.78 |
|  | 27 | 34 | 4578.9336 | 6211.34 | 26.28 |
|  | 30 | 37 | 3202.6400 | 3874.61 | 14.54 |
| CatRobot | 18 | 25 | 12653.9184 | 16198.71 | 21.88 |
|  | 22 | 29 | 7167.4720 | 8415.78 | 14.83 |
|  | 27 | 34 | 3557.8368 | 3968.90 | 10.36 |
|  | 31 | 38 | 2066.8096 | 2219.36 | 6.87 |
| DaylightRoad2 | 17 | 24 | 15840.4640 | 21671.21 | 26.91 |
|  | 23 | 30 | 6199.0672 | 7618.63 | 18.63 |
|  | 27 | 34 | 3434.8016 | 4019.61 | 14.55 |
|  | 31 | 38 | 1958.2592 | 2183.62 | 9.34 |
| ParkRunning3 | 17 | 24 | 65710.7387 | 101908.99 | 35.52 |
|  | 22 | 29 | 31508.1573 | 41049.96 | 23.24 |
|  | 27 | 34 | 13689.6507 | 16626.14 | 17.66 |
|  | 31 | 38 | 6839.0813 | 7743.63 | 11.68 |

Table IX. Simulation results of VVC ML vs VVC SL as anchor (BD-rate, QP_BL=QP_EL-8)

| Sequence | QP_EL | Random Access, Over VTM-21.0 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | PSNR_Y | PSNR_Cb | PSNR_Cr | PSNR_YCbCr |
| Tango2 | 25, 30, 34, 39 | -2.05% | -2.87% | 8.27% | -0.37% |
| FoodMarket4 | 25, 30, 34, 39 | 2.10% | 2.00% | 2.57% | 2.15% |
| Campfire | 23, 30, 34, 38 | 17.30% | 39.73% | 30.10% | 22.92% |
| CatRobot | 25, 30, 35, 39 | 28.05% | 42.81% | 57.72% | 33.09% |
| DaylightRoad2 | 25, 30, 35, 39 | 38.32% | 17.26% | 29.72% | 33.32% |
| ParkRunning3 | 25, 30, 34, 39 | 16.56% | 69.32% | 57.08% | 22.70% |
| Overall |  | 16.7% | 28.0% | 30.9% | 18.97% |

Table X. Bitrates used for BL, total bitrate and EL bitrate percentage used for sequences from VVC ML, QP_BL=QP_EL-8

| Sequence | QP_BL | QP_EL | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BL bitrate, kbps | Total bitrate, kbps | EL bitrate, % |
| Tango2 | 17 | 25 | 13966.5731 | 14354.52 | 2.70 |
|  | 22 | 30 | 6256.5078 | 6334.83 | 1.24 |
|  | 26 | 34 | 3575.3535 | 3611.93 | 1.01 |
|  | 31 | 39 | 1855.6424 | 1878.66 | 1.23 |
| FoodMarket4 | 17 | 25 | 12906.6048 | 13848.15 | 6.80 |
|  | 22 | 30 | 6687.0352 | 6937.57 | 3.61 |
|  | 26 | 34 | 3824.2816 | 3934.90 | 2.81 |
|  | 31 | 39 | 1926.2368 | 1959.78 | 1.71 |
| Campfire | 15 | 23 | 29175.9152 | 47913.50 | 39.11 |
|  | 22 | 30 | 9382.1024 | 12045.51 | 22.11 |
|  | 26 | 34 | 5283.3664 | 6757.13 | 21.81 |
|  | 30 | 38 | 3202.6400 | 3617.64 | 11.47 |
| CatRobot | 17 | 25 | 14424.7952 | 17765.39 | 18.80 |
|  | 22 | 30 | 7167.4720 | 8103.40 | 11.55 |
|  | 27 | 35 | 3557.8368 | 3857.18 | 7.76 |
|  | 31 | 39 | 2066.8096 | 2174.33 | 4.94 |
| DaylightRoad2 | 17 | 25 | 15840.4640 | 20295.76 | 21.95 |
|  | 22 | 30 | 7196.6768 | 8542.57 | 15.76 |
|  | 27 | 35 | 3434.8016 | 3875.06 | 11.36 |
|  | 31 | 39 | 1958.2592 | 2119.12 | 7.59 |
| ParkRunning3 | 17 | 25 | 65710.7387 | 93744.34 | 29.90 |
|  | 22 | 30 | 31508.1573 | 38505.73 | 18.17 |
|  | 26 | 34 | 16193.9307 | 18877.05 | 14.21 |
|  | 31 | 39 | 6839.0813 | 7482.93 | 8.60 |

Conclusions

This contribution presents information about running tests on spatial scalability using VVC Reference software (VTM-21.0) and initial objective evaluation results. Overall, +18% PSNR BD-rate was obtained on average for the 4K CTC sequences. For the two sequences, Campfire and FoodMarket4, the average PSNR_Y BD-rate is about +11%. At the same time, as it can be seen from the results presented in JVET-AB0045, for the same sequences, about -4% PSNR BD-rate can be obtained at lower bitrates.

References

F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring “VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video”, JVET-AB2010, Dec. 2022.

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.

https://vcgit.hhi.fraunhofer.de/jvet/SHM/-/tree/SHM-12.4?ref_type=tags

Patent rights declaration

MediaTek Inc. does not have any current or pending patent rights relating to the technology described in this contribution.