| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0077-v2 |
| --- | --- |

| Title: | AHG11: A Low-Complexity Neural Network Loop Filter based on Partial Convolution and Over-Parameterization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ao Li, Ce Zhu, Lei Luo, Yongkai Huo, Yutian Liu | Tel: Email: | aoli@std.uestc.edu.cn eczhu@uestc.edu.cn luolei1010@uestc.edu.cn yongkai.huo@transsion.com yutian.liu@transsion.com |
| Source: | University of Electronic Science and Technology of China, Transsion Holdings |  |  |

# Abstract

This contribution proposes the use of partial convolution to reduce the inference complexity of neural-network-based loop filtering. To mitigate the performance loss associated with reducing complexity, this contribution further proposes an over-parameterization training strategy to improve performance while keeping complexity unchanged. LOP1 filter is performed on top of NNVC-8.0 with NnIntraPred enabled as a comparison baseline. Using block-level KMAC/Pixel as the complexity metric, the baseline model has a worst-case complexity of 16.2 KMAC/Pixel. The proposed low-complexity neural network loop filter has about 15.3 KMAC/Pixel, representing approximately a 5.5% complexity reduction. Additionally, this contribution significantly reduces the number of convolutional layers by about 24%. The new proposed model is implemented on top of the NNVC-8.0.

The AI BD-Rate gain over NNVC-8.0 VTM with tools off are as follows:

- SADL, fp32:  AI {-7.97%, -9.54%, -9.03%}

- SADL, int16:  AI {-7.91%, -9.52%, -8.96%}

The AI BD-Rate results over LOP1&IntraPred are as follows:

- SADL, fp32:  AI { -0.02%, -0.52%, 0.12%}

- SADL, int16:  AI {-0.12%, -0.97 %, -0.12%}

# Introduction

Neural-network-based loop filters (NNLF) have shown great potential in improving coding efficiency. However, complexity remains a concern for real-world applications. JVET-AD0156 [1] proposed the use of CP decomposition [2] to reduce the complexity of NNLF and it is the default LOP1 filter in NNVC-8.0.

Figure 1 shows the baseline LOP1 architecture: CP-decomposition of 3x3 convolution layers and fusion of adjacent 1x1 pointwise convolutions. The 3x3 convolutions of each hidden layer are decomposed into 4 layers with rank R followed by fusion of adjacent 1x1 convolution:

- 1st layer: 1x1xKxR pointwise convolution

- 2nd layer: 3x1xRxR separable convolution

- 3rd layer: 1x3xRxR separable convolution

- 4th layer: 1x1xRxK pointwise convolution

Figure 1 LOP1 Model ( CP Decomposition + fusion of 1x1 conv)

Although significant complexity reduction has been achieved, with the worst-case complexity being 16.2 KMAC/Pixel compared to the JVET-X0140 low complexity model (EE1-1.4.1) [3], three main issues still remain: (1) As mentioned in JVET-AD0250 [4], the CP decomposition step within this structure sometimes produces initial values with abnormally large absolute values after weight decomposition. This leads to instability in network training in subsequent phases and can cause the network to fail to converge. (2) In neural network models, it is common to increase the number of convolutional layer channels to enhance model performance. However, this often leads to an excessive number of channels, causing model redundancy and implying that the network's complexity can still be further reduced. (3) The use of CP decomposition results in more convolutional layers, potentially slowing down the inference time. To address these issues, this contribution presents a study on NNLF inference complexity reduction using partial convolution [5] and over-parameterization [6].

# Proposed methods

This proposal describes a low-complexity neural network loop filter structure design based on partial convolution and over-parameterization. The network architecture is shown in Figure 2.

Figure 2 Proposed Structure (Partial Convolution + Over-parameterization)

## Partial convolution

The partial convolution is shown as the blue part in Figure 2. Depending on the desired degree of complexity reduction, the partial convolution coefficient α is set to control the number of channels involved in the convolution operation in the convolution layer, thereby achieving the purpose of reducing the network's computational complexity. Specifically, for a feature with K channels, based on the partial convolution coefficient α, it is divided into two sub-features by channel, which respectively have the number of channels: K/α, K*(1-1/α). The proposed structure only performs convolution operations on the sub-feature with K/α channels, and after the convolution, it is concatenated with the remaining sub-feature having K*(1-1/α) channels to obtain the final output of the partial convolution layer.

Furthermore, to ensure full integration between the two features after concatenation, and to maintain the filter complexity without additional increase, this structure moves the second 1x1 convolution layer of each hidden layer in the baseline model to after the partial convolution layer to serve as feature integration, keeping the position of the first 1x1 convolution layer unchanged, and adjusts the activation function to the last position to enable the network to gain nonlinear capabilities.

## Over-parameterization

Another contribution is that during the training phase, the network is widened through the expansion coefficient β and deepened using the reparameterization method. The original 3x3 partial convolution is expanded into a form consisting of a 3x3 convolution and a 1x1 convolution. In the inference phase, by using the reparameterization method, the aforementioned two convolutions are merged into a new 3x3 convolution, thus enhancing network performance without increasing complexity. The principle is as follows:

For a given input image X∈RC×H×W , where C is the number of channels, H is the image height, and W is the image width. The sizes of the two convolution kernels are respectively W1∈RCout1×Cin1×k1×k1 and W2∈RCout2×Cout1×k2×k2, where Cout1, Cout1 and Cout2 represent the input and output channel numbers of the convolution layers, and k1 and k1 represent the sizes of the convolution kernels. The convolution process can be expressed as:

Xout=(X*W1)*W2,

where ∗ represents the convolution process. By the property of convolution, the above process can be re-expressed as:

Xout=X*W1*W2=X*Wnew,

where the Wnew indicates the fused weight. Through the above transformation, it is possible to use a new convolution kernel to replace the two previous kernels, thereby completing the reparameterization process.

## Complexity Analysis

The block-level complexity calculation method on the inference stage is as follows:

Given the parameter settings M=72, K=24, L=6, α=0.25, n=11.

-	MACs of the input layer: 3*3*10*M = 6480

-	MACs of the last hidden layer: K*M + K*K + 3*3*(K*α)*(K*α) = 2628

-	MACs of the other hidden layers: (n-1)*(K*M + K*M + 3*3*(K*α)*(K*α)) = 37800

-	MACs of the output layer: 3*3*K*L = 1296

-	Thus, KMAC/Pixel: (6480+2628+37800+1296)*72*72/(128*128) = 15.252 KMAC/Pixel

Table 1 provides a complexity comparison of this technical solution relative to the baseline model. The results show that the architecture of this technical solution can reduce complexity by 5.6%, and in terms of the number of parameters, this solution can reduce complexity by 6.1%.

Table 1 Complexity Comparison (Block-wise)

| Model | KMAC/Pixel | Params | Layers |
| --- | --- | --- | --- |
| LOP1 | 16.2 | 52566 | 46 |
| Ours | 15.3 (-5.6%) | 49356 (-6.1%) | 35 (-24%) |

# Simulation results

Simulations are performed based on the common test conditions for neural-network-based video coding technology for AI configurations [7].

Table 2 and Table 3 show the AI performances of partial convolution with over-parameterization model in compared against NNVC-8.0 VTM for SADL fp32 and SADL int16. Table 4 and Table 5 show the AI performances of partial convolution with over-parameterization model in compared against LOP1 for SADL fp32 and SADL int16. It is noted that the encoder time ratio may be not reliable.

Table 2 AI performance of Partial Convolution + Over-parameterization (SADL, fp32)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 VTM (tools off) |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |  |
| Class A1 | -8.62% | -8.09% | -8.79% | #DIV/0! | #DIV/0! |  |
| Class A2 | -6.90% | -9.22% | -7.90% | #DIV/0! | #DIV/0! |  |
| Class B | -7.08% | -9.49% | -9.08% | #DIV/0! | #DIV/0! |  |
| Class C | -7.53% | -10.45% | -9.92% | #DIV/0! | #DIV/0! |  |
| Class E | -10.48% | -10.20% | -9.14% | #DIV/0! | #DIV/0! |  |
| Overall | -7.97% | -9.54% | -9.03% | #DIV/0! | #DIV/0! |  |
| Class D | -7.76% | -8.84% | -10.14% | #DIV/0! | #DIV/0! |  |

Table 3  AI performance of Partial Convolution + Over-parameterization (SADL, int16)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 VTM (tools off) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -8.61% | -8.09% | -8.61% | #DIV/0! | #DIV/0! |
| Class A2 | -6.82% | -9.48% | -7.77% | #DIV/0! | #DIV/0! |
| Class B | -7.03% | -9.41% | -8.99% | #DIV/0! | #DIV/0! |
| Class C | -7.46% | -10.34% | -9.88% | #DIV/0! | #DIV/0! |
| Class E | -10.39% | -10.08% | -9.22% | #DIV/0! | #DIV/0! |
| Overall | -7.91% | -9.52% | -8.96% | #DIV/0! | #DIV/0! |
| Class D | -7.71% | -8.73% | -10.06% | #DIV/0! | #DIV/0! |

Table 4 AI performance of Partial Convolution + Over-parameterization (SADL, fp32)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 (LOP1 & NN-Intra ON) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.07% | -0.51% | 0.14% | 91% | #DIV/0! |
| Class A2 | -0.05% | -0.77% | 0.37% | 106% | #DIV/0! |
| Class B | 0.03% | -0.30% | 0.10% | 78% | #DIV/0! |
| Class C | -0.06% | -0.34% | 0.24% | 79% | #DIV/0! |
| Class E | 0.02% | -0.87% | -0.29% | 55% | #DIV/0! |
| Overall | -0.02% | -0.52% | 0.12% | 80% | #DIV/0! |
| Class D | -0.08% | -0.27% | -0.40% | 66% | #DIV/0! |

Table 5  AI performance of Partial Convolution + Over-parameterization (SADL, int16)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 (LOP1 & NN-Intra ON) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.31% | -0.83% | -0.02% | 112% | #DIV/0! |
| Class A2 | -0.13% | -1.62% | 0.26% | 104% | #DIV/0! |
| Class B | -0.01% | -0.87% | -0.28% | 106% | #DIV/0! |
| Class C | -0.13% | -0.63% | -0.07% | 99% | #DIV/0! |
| Class E | -0.11% | -1.08% | -0.39% | 84% | #DIV/0! |
| Overall | -0.12% | -0.97% | -0.12% | 101% | #DIV/0! |
| Class D | -0.14% | -0.60% | -0.74% | 83% | #DIV/0! |

# Training and inference information

The models are trained based on DIV2K [8] for AI only. The raw DIV2K image data was compressed using AI configurations to create the training examples for AI. As in LOP1, joint models are used for luma and chroma components. Two models are trained using the AI-compressed data. More information related to the training and inference of models can be found in the tables below.

Table 4  Training information of Partial Convolution + Over-parameterization model

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type |  |
|  | Framework: | Tensorflow 2.8.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 400 |
|  | Batch size: | 16 |
|  | Training time: | ~140 hours |
|  | Training data information: | DIV2K |
|  | Training configurations for generating compressed training data (is different to VTM CTC): | QP=20,25,30,35,40,45 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 72x72 (4 luma and 2 chroma planes) |
|  | Learning rate: | AI: - 2E-4 0-300 epochs - 2E-5 300-350 epochs - 2E-6 350-400 epochs |
|  | Optimizer: | ADAM |
|  | Loss function: | - 4-1-1 Weighted L2 0-150 epochs - 12-1-1 Weighted L2 150-400 epochs |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Other information: |  |

Table 5  Inference information of Partial Convolution + Over-parameterization model

| Network Information in Inference Stage |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Baseline CP Fused |  | Proposed model |
| Mandatory | HW environment: |  |  |  |
|  | GPU Type | CPU only |  |  |
|  | Framework: | SADL float, SADL int16 |  |  |
|  | Number of GPUs per Task | 0 |  |  |
|  | Number of Parameters (Each Model) | 52566 |  | 49356 |
|  | Total Parameter Number | 105132 |  | 98712 |
|  | Parameter Precision (Bits) | 32 (F) / 16 (I) |  |  |
|  | Memory Parameter (MB) |  |  |  |
|  | MAC/Pixel (Kilo) | 16.2 |  | 15.3 |
| Optional |  |  |  |  |
|  | Total Conv. Layers | 46 |  | 35 |
|  | Total FC Layers | 0 |  |  |
|  | Total Memory (MB) |  |  |  |
|  | Batch size: |  |  |  |
|  | Patch size | 72x72 (4 luma and 2 chroma planes ) |  |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |  |
|  | Peak Memory Usage (Total) |  |  |  |
|  | Peak Memory Usage (per Model) |  |  |  |
|  | Other information: |  |  |  |

# Conclusion

This contribution demonstrates the potential of using partial convolution and over-parameterization to achieve a computational complexity reduction of approximately 5.5% compared to LOP1 model. The AI luma gain over LOP1 model is about 0.02% in the float32 condition and 0.12% in the int16 condition, with a worst-case complexity of 15.3 KMAC/Pixel. Additionally, this contribution significantly reduces the number of convolutional layers by about 24%. The contribution proposes further study of the complexity reduction framework for the RA case and its integration into the LOP.2 framework for more in-depth analysis.

# References

- J. N. Shingala, S. Kadaramandalgi, A. Shyam, T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy, “EE1: Complexity reduction on neural-network loop filter”, JVET-AD0156, Antalya, April 2023.

- V. Lebedev, Y. Ganin et al “Speeding-up Convolutional Neural Networks Using Fine-tuned CP-Decomposition”, ICLR 2015.

- H. Wang, J. Chen, K. Reuze, A.M. Kotra, M. Karczewicz, “EE1-1.4: Tests on Neural Network-based In-Loop Filter with Constrained Computational Complexity”, JVET-X0140, teleconference, Oct 2021.

- J. Ström, M. Damghanian. “Crosscheck of JVET-AD0156 (EE1-1.1: Complexity Reduction on   Neural-Network Loop Filter)”, document JVET-AD0250, JVET, Antalya, Apr. 2023.

- J. Chen, S.H. Kao, H. He, W. Zhuo, S. Wen, C. H. Lee, S.H.G. Chan, “Run, Don't walk: Chasing higher FLOPS for faster neural networks”, CVPR 2024.

- Bhardwaj, K., Milosavljevic, M., O'Neil, L., Gope, D., Matas, R., Chalfin, A., ... & Loh, D. (2022). Collapsible linear blocks for super-efficient super resolution. Proceedings of Machine Learning and Systems, 4, 529-547.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AF2016, Hannover, Oct. 2023.

- R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool, https://data.vision.ee.ethz.ch/cvl/DIV2K/

# Patent rights declaration(s)

University of Electronic Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Shenzhen Transsion Holdings Co., Ltd. (Transsion) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).