| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0082-v1 |
| --- | --- |

| Title: | Non-EE2: GPM with CCCM for intra chroma prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongkai Huo Yutian Liu | Email: | yongkai.huo@transsion.com, yutian.liu@transsion.com |
| Source: | Transsion Holdings |  |  |

_____________________________

- Abstract

This contribution combines GPM with convolutional cross-component model (CCCM). The proposed method splits each block into two partitions using at most 64 modes. The impacts on coding efficiency and runtimes over ECM-12.0 are reportedly {for Y, U, V, EncT, DecT}:

AI { xx.xx%, xx.xx%, xx.xx%, xxx%, xxx%},

RA { xx.xx%, xx.xx%, xx.xx%, xxx%, xxx%}

Introduction

A number of cross-component tools have been proposed, such as the Cross-Component Linear Model (CCLM) and Convolutional Cross-Component Model (CCCM) [1]. Geometric prediction merge mode (GPM) splits a block into two non-rectangular partitions, which may be predicted independently. It is intuitive to investigate cross-component tools together with GPM.

CCCM predicts chroma samples from reconstructed luma samples using a 7-tap filter. Similar to CCLM, there is an option of using a single model or multi-model variant of CCCM. The current 6 CCCM modes are listed as follows:

- Single-model CCCM from above and left templates

- Single-model CCCM from above template

- Single-model CCCM from left template

- Multi-model CCCM from above and left templates

- Multi-model CCCM from above template

- Multi-model CCCM from left template

Proposed method

Description

This proposal propose GPM with CCCM for intra chroma prediction. Similar to GPM, each chroma block is split into two partitions and both partitions are predicted using a CCCM mode.

For each block, a maximum of 64 partition modes are designed consisting of 8 partitioning angles each with 8 partitioning shifts, while only the “Single-model CCCM from above and left templates” and “Multi-model CCCM from above and left templates” are allowed for predicting the chroma partitions.

Figure 1. Example of two CCCM partitions

Bitstream signalling

Usage of the mode is signalled with a CABAC coded PU level flag, followed by a maximum of  truncated binary coded 6 bits indicating the partitioning mode.

Experimental results

Tables below summarize the coding efficiency and runtime impact of the proposed method in ECM 12.0 using the JVET common test conditions [2].

Table 1: performances of proposed method over ECM-12.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.01% | -0.31% | -0.13% | 1.13 | 1.00 | 1.00 | 1.00 |
| Class E | -0.05% | 0.00% | -0.15% | 1.14 | 1.00 | 1.01 | 1.00 |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.01% | -0.23% | -0.32% | 1.09 | 0.99 | 1.01 | 1.00 |
| Class F | -0.03% | -0.32% | -0.47% | 1.07 | 0.99 | 1.03 | 1.00 |
| Class TGM |  |  |  |  |  |  |  |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Conclusion

This contribution proposes GPM with CCCM for intra chroma prediction. The results show that the gain can be further improved. It is suggested to further investigate the proposed method in the next round of EE2.

References

- P. Astola, J. Lainema, R. G. Youvalari, A. Aminlou, and K. Panusopone, “EE2-1.1a: Convolutional cross-component intra prediction model,” JVET-AA0057, Jul. 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

Patent rights declaration(s)

Transsion may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).