| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0083-v02 |
| --- | --- |

| Title: | AHG12: Additional TIMD mode with different cost metric |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Döne Bugdayci Sansli Saverio Blasi Jani Lainema | Tel: Email: | - done.bugdayci_sansli@nokia.com saverio.blasi@nokia.com jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes an additional TIMD based intra coding mode, which is signaled as a sub mode of TIMD. In this coding mode, template costs are calculated with mean removed SAD. To further differentiate the proposed mode from the TIMD mode of ECM-12, the selection of template sizes and candidate modes for the new mode are also modified.

It is reported that the proposed additional intra mode produces the following coding efficiency results for AI configuration when implemented on top of ECM-12.0 {for Y, U, V}:

  - AI - {-0.08%, 0.03%, 0.05%,       XXX%, XXX%, XXX%, XXX%}

RA: { XXX%, XXX%, XXX%,  XXX%, XXX%, XXX%, XXX%

# Introduction

Template-based intra mode derivation (TIMD) is an intra coding mode in ECM-12.0 where up to 3 predictors (two predictors with least SATD costs and a non-angular predictor) are blended. The blending weights are determined based on the respective SATD based template costs. In addition to blending with block-wise weights, location-dependent sample-based blending is also supported where the selection of location-dependent weights depends on a ratio of the normalized SATD of the selected TIMD predictors computed in ABOVE and LEFT templates area.

# Proposal

The mode derivation process for the proposed additional TIMD mode differs from the regular TIMD mode in three aspects:

- Cost metric is changed from SATD to MR SAD,

- Template size is doubled,

- List of candidate modes for which the costs are calculated is modified,

This increases the diversity between regular and alternative TIMD modes. The selection of modes and blending weights are left unchanged from the regular TIMD design.

# Experimental results

Table 1 All Intra configuration simulation results of proposed method against ECM-12.0 anchor.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.12 % | -0.02 % | 0.04 % |  |  |  |  |
| Class A2 | -0.07 % | 0.09 % | 0.04 % |  |  |  |  |
| Class B | -0.09 % | 0.03 % | 0.05 % |  |  |  |  |
| Class C | -0.07 % | 0.03 % | 0.09 % |  |  |  |  |
| Class E | -0.03 % | 0.06 % | 0.00 % |  |  |  |  |
| Overall | -0.08 % | 0.03 % | 0.05 % |  |  |  |  |
| Class D | -0.10 % | 0.36 % | 0.11 % |  |  |  |  |
| Class F | -0.01 % | 0.13 % | 0.15 % |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 2 Random access configuration simulation results of proposed method against ECM-12.0 anchor.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).