| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0084-v2 |
| --- | --- |

| Title: | EE2-6.3: Combination of EE2-6.1 and EE2-6.2 on entropy coding extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin, F. Lo Bianco C. Salmon-Legagneur M. Balcilar Ru-Ling Liao Yan Ye Jie Chen Xinwei Li | Tel: Email: | franck.galpin@interdigital.com ruling.lrl@alibaba-inc.com |
| Source: | InterDigital Inc., Alibaba group |  |  |

_____________________________

# Abstract

This contribution shows the results of the combination of EE2 tests 6.1 and 6.2. The entropy models are extended on 2 aspects: a fourth type model for low delay B slices is added to the B/P/I models and an additional NEP (constant probability model) is added. EP bins are further allowed to use NEP model. The results show …

# Introduction

In the EE2 test 6.1, a new type of model (hereinafter referred to as L model) for B slices is proposed to differentiate between low-delay B and non-low delay B slices. Each B slice initializes its context model according to the low delay condition. The L model is used for low delay B slices, whereas the B model is used for non-low delay B slices.

In the EE2 test 6.2, a new entropy model called NEP (non EP) between EP (equiprobable) and CABAC (contextual, adaptive, arithmetic coding) is proposed. Furthermore, previously EP coded bins are given an associated context and can therefore uses the new NEP model.

# Simulations

In both tests, full encoding in AI, RA, LDB and LDP configurations using the 4 CTCs QPs is used.

## Test 6.3 – using CTC sequences

All the CTC sequences including F and TGM are used.

The CABAC/NEP/EP models are taken as follow:

- BPI model is taken from EE2-6.2: the models are retrained using all AI/RA/LDB/LDP sequences.

- L model is taken from EE2-6.1 for CABAC bins

- B model is duplicated to L model for non CABAC bins (NEP bins).

The following statistics is obtained:

| Model | Before | After |
| --- | --- | --- |
| CABAC | 6211 |  |
| NEP | 0 |  |
| EP |  |  |

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.00% | 0.02% | 100.3% | 100.1% | 100.2% | 100.6% |
| Class A2 | -0.04% | -0.05% | -0.06% | 100.1% | 101.7% | 100.3% | 101.4% |
| Class B | -0.03% | -0.03% | 0.08% | 100.7% | 100.2% | 96.8% | 101.0% |
| Class C | -0.03% | -0.17% | -0.10% | 100.4% | 101.8% | 100.5% | 100.2% |
| Class E | -0.03% | -0.04% | -0.03% | 101.3% | 102.0% | 99.8% | 100.6% |
| Overall | -0.03% | -0.06% | -0.01% | 100.6% | 101.1% | 99.3% | 100.8% |
| Class D | -0.07% | 0.05% | -0.40% | 99.7% | 101.4% | 100.0% | 100.1% |
| Class F | -0.09% | -0.19% | -0.20% | 99.2% | 100.5% | 101.1% | 100.3% |
| Class TGM | -0.26% | -0.16% | -0.32% | 100.9% | 89.9% | 99.7% | 100.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.14% | -0.02% | -0.10% | 100.3% | 99.8% | 100.2% | 100.0% |
| Class A2 | -0.01% | -0.15% | -0.14% | 101.7% | 100.3% | 99.5% | 100.2% |
| Class B | -0.19% | -0.15% | -0.43% | 99.8% | 100.5% | 100.1% | 100.0% |
| Class C | -0.13% | -0.43% | -0.13% | 97.7% | 100.0% | 99.6% | 99.8% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.13% | -0.20% | -0.23% | 99.7% | 100.2% | 99.9% | 100.0% |
| Class D | -0.21% | -0.32% | -0.29% | 95.6% | 96.3% | 100.3% | 100.1% |
| Class F | -0.11% | -0.41% | -0.37% | 100.9% | 102.0% | 100.0% | 100.3% |
| Class TGM | -0.60% | -0.64% | -0.51% | 101.9% | 107.2% | 99.6% | 100.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.11% | -0.17% | -0.10% | 101.4% | 101.1% | 99.5% | 101.0% |
| Class C | -0.11% | -0.45% | -0.12% | 98.4% | 101.2% | 100.0% | 100.3% |
| Class E | -0.08% | 1.18% | 0.41% | 97.0% | 98.6% | 100.4% | 99.8% |
| Overall | -0.10% | 0.08% | 0.02% | 99.3% | 100.5% | 99.9% | 100.5% |
| Class D | -0.33% | 0.32% | -1.06% | 94.6% | 99.1% | 100.0% | 100.0% |
| Class F | -0.02% | -0.16% | -0.72% | 99.1% | 99.6% | 101.5% | 100.6% |
| Class TGM | -1.22% | -1.32% | -1.14% | 102.4% | 100.5% | 99.9% | 100.4% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.07% | -0.40% | 0.06% | 102.5% | 100.3% | 100.0% | 100.0% |
| Class C | -0.08% | -0.77% | -0.47% | 99.5% | 100.4% | 100.1% | 100.2% |
| Class E | -0.11% | 0.13% | 0.52% | 100.4% | 103.9% | 99.0% | 100.2% |
| Overall | -0.08% | -0.39% | 0.00% | 101.0% | 101.2% | 99.8% | 100.1% |
| Class D | -0.37% | -1.88% | -0.21% | 98.3% | 100.8% | 99.6% | 100.1% |
| Class F | -0.27% | -0.32% | 0.20% | 100.4% | 98.3% | 101.0% | 100.1% |
| Class TGM | -0.99% | -1.09% | -1.15% | 102.7% | 102.0% | 100.3% | 100.1% |

## Additional results - test 6.3 – using BVI sequences

Compared to previous version, EP contexts have been trained using BVI training set as described in EE2-6.2.

Note: encoding/decoding have been done on non homogeneous cluster, timings are removed from the results.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.06% | -0.03% |  |  | 99.7% | 101.1% |
| Class A2 | -0.03% | 0.03% | -0.01% |  |  | 101.4% | 101.2% |
| Class B | -0.01% | -0.09% | -0.01% |  |  | 100.3% | 100.6% |
| Class C | -0.01% | -0.14% | 0.00% |  |  | 100.0% | 100.2% |
| Class E | -0.04% | -0.24% | 0.17% |  |  | 99.9% | 100.4% |
| Overall | -0.02% | -0.08% | 0.02% |  |  | 100.2% | 100.7% |
| Class D | -0.07% | 0.01% | -0.34% |  |  | 100.0% | 100.1% |
| Class F | -0.01% | 0.00% | 0.02% |  |  | 101.2% | 100.6% |
| Class TGM | -0.11% | -0.04% | -0.14% |  |  | 97.4% | 101.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09% | -0.20% | 0.04% |  |  | 100.0% | 99.7% |
| Class A2 | -0.01% | -0.05% | -0.07% |  |  | 100.0% | 99.9% |
| Class B | -0.20% | -0.29% | -0.18% |  |  | 100.0% | 100.0% |
| Class C | -0.06% | -0.30% | -0.07% |  |  | 100.0% | 99.8% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.10% | -0.23% | -0.08% |  |  | 100.0% | 99.9% |
| Class D | -0.20% | -0.03% | 0.07% |  |  | 100.1% | 100.1% |
| Class F | -0.06% | -0.22% | -0.27% |  |  | 99.9% | 100.2% |
| Class TGM | -0.21% | -0.19% | -0.11% |  |  | 99.9% | 99.8% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.12% | -0.34% | -0.23% |  |  | 99.8% | 100.2% |
| Class C | -0.07% | -0.15% | 0.11% |  |  | 100.0% | 100.3% |
| Class E | 0.02% | 0.58% | -0.32% |  |  | 99.6% | 99.6% |
| Overall | -0.07% | -0.05% | -0.14% |  |  | 99.8% | 100.1% |
| Class D | -0.26% | 0.43% | -0.15% |  |  | 100.2% | 100.2% |
| Class F | 0.08% | -0.18% | -1.04% |  |  | 100.4% | 101.0% |
| Class TGM | -0.57% | -0.60% | -0.46% |  |  | 100.0% | 98.8% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.08% | -0.74% | -0.51% |  |  | 100.0% | 100.3% |
| Class C | 0.00% | -0.96% | -0.69% |  |  | 100.1% | 100.1% |
| Class E | -0.19% | -0.58% | 0.25% |  |  | 100.0% | 99.9% |
| Overall | -0.08% | -0.77% | -0.38% |  |  | 100.0% | 100.1% |
| Class D | -0.32% | -1.48% | 0.13% |  |  | 100.0% | 100.0% |
| Class F | -0.17% | -0.09% | 0.27% |  |  | 101.4% | 100.0% |
| Class TGM | -0.40% | -0.58% | -0.45% |  |  | 99.4% | 99.9% |

# Conclusion

This contribution proposes to add more flexibility to the current bin coding in ECM. It adds a new model to handle low delay slice and an intermediate entropy model between EP and CABAC. Training on different datasets shows little difference in term of results and it is proposed to use the CTC training process in order to ease the training process for the group. It is proposed to adopt these extensions to the entropy coding in the next version of ECM.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).